Cessna 206H Stationair — 2000_Cessna_206H_Checklist_v1

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "ENGINE FAILURE – IMMEDIATELY AFTER TAKEOFF" item count differs between passes (10 vs 11); verify nothing was dropped.

content_gap: Section "FORCED LANDING W/O ENGINE POWER" item count differs between passes (14 vs 13); verify nothing was dropped.

BEFORE ENGINE START

[ ]  Pilot's Operating Handbook

[ ]  Airplane Weight and Balance .... CHECKED

[ ]  Brakes .... TEST AND SET

[ ]  Control Wheel Lock

[ ]  Hobbs Meter

[ ]  Ignition & Avionics Master .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Quantity Indicator .... CHK QTY & ENSURE LOW FUEL ANNUNCIATORS OFF

[ ]  Avionics Master Switch .... ON

[ ]  Avionics Cooling Fan .... CHECK AUDIBLY

[ ]  Avionics Master Switch .... OFF

[ ]  Flaps

[ ]  Static Pressure Alternate Source .... OFF

[ ]  Annunciator Panel .... TEST/ALL LIGHTS ON

[ ]  Fuel Selector Valve .... BOTH

[ ]  Pitot Heat .... ON FOR 30 SECONDS

[ ]  Pitot Heat .... TEST FOR HEAT & POWER OFF

[ ]  Master Switch .... OFF

[ ]  Trim Controls

[ ]  Cockpit Inspect Done

[ ]  Passenger briefing

[ ]  Cabin doors .... CLOSED AND LOCKED

[ ]  Seats/seat belts .... ADJUSTED AND LOCKED

[ ]  Circuit breakers

[ ]  Avionics circuit breakers

[ ]  Electrical equipment .... OFF

[ ]  Avionics master switch .... OFF

[ ]  Cowl flaps

[ ]  Fuel selector valve .... BOTH

ENGINE START

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Prop

[ ]  Master switch .... ON

[ ]  Beacon

[ ]  Mixture

[ ]  Prop area

[ ]  Auxiliary fuel pump switch .... ON

[ ]  Mixture .... ADVANCE UNTIL THERE IS A RISE IN FUEL FLOW AND MAINTAIN IN THIS POSITION APPROXIMATELY FIVE SECONDS WITH AUX FUEL PUMP ON

[ ]  Auxiliary fuel pump switch .... OFF

[ ]  Ignition switch

[ ]  Mixture .... SMOOTHLY TO RICH

NOTE: IF ENGINE FLOODS, VERIFY AUXILIARY FUEL PUMP OFF, OPEN THROTTLE ½ TO FULL, AND CRANK ENGINE. WHEN ENGINE FIRES, ADVANCE MIXTURE TO FULL RICH & RETARD THROTTLE

[ ]  Oil Pressure

[ ]  Mixture .... LEANED at 1200 RPM

[ ]  Flaps .... RETRACT TO NORMAL

[ ]  Avionics Master / Radios .... ON

[ ]  EDM 800 .... ENTER/VERIFY FUEL VOLUME

[ ]  KLN94 & MX20 Databases .... CURRENT

[ ]  KLN94 .... FLIGHT PLAN ENTERED

[ ]  ATIS/AWOS

[ ]  Clearance if applicable

[ ]  Navigation lights .... AS REQUIRED

[ ]  Taxi light .... AS REQUIRED

[ ]  Brakes

BEFORE TAKEOFF

[ ]  Brakes

[ ]  Seat backs

[ ]  Passenger Seats .... AS FAR FWD AS POSSIBLE

[ ]  Seats/seat belts .... CHECK SECURED

[ ]  Cabin doors .... CLOSED AND LOCKED

[ ]  Flight controls .... FREE AND CORRECT

[ ]  Flight instruments .... CHECK AND SET

[ ]  Directional Gyro

[ ]  Altimeter

[ ]  Transponder

[ ]  GPS & Autopilot

[ ]  Attitude Indicator

AUTOPILOT PREFLIGHT

[ ]  Power

MANUAL ELECTRIC TRIM TEST

[ ]  Both Halves Down .... VERIFY

[ ]  Both Halves Up .... VERIFY

[ ]  MET Up, Hold DISC .... VERIFY

[ ]  Wheel/Ind Not Moving .... VERIFY

[ ]  Release AP/DISC .... VERIFY

[ ]  Wheel/Ind Moving .... VERIFY

[ ]  Baro Setting .... VERIFY

[ ]  Autopilot .... PRESS AP BUTTON

[ ]  Flight Controls .... VERIFY OVRPWER

[ ]  A/P Disc .... PRESS & VERIFY

[ ]  Trim

[ ]  Fuel quantity

[ ]  Auxiliary fuel pump .... OFF

[ ]  Mixture

[ ]  Cowl flaps

[ ]  Fuel selector valve .... RECHECK BOTH

[ ]  Throttle .... 1800 RPM

[ ]  Magnetos

[ ]  Prop .... CYCLE THEN RETURN TO HIGH

[ ]  Vacuum gauge

[ ]  Oil pressure

[ ]  Oil temperature

[ ]  Ammeter

[ ]  Annunciator panel .... TEST AND CHECK

[ ]  Throttle .... IDLE/1000 RPM OR LESS

[ ]  Friction lock

[ ]  Wing flaps .... 0 to 20°

[ ]  Cowl Flaps

[ ]  Electric trim

[ ]  Elevator/rudder trip

[ ]  Radios/avionics

[ ]  NAV/GPS switch

[ ]  Autopilot

[ ]  Strobe lights .... AS DESIRED

[ ]  Taxi/Landing lights .... AS DESIRED

[ ]  Transponder

[ ]  Brakes

NORMAL TAKEOFF

[ ]  Before Taking Runway .... XPONDER SET TO ALTITUDE

[ ]  Tower & Departure Freqs .... SET

[ ]  Wing flaps .... 0 to 20°

[ ]  Take Runway

[ ]  Power .... FULL POWER/2700 RPM

[ ]  Mixture .... MAX OR LEANED TO MAX POWER ABOVE 3000 FEET

[ ]  Elevator control .... ROTATE 56 KIAS

[ ]  Climb speed

[ ]  Wing flaps .... RETRACT WHEN CLEAR OF OBSTACLES

SHORT FIELD TAKEOFF

[ ]  Wing flaps

[ ]  Brakes

[ ]  Power .... FULL POWER/2700 RPM

[ ]  Mixture .... MAX OR LEANED TO MAX POWER

[ ]  Brakes

[ ]  Elevator control .... MAINTAIN SLIGHT TAIL LOW ATTITUDE

[ ]  Climb Speed

[ ]  Wing flaps .... RETRACT SLOWLY AFTER REACHING 80 KIAS

ENROUTE CLIMB

[ ]  Airspeed

[ ]  Power .... 25 INCH OR FULL(LESS)/2500 RPM

[ ]  Mixture .... LEAN TO 20 GPH

[ ]  Fuel Selector Valve

[ ]  Wing flaps .... VERIFY RETRACTED

[ ]  Cowl flaps

MAXIMUM PERFORMANCE CLIMB

[ ]  Airspeed

[ ]  Power .... FULL THROTTLE/2700 RPM

[ ]  Mixture .... LEAN FOR MAX POWER SEE MINIMUM FUEL FLOW PLACARD

[ ]  Fuel selector valve

[ ]  Cowl flaps

CRUISE

[ ]  Landing light .... VERIFY OFF

POWER – 75% MAX

[ ]  Throttle .... 15 to 25 INCHES

[ ]  Prop .... 2100 to 2500 RPM

[ ]  Mixture .... LEAN FOR CRUISE

[ ]  Cowl flaps

DESCENT

[ ]  Power .... AS DESIRED

[ ]  Mixture .... ENRICHEN AS REQUIRED

[ ]  Cowl flaps

[ ]  Altimeter

[ ]  Fuel selector valve

[ ]  Flaps .... AS DESIRED

[ ]  0° to 10° below 140 KIAS

[ ]  10° to 40° below 100 KIAS

INSTRUMENT APPROACH

[ ]  Approach brief

[ ]  KLN94 CDI switch

[ ]  Frequency/Approach

[ ]  IDENT

[ ]  Marker beacon

[ ]  HSI to Compass

AT FAF

[ ]  Time

[ ]  Turn

[ ]  Twist

[ ]  Throttle

[ ]  Talk

[ ]  Mixture

[ ]  Prop

[ ]  Landing light

[ ]  Flaps .... AS DESIRED

[ ]  0° to 10° below 140 KIAS

[ ]  10° to 40° below 100 KIAS

BEFORE LANDING

[ ]  Passenger Seats As Fwd As Practical .... VERIFY

[ ]  Seat backs

[ ]  Seatbelts

[ ]  Fuel Selector Valve

[ ]  Mixture

[ ]  Prop

[ ]  Landing light

[ ]  Autopilot

NORMAL LANDING

[ ]  Airspeed (flaps up)

[ ]  Flaps .... AS DESIRED

[ ]  0° to 10° below 140 KIAS

[ ]  10° to 40° below 100 KIAS

[ ]  Airspeed (flaps full)

[ ]  Braking

SHORT FIELD LANDING

[ ]  Airspeed (flaps up)

[ ]  Flaps .... FULL (40°) BELOW 100 KIAS

[ ]  Airspeed

[ ]  Power .... REDUCE TO IDLE AS OBSTACLES ARE CLEARED

[ ]  Braking

[ ]  Flaps

BALKED LANDING

[ ]  Power .... FULL POWER/2700 RPM

[ ]  Mixture

[ ]  Wing flaps

[ ]  Climb speed

[ ]  Wing flaps

[ ]  Cowl flaps

AFTER LANDING

[ ]  Wing flaps

[ ]  Cowl flaps

[ ]  Landing light

[ ]  Taxi light .... AS REQUIRED

[ ]  Strobes

[ ]  Transponder

SHUT DOWN

[ ]  Throttle

[ ]  Electrical equipment

[ ]  Avionics master

[ ]  Mixture

[ ]  Ignition switch

[ ]  Master

[ ]  Hobbs time

[ ]  Control lock

[ ]  Cowl flaps

[ ]  Fuel Selector Valve

[ ]  Nosewheel Chock

[ ]  Fuel Top Off

[ ]  Cowl Inlet Cover In Place

ENGINE FAILURE – TAKEOFF ROLL  [EMERGENCY]

[ ]  Throttle

[ ]  Brakes

[ ]  Wing flaps

[ ]  Mixture

[ ]  Ignition switch

[ ]  Master switch

ENGINE FAILURE – IMMEDIATELY AFTER TAKEOFF  [EMERGENCY]

AIRSPEED

[ ]  Flaps up

[ ]  Flaps down

[ ]  Mixture

[ ]  Fuel selector valve

[ ]  Ignition switch

[ ]  Wing flaps .... AS REQUIRED (40° RECOMMENDED)

[ ]  Master switch

[ ]  Cabin door

[ ]  Land

ENGINE FAILURE DURING FLIGHT/RESTART  [EMERGENCY]

[ ]  Airspeed

[ ]  Fuel selector valve

[ ]  Auxiliary fuel pump

[ ]  Engine power

NOTE: IF RESTART DOES NOT OCCUR

[ ]  Mixture

[ ]  Ignition switch .... CHECK BOTH (START IF PROPELLER IS STOPPED)

[ ]  Auxiliary fuel pump .... OFF (IF FUEL FLOW DROPS TO ZERO RETURN TO ON)

FORCED LANDING W/O ENGINE POWER  [EMERGENCY]

[ ]  Passenger seats

[ ]  Seatbacks

[ ]  Seats/Seat Belts

AIRSPEED

[ ]  Flaps up

[ ]  Flaps down

[ ]  Transmit 121.5

[ ]  Squawk

[ ]  Mixture

[ ]  Fuel selector valve

[ ]  Wing flaps .... AS REQUIRED (40° RECOMMENDED)

[ ]  Master switch

[ ]  ELT

[ ]  Door

PRECAUTIONARY LANDING W/ENGINE POWER  [EMERGENCY]

[ ]  Passenger seats

[ ]  Seatbacks

[ ]  Seats/Seat Belts

[ ]  Transmit 121.5

[ ]  Squawk

[ ]  Airspeed

[ ]  Wing flaps

[ ]  Avionics Master

[ ]  Electrical switches

[ ]  Wing flaps

[ ]  Airspeed

[ ]  Master switch

[ ]  ELT

[ ]  Doors

[ ]  Ignition switch

[ ]  Mixture

[ ]  Fuel selector valve

DITCHING  [EMERGENCY]

[ ]  Transmit 121.5

[ ]  Squawk

[ ]  Heavy objects

[ ]  Passenger seats

[ ]  Seatbacks

[ ]  Seats/Seat Belts

[ ]  Wing flaps

[ ]  POWER .... ESTABLISH 300 FT/MINUTE DESCENT AT 65 KIAS

[ ]  If no power

[ ]  Flaps up

[ ]  Flaps 10°

APPROACH

[ ]  High winds/Heavy seas .... INTO THE WIND

[ ]  Light winds/Heavy seas .... PARALLEL TO SWELLS

[ ]  Cabin doors

[ ]  Plan touchdown

[ ]  Face

[ ]  ELT

[ ]  Airplane

FIRE DURING GROUND START  [EMERGENCY]

[ ]  Ignition switch .... START (CONTINUE CRANKING)

NOTE: ENGINE STARTS

[ ]  Power

[ ]  Engine

NOTE: ENGINE FAILS TO START

[ ]  Ignition switch .... START (CONTINUE CRANKING)

[ ]  Throttle

[ ]  Mixture

[ ]  Fuel selector valve

[ ]  Auxiliary fuel pump

[ ]  Master switch

[ ]  Ignition switch

[ ]  Airplane

[ ]  Fire

ENGINE FIRE – IN FLIGHT  [EMERGENCY]

[ ]  Mixture

[ ]  Fuel selector valve

[ ]  Auxiliary fuel pump

[ ]  Master switch

[ ]  Cabin air

[ ]  Cabin heat

[ ]  Overhead vents

[ ]  Airspeed .... 105 KIAS (INCREASE WITHIN AIRSPEED LIMITATIONS UNTIL FIRE EXTINGUISHED)

[ ]  Forced landing

ELECTRICAL FIRE IN FLIGHT  [EMERGENCY]

[ ]  Master switch

[ ]  Vents/cabin air/heat

[ ]  Fire extinguisher

[ ]  Avionics master switch

[ ]  All other switches (except ignition) .... OFF

[ ]  Vents/cabin air .... OPEN WHEN FIRE EXTINGUISHED

[ ]  Master switch

[ ]  Circuit breakers .... CHECK FOR FAULTY CIRCUIT/DO NOT RESET

[ ]  Radio switches

[ ]  Avionics master

[ ]  Radios/electrical switches

CABIN FIRE  [EMERGENCY]

[ ]  Master switch

[ ]  Vents/cabin air/heat

[ ]  Fire extinguisher

[ ]  Vents/cabin air .... OPEN WHEN FIRE EXTINGUISHED

[ ]  Land

WING FIRE  [EMERGENCY]

[ ]  Landing light

[ ]  Taxi light

[ ]  Navigation lights

[ ]  Strobe light

[ ]  Pitot heat

[ ]  Sideslip

[ ]  Land

[ ]  Flaps

INADVERTENT ICING ENCOUNTER  [EMERGENCY]

[ ]  Pitot heat

[ ]  Turn back or change altitude .... PULL FULL OUT

[ ]  Defrost controls

[ ]  Increase RPM .... IF EXCESSIVE VIBRATION, REDUCE TO 2200 RPM AND THEN RAPIDLY INCREASE TO FULL. CYCLE AS NEEDED.

[ ]  Watch for induction air filter icing .... 15 INCHES HG IN MP CAN OCCUR ABOVE 8000 FEET

[ ]  Plan landing

[ ]  Plan for

[ ]  Open left window

[ ]  Flaps .... USE 10-20° FLAPS FOR ICE ACCUMULATIONS OF 1 INCH OR LESS. FOR GREATER ICE ACCUMULATION, LAND WITH FLAPS RETRACTED TO ENSURE ADEQUATE ELEVATOR EFFECTIVENESS.

APPROACH

[ ]  20° flaps

[ ]  0-10° flaps

[ ]  Avoid missed approach .... IF MISSED APPROACH IS MANDATORY, APPLY MAXIMAL POWER AND MAINTAIN 100 KIAS WHILE RETRACTING FLAPS IN 10° INCREMENTS.

EXCESSIVE FUEL VAPOR  [ABNORMAL]

[ ]  Auxiliary fuel pump

[ ]  Mixture

[ ]  Fuel selector valve .... SELECT OPPOSITE TANK IF SYMPTOMS CONTINUE

[ ]  Auxiliary fuel pump switch .... OFF AFTER FUEL FLOW HAS STABILIZED

AMMETER – EXCESSIVE RATE OF CHARGE  [ABNORMAL]

[ ]  Alternator

[ ]  Nonessential electrical equipment

[ ]  With alternator side of master OFF

[ ]  Nonessential electrical equipment

[ ]  Terminate flight

LOW VOLTAGE ANNUNCIATOR – AMMETER INDICATES DISCHARGE  [ABNORMAL]

[ ]  Avionics master

[ ]  Alternator circuit breaker (ALT FLD)

[ ]  Master switch

[ ]  Master switch

[ ]  Low voltage annunciator

NOTE: IF LOW VOLTAGE ILLUMINATES

[ ]  Alternator

[ ]  Nonessential electrical equipment

[ ]  Terminate flight

EMERGENCY DESCENT  [EMERGENCY]

SMOOTH AIR

[ ]  Seats and seat belts

[ ]  Throttle

[ ]  Propeller

[ ]  Mixture

[ ]  Wing flaps

[ ]  Airspeed

ROUGH AIR

[ ]  Seats and seat belts

[ ]  Throttle

[ ]  Propeller

[ ]  Mixture

[ ]  Wing flaps

WEIGHTS AND AIRSPEED

[ ]  3600

[ ]  2950

[ ]  2300

AUTOPILOT FAILURE  [EMERGENCY]

[ ]  Control wheel .... GRASP FIRMLY AND MAINTAIN CONTROL OF AIRCRAFT

[ ]  A/P DISC/TRIM INT switch .... PRESS AND HOLD

[ ]  Aircraft

[ ]  Autopilot breaker

STATIC SOURCE BLOCKAGE  [EMERGENCY]

[ ]  Static pressure alternate source valve .... PULL ON

[ ]  Heat and air vents .... PULL ON

[ ]  Vents

[ ]  Airspeed and altitude .... REFER TO CALIBRATION CHARTS

SPEEDS

[ ]  Vr — Rotation speed .... 55 KIAS

[ ]  Vx — Best angle climb .... 70 (SL) 75 (10K)

[ ]  Vy — Best rate climb .... 86 (SL) 82 (10K)

[ ]  Vso — Stall with flaps .... 38 (rcog) 44 (fcog)

[ ]  Vs — Stall without flaps .... 50 (rcog) 54 (fcog)

VA — MANEUVERING SPEED

[ ]  3600 pounds

[ ]  2900 pounds

[ ]  2200 pounds

[ ]  Vfe — 0 to 10° .... 140 KIAS

[ ]  Vfe — 20 to 40° .... 100 KIAS

[ ]  Vne — Never exceed speed .... 182 KIAS

[ ]  Vno — Maximum structural cruising speed .... 149 KIAS

[ ]  Maximum window open

[ ]  Normal climb out .... 75-85 KIAS

[ ]  Shortfield/Flaps 20° climb .... 74 KIAS

[ ]  Enroute climb

[ ]  Normal approach flaps up .... 80-90 KIAS

[ ]  Normal approach flaps 40° .... 70-80 KIAS

[ ]  Short field approach flaps 40° .... 67 KIAS

[ ]  Balked landing flaps 20° .... 80 KIAS

[ ]  Maximum demonstrated crosswind .... 20 KNOTS

[ ]  Engine failure after takeoff flaps up .... 80 KIAS

[ ]  Engine failure after takeoff flaps down .... 70 KIAS

MAXIMUM GLIDE

[ ]  3600 pounds

[ ]  3200 pounds

[ ]  2800 pounds

[ ]  Precautionary landing w/engine power .... 70 KIAS

[ ]  Landing without engine power/flaps up .... 80 KIAS

[ ]  Landing without engine power/flaps down .... 70 KIAS

EMERGENCY DESCENT

[ ]  Smooth air

[ ]  Rough air — 3600 pounds

[ ]  Rough air — 2900 pounds

[ ]  Rough air — 2200 pounds

PERFORMANCE TABLE

STAGE — MP — RPM — PITCH — KIAS — VSI

[ ]  Climb

[ ]  Cruise

[ ]  Cruise Descent

[ ]  Appr Climb

[ ]  Approach

[ ]  Appr Descnt

[ ]  Downwind -10 Degr Flps

[ ]  Downwind -20 Degr Flps

[ ]  Base

[ ]  Final

EDM 800 REFERENCE

[ ]  Voltage

[ ]  Outside Air Difference between hottest and coldest EGT

[ ]  RPM/MAP

[ ]  Fuel Remaining

[ ]  Fuel required to next GPS WPT or destination

[ ]  Fuel Reserve at next GPS WPT or destination

[ ]  Nautical Miles per Gallon

[ ]  Time to Empty

[ ]  Fuel flow rate

[ ]  Total Fuel Used

[ ]  EGT, CHT

[ ]  Oil Temperature

[ ]  Shock cooling

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/206H Stationair/2000_Cessna_206H_Checklist_v1.txt

Rights: licensed

Checklist hash: 71d5b038a1a201406df0c3e84efb4b7d983c1f996413da73518a838261b7f4c6

Verify against the aircraft's own approved documentation before use.