Cessna 210_210B Centurion — jochman-clc210

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Landing" item count differs between passes (11 vs 10); verify nothing was dropped.

Aircraft Performance Speeds

[ ]  VS0 – Landing configuration stall at gross weight, power off .... 65 MPH

[ ]  VS1 – Cruise configuration stall at gross weight, power off .... 75 MPH

[ ]  VR .... 60-70 MPH

[ ]  VX – Best Angle of Climb Speed .... 82 MPH

[ ]  VY – Best Rate of Climb Speed .... 109 MPH

[ ]  VFE – Max. Flap Extension 0-10º .... 160 MPH

[ ]  VFE – Max. Flap Extension 10-30º .... 110 MPH

[ ]  VA – Maneuvering Speed .... 135 MPH

[ ]  VNO – Top of Green Arc .... 190 MPH

[ ]  VNE – Red Line – Never Exceed .... 225 MPH

[ ]  VL/D .... 85-95 MPH

Aircraft Specifications

[ ]  Maximum Gross Weight .... 3800 LBS

[ ]  Basic Empty Weight (N4667Q) .... 2206 LBS

[ ]  Useful Load (N4667Q) .... 1594 LBS

[ ]  Basic Empty Weight (N8280M) .... 2139 LBS

[ ]  Useful Load (N8280M) .... 1661 LBS

[ ]  Fuel Capacity (useable) .... 89 GALS

[ ]  Fuel Consumption 75%@2500’ .... 15.5 GPH

Before Starting Engine

[ ]  Preflight .... Complete

[ ]  Seats and Seat Belts .... Adjust and Lock

[ ]  Brakes .... Test and Set

[ ]  Cowl Flaps .... Open

[ ]  Fuel Selector .... Fullest Tank

[ ]  Lights .... OFF

[ ]  Circuit Breakers .... Check

[ ]  Clock .... Set

[ ]  Altimeter .... Set

[ ]  Controls .... Free & Correct

[ ]  Flaps .... Check

[ ]  Trim .... Set for T/O

[ ]  Radios and Electrical Equipment .... OFF

[ ]  Master Switch .... ON

[ ]  Landing Gear .... Handle Neutral

[ ]  Landing Gear .... Green DN Light ON

[ ]  Landing Gear Lights, Horn .... Press to Test

Starting Engine

[ ]  Anti-collision Light .... ON

[ ]  Mixture .... Full Rich

[ ]  Propeller .... High RPM

[ ]  Throttle .... Full Open

[ ]  Propeller Area .... CLEAR

[ ]  Aux. Fuel Pump Switch .... High for 5 Sec.

[ ]  Aux. Fuel Pump Switch .... OFF

[ ]  Throttle .... 50%

[ ]  Ignition Key .... Start

[ ]  Throttle .... Reduce to Idle

[ ]  Oil Pressure .... Check

[ ]  Radios .... ON, Check 121.5

[ ]  Transponder .... Standby

[ ]  Fuel Selector .... Switch Tanks for Taxi

[ ]  Brakes .... OFF

[ ]  Toe Brakes .... Check

Before Take-Off

[ ]  Parking Brake .... SET

[ ]  Cowl Flaps .... Check FULL

[ ]  Flight Controls .... Free and Correct

[ ]  Elevator and Rudder Trim .... TAKE OFF

[ ]  Fuel Selector .... Fullest Tank

[ ]  Throttle Setting .... 1700 RPM

[ ]  Engine Instruments .... Check

[ ]  Ammeter .... Check

[ ]  Suction Gauge .... Check (4.6 to 5.4")

[ ]  Oil Pressure .... Check

[ ]  Oil Temperature .... Check

[ ]  Alternator .... Check

[ ]  Annunciator Panel .... Check

[ ]  Fuel Flow .... Check

[ ]  Magnetos .... Check (50 RPM Max diff.)

[ ]  Propeller .... Cycle

[ ]  Throttle .... Check Idle

[ ]  Flight Instruments and Radios .... SET

[ ]  Autopilot or Wing Leveler .... OFF

[ ]  Cabin Doors, Window .... Closed and Locked

[ ]  Seat Belts/Harness .... Buckled

[ ]  Transponder .... ON ALT, Code Set

Normal Take-Off

[ ]  Wing Flaps .... 0º - 10º

[ ]  Power .... Full Throttle and 2850 RPM

[ ]  Mixture .... Lean for Field Elevation

[ ]  Elevator Control .... Lift nose at 60-70 MPH

[ ]  Climb Speed .... 100-110 MPH

[ ]  Brakes .... Apply (when airborne)

[ ]  Landing Gear .... Retract (in climb out)

[ ]  Wing Flaps .... Retract (obstacles cleared)

Maximum Performance Take-Off

[ ]  Wing Flaps .... 10º

[ ]  Brakes .... APPLY

[ ]  Power .... Full Throttle and 2850 RPM

[ ]  Mixture .... Lean for Field Elevation

[ ]  Brakes .... RELEASE

[ ]  Elevator Control .... Slightly Tail Low

[ ]  Climb Speed .... 82 MPH

[ ]  Landing Gear and Wing Flaps .... RETRACT (after obstacles are cleared and 90)

NOTE: Do not reduce power until wing flaps and landing gear have been retracted.

Normal Climb

[ ]  Airspeed .... 120 to 140 MPH

[ ]  Power .... 25" and 2550 RPM

[ ]  Mixture .... Lean to 18 GPH

[ ]  Cowl Flaps .... Open as Required

Maximum Performance Climb

[ ]  Airspeed .... 109 MPH (sea level) to 102 MPH (10,000')

[ ]  Power .... Full Throttle and 2700 RPM

[ ]  Mixture .... Lean for Altitude

[ ]  Cowl Flaps .... Full Open

Cruise

[ ]  Power .... 15-25" of MP, 2200-2550 RPM

[ ]  Power .... no more than 75%

[ ]  Cowl Flaps .... Open as Required

[ ]  Elevator and Rudder Trim .... Adjust

[ ]  Mixture .... Lean for Cruise

Let-Down

[ ]  Power .... As Desired

[ ]  Mixture .... Lean for Smoothness

[ ]  Mixture .... Use Full Mixture for Idle Power

[ ]  Cowl Flaps .... Closed

Before Landing

[ ]  Fuel Selector .... Fullest Tank

[ ]  Landing Gear Level .... DOWN (160 MPH)

[ ]  Landing Gear Light .... GREEN

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Wing Flaps .... DOWN

[ ]  Wing Flaps .... 0º-10º (below 160 MPH)

[ ]  Wing Flaps .... 10º-30º (below 110 MPH)

[ ]  Airspeed .... 95-105 MPH (flaps retracted)

[ ]  Airspeed .... 85-95 MPH (flaps extended)

[ ]  Elevator Trim .... Adjust

Balked Landing (Go Around)

[ ]  Power .... Full Throttle and 2850 RPM

[ ]  Wing Flaps .... Retract to 20º

[ ]  Wing Flaps .... Upon reaching an airspeed of 90 MPH, retract flaps slowly.

Normal Landing

[ ]  Touchdown .... Main Wheels First

[ ]  Landing Roll .... Lower Nose Wheel Gently

[ ]  Braking .... Minimum Required

After Landing

[ ]  Cowl Flaps .... OPEN

[ ]  Wing Flaps .... RETRACT

[ ]  Transponder .... OFF

Securing Aircraft, Stopping Engines

[ ]  Parking Brake .... SET

[ ]  Radios and Electrical Equipment .... OFF

[ ]  Mixture .... Idle Cut-Off

[ ]  Ignition and Master Switch .... OFF

[ ]  Control Lock .... Installed

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/210_210B Centurion/jochman-clc210.txt

Rights: licensed

Checklist hash: 5b66375463172ea76f5af0de1fbe79d4d946122b1fad9771ce5f7f4155a76891

Verify against the aircraft's own approved documentation before use.