Cessna 310 — C_R_Cessna_310L_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE STARTING ENGINE

[ ]  Seat belts

[ ]  Brakes

[ ]  Landing Gear Switch .... Check DOWN

[ ]  Auxiliary Fuel Pumps .... OFF

[ ]  Emergency Power Switch .... OFF

[ ]  Battery Switch .... ON

[ ]  Alternator Switches .... ON

[ ]  Voltage Regulator Switch .... Main

[ ]  Landing Gear Lights

[ ]  Trim Controls

[ ]  Flight Controls

[ ]  Alternate Air Control .... Check in

[ ]  Altimeter and Clock .... Set

[ ]  Avionics Master .... OFF

[ ]  Throttle .... Open 1 inch

[ ]  Propeller .... High RPM

[ ]  Mixture .... Full Rich

[ ]  Fuel Selectors .... LR MAIN

[ ]  Rotating Beacon .... ON

STARTING ENGINE

[ ]  Magneto Switches .... ON

[ ]  Start Engine

[ ]  Auxiliary Fuel Pumps .... LOW

[ ]  Avionics Master .... ON

RUN UP

[ ]  Parking Brake .... SET

[ ]  Cabin Door .... CLOSED and LOCKED

[ ]  Throttle setting .... 1700 RPM

[ ]  Alternators – Check

[ ]  Magnetos -- Check

[ ]  Propellers .... Check to 1200 RPM

[ ]  Vacuum Pressure .... Check (4.75 to 5.25”)

[ ]  Oil Temperature .... Check green arc

[ ]  Throttle setting .... 700 - 1000 RPM

[ ]  Trim Controls .... Checked and set

[ ]  Wing Flaps .... 0°

[ ]  Flight Controls

[ ]  Engine instruments -- Check

[ ]  Parking brake .... Released

[ ]  Lights .... as Required

[ ]  ATIS -- Check

[ ]  Transponder -- Set

TAKEOFF - Normal TAKEOFF

[ ]  Auxiliary Fuel Pumps .... ON (high)

[ ]  Power .... Full throttle and 2625 RPM

[ ]  Mixture .... FULL RICH (lean for maximum power above 3000 feet)

[ ]  Maintain Level Attitude

[ ]  Elevator Control .... Raise nose wheel at 90 MPH

[ ]  Break ground at .... 105 MPH

[ ]  Brakes .... Apply momentarily

[ ]  Landing gear .... retract

[ ]  Climb Speed .... 124 MPH

[ ]  Auxiliary fuel pumps .... OFF

TAKEOFF - MAXIMUM PERFORMANCE

[ ]  Auxiliary Fuel Pumps .... ON (high)

[ ]  Wing Flaps .... 15°

[ ]  Power .... Full throttle and 2625 RPM

[ ]  Mixture .... FULL RICH (lean for maximum power above 3000 feet)

[ ]  Maintain Level Attitude

[ ]  Elevator Control .... Raise nose wheel at 84 MPH

[ ]  Break ground at 93 MPH .... Hold speed until all obstacles are cleared

[ ]  Brakes .... Apply momentarily

[ ]  Landing gear .... retract

[ ]  Flaps .... Retract (after obstacles are cleared)

[ ]  Auxiliary Fuel Pumps .... OFF

CLIMB

NORMAL CLIMB

[ ]  Airspeed .... 130-150 MPH

[ ]  Power .... 24 INCHES Hg or FULL THROTTLE and 2450 RPM

[ ]  Mixture .... Adjust to climb fuel flow

[ ]  Auxiliary Fuel Pumps .... ON (above 12,000' altitude to minimize vapor formation)

MAXIMUM PERFORMANCE CLIMB

[ ]  Airspeed .... 124 MPH at sea level, 122 MPH at 10,000'

[ ]  Power .... FULL THROTTLE and 2625 RPM

[ ]  Mixture .... Adjust for altitude and power

[ ]  Auxiliary Fuel Pumps .... ON (above 12,000' altitude to minimize vapor formation)

CRUISING

[ ]  Cruise Power .... 15-24 inches Hg and 1200 - 2450 RPM

[ ]  Mixtures .... Lean as required

[ ]  Fuel Selectors .... aux after 60 min

[ ]  Trim Tabs .... adjust

LETDOWN

[ ]  Power .... As required

[ ]  Mixture .... ADJUST for smooth operation and gradual enrichment as altitude is lost

BEFORE LANDING

[ ]  Fuel Selectors .... MAIN

[ ]  Mixtures .... Full Rich

[ ]  Propellers .... High RPM

[ ]  Wing Flaps

[ ]  Landing Gear .... Extended below 160 MPH

[ ]  Auxiliary Fuel Pumps .... ON

[ ]  Approach .... 105 MPH

LANDING

[ ]  Touchdown .... Main wheels first

[ ]  Landing roll .... Lower nose wheel gently

[ ]  Braking .... As required

GO-AROUND (Twin Engine)

[ ]  Increase engine speed to .... 2625 RPM and apply full throttle

[ ]  Reduce flap setting to .... 15°

[ ]  Trim airplane for .... climb

[ ]  Retract flaps .... as all obstacles are cleared, and a safe altitude and airspeed are obtained.

[ ]  Do not retract landing gear .... if another landing approach is to be conducted.

AFTER LANDING

[ ]  Auxiliary Fuel Pumps .... LOW

[ ]  Wing Flaps .... RETRACT

[ ]  Transponder .... stdby

SECURING AIRPLANE

[ ]  Auxiliary Fuel Pumps .... OFF

[ ]  Throttles .... IDLE

[ ]  Propellers .... High RPM (full forward)

[ ]  Mixtures .... IDLE CUT OFF

[ ]  Magneto Switches .... OFF, after engine stops

[ ]  Avionics Switch .... OFF

[ ]  All switches .... OFF

[ ]  Brakes .... SET

[ ]  Control Lock .... Install

[ ]  Cabin Door .... Secure

ENGINE-OUT CHECKLISTS  [EMERGENCY]

ENGINE-OUT AFTER TAKEOFF - ABOVE 105 MPH (WITHOUT SUFFICIENT RUNWAY AHEAD)

[ ]  Mixture .... Full rich

[ ]  Propellers .... Full forward

[ ]  Throttles .... Full forward

[ ]  Landing gear .... UP

[ ]  Determine inoperative engine .... (dead foot dead engine)

[ ]  Inoperative engine .... FEATHER

[ ]  Establish bank .... 5° toward good engine

[ ]  Climb out to clear any obstacles .... 105 MPH

[ ]  Accelerate to .... 120 MPH after obstacle is cleared

[ ]  Wing Flaps .... UP (if extended)

[ ]  Trim tabs .... Adjust 5° back toward good engine

[ ]  Secure inoperative engine as follows:

[ ]  A. Mixture .... IDLE CUT OFF

[ ]  B. Fuel Selector .... OFF

[ ]  C. Auxiliary Fuel Pump .... OFF

[ ]  D. Magneto Switches .... OFF

[ ]  E. Alternator Switch .... OFF

[ ]  As soon as practical .... LAND

ENGINE-OUT DURING FLIGHT

[ ]  Determine inoperative

[ ]  Power .... Increase as required

[ ]  Fuel Flow - Check, if deficient, position auxiliary fuel pump switch to .... ON

[ ]  Fuel Quantity - Check

[ ]  Oil Pressure and Oil Temperature - Check, shut down inoperative engine if oil pressure is low. .... Check

[ ]  Magneto Switches - Check

[ ]  Secure inoperative engine as follows:

[ ]  A. Throttle .... CLOSED

[ ]  B. Mixture .... IDLE CUT-OFF

[ ]  C. Propeller .... FEATHER

[ ]  D. Fuel Selector .... OFF

[ ]  E. Auxiliary Fuel Pump .... OFF

[ ]  F. Magneto Switch .... OFF

[ ]  G. Alternator Switch .... OFF

OPERATIVE ENGINE - ADJUST

[ ]  A. Power .... Increase as required

[ ]  B. Mixture .... Adjust for power

[ ]  C. Fuel Selector .... Main tank

[ ]  D. Auxiliary Fuel Pump .... ON

[ ]  Trim Tabs .... ADJUST 5° bank towards good engine

[ ]  Electrical load .... DECREASE

[ ]  As soon as practical .... LAND

ENGINE RESTARTS IN FLIGHT (AFTER FEATHERING)

[ ]  Fuel Selector Valve .... Main Tank

[ ]  Throttle .... Advance

[ ]  Propeller .... HIGH RPM

[ ]  Mixture .... FULL RICH

[ ]  Magneto Switches .... ON

[ ]  Primer Switch .... ENGAGE

[ ]  Starter Button .... PRESS

[ ]  Power .... Increase slowly

NOTE: NOTE: If start is unsuccessful, turn magneto switches OFF, retard mixture to IDLE CUT-OFF, open throttle fully, and engage starter for several revolutions. Then repeat air start procedure.

SINGLE ENGINE APPROACH AND LANDING

[ ]  Approach at .... 110 MPH with excess altitude

[ ]  Delay extension of landing gear until .... within gliding distance of field

[ ]  Avoid use of flaps until .... landing is assured

[ ]  Decrease speed below 105 MPH only if .... landing is a certainty

GO-AROUND (SINGLE ENGINE)

[ ]  If absolutely necessary, and speed is above 105 MPH, increase engine speed to 2625 RPM and apply .... full throttle

[ ]  Retract landing gear .... UP

[ ]  Reduce flap setting to .... 0° (if extended)

[ ]  Climb at .... 120 MPH (108 MPH with obstacles directly ahead)

[ ]  Trim airplane for .... single-engine

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/310/C_R_Cessna_310L_checklist.txt

Rights: licensed

Checklist hash: 3b9b9744bb450559343584c47a52975e7e9834facb2ae60e86c86837cc2189e4

Verify against the aircraft's own approved documentation before use.