Cessna 401 — Louis_Janos_401_Emergency1_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "ALTERNATOR FAILURE" item count differs between passes (12 vs 10); verify nothing was dropped.

content_gap: Section "ALTERNATOR FAILURE DUAL" item count differs between passes (14 vs 12); verify nothing was dropped.

content_gap: Section "ENGINE INOPERATIVE PROCEDURES" item count differs between passes (22 vs 21); verify nothing was dropped.

ENGINE INOPERATIVE PROCEDURES  [EMERGENCY]

DURING TAKEOFF

[ ]  Throttles .... Closed

[ ]  Brakes .... As Required

AFTER TAKEOFF

[ ]  Mixtures .... Rich

[ ]  Props .... Forward

[ ]  Throttles .... Forward (34.5" MP)

[ ]  Gear .... Up

[ ]  Inoperative Propeller .... Feather

INOPERATIVE ENGINE –SECURE:

[ ]  Throttle .... Close

[ ]  Mixture .... Idle Cutoff

[ ]  Prop .... Feather

[ ]  Fuel Selector .... Off

[ ]  Aux fuel pump .... Off

[ ]  Magnetos .... Off

[ ]  Alternator .... Off

[ ]  Cowl flap .... close

DURING FLIGHT

[ ]  Inoperative Engine .... Secure

[ ]  Operative Engine .... Adjust

[ ]  Electrical Load .... Reduce

LANDING GEAR EMERGENCY  [EMERGENCY]

[ ]  Landing Gear Motor C/B .... Pull

[ ]  Landing Gear Switch .... Neutral (center)

[ ]  Pilot's Seat .... Tilt Aft

[ ]  Hand Crank .... Extend and Lock

[ ]  Rotate Crank .... Clockwise four turns past the point where gear-down  lights come on (approximately 54 turns)

[ ]  Gear .... Check Down

[ ]  Gear Warning Horn .... Check

[ ]  Hand Crank .... Push Button and Stow

OBSTRUCTION/ICING OF STATIC SOURCE  [EMERGENCY]

[ ]  Alternate Static Source .... Open

[ ]  Maintain Altitude and Airspeed

AIR INLET/FILTER ICING  [EMERGENCY]

[ ]  Alternate Air Control .... Pull Out

[ ]  Props .... Increase (2550 RPM)

[ ]  Mixture .... Lean as required

ENGINE DRIVEN INJECTOR PUMP FAILURE  [EMERGENCY]

[ ]  Fuel Selector .... Main Tank

[ ]  Auxiliary Fuel Pump .... High

[ ]  Cowl Flap .... Open

[ ]  Mixture .... Full Rich

[ ]  Land .... As Soon As Practical

ALTERNATOR FAILURE  [ABNORMAL]

indicated by illumination of failure light

[ ]  Reduce load

IF CIRCUIT BREAKER IS TRIPPED

[ ]  Shut off affected alternator.

[ ]  Reset affected alternator circuit breaker .

[ ]  Turn on affected alternator switch.

[ ]  If circuit breaker reopens, turn off alternator.

IF CIRCUIT BREAKER DOES NOT TRIP

[ ]  Select affected alternator on ammeter and monitor output.

[ ]  If output is normal and failure light remains on, disregard fail indication and have indicator checked after landing.

[ ]  If output is insufficient turn off alternator and reduce electrical load to one alternator capacity.

[ ]  If complete loss of alternator output occurs check field fuse and replace if necessary.

[ ]  If an intermittent light indication accompanied by ammeter fluctuation is observed - shut off affected alternator and reduce load to one alternator capacity.

ALTERNATOR FAILURE DUAL  [ABNORMAL]

[ ]  Reset over-voltage protector if out

[ ]  If still inoperative, switch to number 2 regulator

IF STILL INOPERATIVE:

[ ]  Shut off both alternator switches

[ ]  Shut off battery master switch

[ ]  Reduce electrical load to a minimum

[ ]  Turn on emergency power switch

[ ]  Turn on left alternator switch

[ ]  Turn on right alternator switch

IF STILL INOPERATIVE:

[ ]  Turn off emergency power switch

[ ]  Turn off both alternator switches

[ ]  Turn on battery master switch

[ ]  Prepare to terminate flight

AIRCRAFT FIRE ON GROUND  [EMERGENCY]

[ ]  Throttles .... Close

[ ]  Brakes .... As Required

[ ]  Mixtures .... Idle Cutoff

[ ]  Battery .... Off

[ ]  Evacuate Aircraft

IN FLIGHT CABIN FIRE OR SMOKE  [EMERGENCY]

[ ]  Electrical Load .... Reduce

[ ]  Isolate Source .... Attempt

[ ]  Wemacs .... Open

[ ]  Cabin Air Controls .... Open

[ ]  Land .... Evacuate Aircraft

IN FLIGHT WING OR ENGINE FIRE  [EMERGENCY]

[ ]  Auxiliary fuel pumps .... Off

[ ]  Appropriate Engine .... Secure

[ ]  Cabin Heater .... Off

[ ]  Evacuate .... As Soon As Possible

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/401/Louis_Janos_401_Emergency1_checklist.txt

Rights: licensed

Checklist hash: 4313064ff73ba193ee2c7ee8b0e61708387924e1efa7e1b4953380f6c17f7ea9

Verify against the aircraft's own approved documentation before use.