Cessna 401 — Louis_Janos_401_Normal_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "AFTER TAKEOFF" item count differs between passes (5 vs 6); verify nothing was dropped.

content_gap: Section "BEFORE TAKEOFF" item count differs between passes (9 vs 8); verify nothing was dropped.

content_gap: Section "CRUISE" item count differs between passes (5 vs 7); verify nothing was dropped.

content_gap: Section "TAKEOFF" item count differs between passes (4 vs 5); verify nothing was dropped.

BEFORE START

[ ]  Preflight Inspection .... COMPLETE

[ ]  Cabin Door .... LATCHED

[ ]  Gear Switch .... DOWN

[ ]  Alternate Air Controls .... FULL IN

[ ]  Alternators and Battery .... ON

[ ]  Control Lock .... REMOVED

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Cowl Flaps .... OPEN

[ ]  Throttles .... OPEN ONE INCH

[ ]  Props .... FORWARD

[ ]  Mixtures .... RICH

[ ]  Fuel Selectors .... MAIN/TIP TANKS

ENGINE START

[ ]  Magnetos .... ON

[ ]  Left Starter .... PRESS

[ ]  Prime Switch .... APPROPRIATE ENGINE

[ ]  Auxiliary Fuel Pump .... LOW

[ ]  Throttle .... 1000 - 1200 RPM

[ ]  Vacuum System .... CHECK

[ ]  Right Engine .... REPEAT 1 THROUGH 6

[ ]  Lights .... ON

[ ]  Radios .... ON

BEFORE TAKEOFF

[ ]  Trim Tabs .... CHECK MOVEMENT, SET FOR TAKEOFF

[ ]  Controls .... FREE AND CORRECT

[ ]  Cowl Flaps .... OPEN

[ ]  Wing Flaps .... UP

[ ]  Engine Instruments .... NORMAL

[ ]  Instruments and Radios .... CHECK/SET

[ ]  Engine Runup .... CHECK

[ ]  Voltammeter, Vacuum System, Magnetos

[ ]  Feather

TAKEOFF

[ ]  Auxiliary Fuel Pumps .... LOW

[ ]  Landing Lights .... ON

[ ]  Transponder .... ALT

[ ]  Engine Power .... 34.5" MP/2700 RPM, CHECK FUEL FLOW IN WHITE ARC

AFTER TAKEOFF

[ ]  Landing Gear .... UP

[ ]  Engine Power .... 29.5" MP/2450 RPM (1000 FT AGL CHECK FUEL FLOW ON BLUE 75% MARK)

[ ]  Auxiliary Fuel Pumps .... OFF OR AS REQUIRED

[ ]  Fuel selector

[ ]  Voltammeter

CRUISE

[ ]  Engine Power .... 27.0" MP/2300 RPM

[ ]  Mixtures .... LEAN, USE EGT BELOW 75% POWER, SET 100º ROP

[ ]  Cowl Flaps .... CLOSED

[ ]  Fuel Selectors .... AUXILIARY - ONE AT A TIME (AFTER THERE IS ROOM IN THE TIP TANK)

[ ]  Landing Lights .... OFF

DESCENT

[ ]  ATIS

[ ]  Altimeter .... SET

[ ]  Reduce M.P. 1" per minute .... CHECK

APPROACH

[ ]  Fuel Selectors .... MAINS/OUTBOARD TANKS

[ ]  Alternate Air .... CHECK IN

[ ]  Mixtures .... RICH OR AS REQUIRED

[ ]  Auxiliary Fuel Pumps .... LOW

[ ]  Heater .... OFF

[ ]  Landing Lights .... ON

[ ]  Cowl Flaps .... CLOSED

[ ]  Flaps .... DOWN 15º BELOW 180 MPH

[ ]  Gear .... DOWN

BEFORE LANDING

[ ]  Flaps .... DOWN 45º BELOW 160 MPH

[ ]  Gear .... CHECK DOWN

[ ]  Props .... FORWARD

AFTER LANDING

[ ]  Auxiliary Fuel Pumps .... LOW

[ ]  Cowl Flaps .... OPEN

[ ]  Flaps .... UP

[ ]  Transponder .... STBY

[ ]  Trim .... SET FOR TAKEOFF

[ ]  Heater .... CHECK OFF

[ ]  Landing Lights .... OFF

SHUTDOWN

[ ]  Auxiliary Fuel Pumps .... OFF

[ ]  Radios .... OFF

[ ]  Throttles .... IDLE

NOTE: Engine cool 3 min. before shutdown

[ ]  Props .... FORWARD

[ ]  Left Mixture .... IDLE CUTOFF

[ ]  Vacuum System .... CHECK

[ ]  Right Mixture .... IDLE CUTOFF

[ ]  Magnetos .... OFF

[ ]  Battery and Alternators .... SET

[ ]  Parking Brake .... SET

[ ]  Control Lock .... INSTALL

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/401/Louis_Janos_401_Normal_checklist.txt

Rights: licensed

Checklist hash: 31f92539b1eab45618e4396a8498d6f806209bc71c01970a795e25d61952ceb5

Verify against the aircraft's own approved documentation before use.