Cessna 401 — Louis_Janos_401A_Preflight_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Preflight Inspection

1.

[ ]  Control Lock .... REMOVE and STOW

[ ]  Parking Brake .... N/A

[ ]  All Switches .... OFF

[ ]  Landing Gear Switch .... DOWN

[ ]  Battery Switch .... ON

[ ]  Fuel Gages .... CHECK QUANTITY and OPERATION

[ ]  Flaps .... EXTEND

[ ]  Battery Switch .... OFF

[ ]  Left Fuel Selector .... LEFT MAIN (feel for detent)

[ ]  Right Fuel Selector .... RIGHT MAIN (feel for detent)

[ ]  Trim Tab Controls (3) .... NEUTRAL

[ ]  N/A .... N/A

2.

[ ]  Battery Compartment Cover .... SECURE

[ ]  Wing Locker Baggage Door .... SECURE

[ ]  Flap .... CHECK SECURITY and ATTACHMENT

[ ]  Wing Locker Sump .... DRAIN

[ ]  Control Surface Lock .... REMOVE

[ ]  Aileron and tab .... CHECK CONDITION and FREEDOM OF MOVEMENT

[ ]  Fuel Sump (Main Tank) .... DRAIN

[ ]  Fuel Vent and Sniffle Valve .... CLEAR

[ ]  Fuel Quantity (Main Tank) .... CHECK, CAP SECURE

[ ]  Stall Warning Vane .... CHECK FREEDOM OF MOVEMENT

[ ]  Wing Tie Down .... REMOVE

[ ]  Fuel Quantity (Auxiliary Tank) .... CHECK, CAP SECURE

[ ]  Fuel Sump (Auxiliary Tank and Wing Locker Transfer Line) .... DRAIN

[ ]  Fuel Vent (Wing Locker Tank) .... CLEAR, CAP SECURE

3.

[ ]  Fuel Strainer .... DRAIN

[ ]  Cowl Flap .... SECURE

[ ]  Engine Compartment General Condition .... CHECK

[ ]  Oil Level .... CHECK, MINIMUM 9 QUARTS

[ ]  Propeller and Spinner .... EXAMINE FOR NICKS, SECURITY and OIL LEAKS

[ ]  Cowl Flap .... SECURE

[ ]  Main Gear, Strut, Doors and Tire .... CHECK

[ ]  Leading Edge Air Intake .... CLEAR

[ ]  Cross Feed Line .... DRAIN

4.

[ ]  Baggage Door .... SECURE

[ ]  Nose Gear, Strut, Doors and Tire .... CHECK

[ ]  Pitot Cover (If Installed) .... REMOVE, Pitot Tube - CLEAR

[ ]  Tie Down .... REMOVE

[ ]  Heater Inlet .... CLEAR

[ ]  Baggage Door .... SECURE

5.

[ ]  Crossfeed Line .... DRAIN

[ ]  Leading Edge Air Intake .... CLEAR

[ ]  Main Gear, Strut, Doors and Tire .... CHECK

[ ]  Cowl Flap .... SECURE

[ ]  N/A .... N/A

[ ]  Oil Level .... CHECK, MINIMUM 9 QUARTS

[ ]  Engine Compartment and General Condition .... CHECK

[ ]  Propeller and Spinner .... EXAMINE FOR NICKS, SECURITY and OIL LEAKS

[ ]  Cowl Flap .... SECURE

[ ]  Fuel Strainer .... DRAIN

6.

[ ]  Fuel Vent (Wing Locker Tank) .... CLEAR

[ ]  Fuel Sump (Auxiliary Tank and Wing Locker Transfer Line) .... DRAIN

[ ]  Fuel Quantity (Auxiliary Tank) .... CHECK, CAP SECURE

[ ]  Wing Tie Down .... REMOVE

[ ]  Fuel Quantity (Main Tank) .... CHECK, CAP SECURE

[ ]  Fuel Vent and Sniffle Valve .... CLEAR

[ ]  Fuel Sump (Main Tank) .... DRAIN

[ ]  Aileron .... CHECK CONDITION and FREEDOM of MOVEMENT

[ ]  Control Surface Lock .... REMOVE

[ ]  Flaps .... CHECK SECURITY and ATTACHMENT

[ ]  Nacelle Baggage Door .... SECURE

7.

[ ]  Static Port .... CLEAR

[ ]  Control Surface Lock .... REMOVE

[ ]  Tie Down .... REMOVE

[ ]  Static Port .... CLEAR

[ ]  Cabin Door, General Condition and Security Check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/401/Louis_Janos_401A_Preflight_checklist.txt

Rights: licensed

Checklist hash: 0ff05bcb9ebef74236ad0470e55a9deca353d603b9b6db9381ae3c3eff71661f

Verify against the aircraft's own approved documentation before use.