Cessna C170 Cessna 170 — cessna_170b_PREFLIGHT

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

PREFLIGHT

CABIN:

[ ]  Documents

[ ]  Master/Mags/ .... OFF

[ ]  Fuel Quantity P. &  S.

[ ]  Fuel Valve: .... BOTH

[ ]  Flaps .... Down

RIGHT WING:

[ ]  Drain Fuel Sump

[ ]  Main Wheel/Tire/Strut

[ ]  Flap

[ ]  Aileron

[ ]  Wingtip/Lights

[ ]  Fuel Filler Cap

[ ]  Wing Tiedown: .... UNHOOK

NOSE:

[ ]  Engine Oil Level

[ ]  Prop/Spinner

[ ]  Cowling/Air Intake

[ ]  Air Filter

[ ]  Gascolator Drain

[ ]  Fuel Tank Vent

LEFT WING:

[ ]  Main Wheel/Tire/Strut

[ ]  Drain Fuel Sump

[ ]  Stall Warning Vane

[ ]  Pitot Tube/Static

[ ]  Landing Lights

[ ]  Wingtip/Lights

[ ]  Aileron

[ ]  Flap

[ ]  Wing Tiedown: .... UNHOOK

EMPENNAGE:

[ ]  Elevator/Cables

[ ]  Trim Tab

[ ]  Rudder/Light

[ ]  Tailwheel/Tire

[ ]  Steering Cables/Chains

[ ]  Tail Tiedown: .... UNHOOK

ENGINE START

[ ]  Brakes: .... ON

[ ]  Seat Stops: .... LOCKED

[ ]  Master Switch: .... ON

[ ]  Mags: .... BOTH

[ ]  Mixture: .... RICH

[ ]  Prime X 2-4 Pumps

[ ]  Throttle: .... Cracked Open

[ ]  Propeller: .... CLEAR!

[ ]  Pull Ignition/START/Return

[ ]  Throttle: @ 800 RPM, X one minute. .... CHECK

[ ]  Oil Pressure: .... >30 PSI

[ ]  Adjust Mixture

[ ]  Radio .... ON/Set Freq.

[ ]  Strobe: .... ON

RUNUP

[ ]  Brakes: .... ON

[ ]  Cabin Doors/Belts

[ ]  Flight Control Check

[ ]  Flight Instruments

[ ]  Throttle @ 1700 RPM .... CHECK

[ ]  Set Mixture

[ ]  L. & R. Mags: .... Check

[ ]  Carb Heat Check

[ ]  Engine Instruments

[ ]  Throttle: .... Check Idle

[ ]  Txpdr: .... ON/ALT

[ ]  Lights: .... As Req'd.

TAKEOFF

[ ]  Flaps: .... @ 10º

[ ]  Trim: .... @ Takeoff

[ ]  Carb Heat: .... OFF

[ ]  Brakes .... OFF/Heels on Floor

LANDING

[ ]  Seatbelts

[ ]  Fuel Valve: .... BOTH

[ ]  Mixture: .... RICH

[ ]  Carb Heat: .... ON

[ ]  Heels on Floor

SECURING AIRPLANE

[ ]  Radios/Electrical: .... OFF

[ ]  Mixture: .... FULL LEAN

[ ]  Mags/Master Switch: .... OFF

[ ]  Intercom: .... OFF

[ ]  Tiedowns/Chocks

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/C170 Cessna 170/cessna_170b_PREFLIGHT.txt

Rights: licensed

Checklist hash: 152af9bb31d8cb1553f686f117d0c66b09433b440230b7fc3b80328068526a1c

Verify against the aircraft's own approved documentation before use.