Cessna C182F — Nigel_Sutcliffe_cessna_C182_F_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Light Signals" item count differs between passes (7 vs 11); verify nothing was dropped.

Before Engine Start Check

[ ]  Exterior check .... COMPLETE

[ ]  Cabin doors .... CLOSED & LOCKED

[ ]  Seats, belts .... ADJUSTED AND LOCK

[ ]  Control lock .... REMOVE

[ ]  Autopilot .... OFF

[ ]  Electric Trim .... OFF

[ ]  Circuit breakers .... CHECKED IN

[ ]  Radio Master switch .... OFF

[ ]  Master switch .... ON

[ ]  Low voltage warning light .... ON

[ ]  Flaps .... UP

[ ]  Fuel qty .... SUFFICIENT (MIN 10G/SIDE)

[ ]  Fuel selector .... BOTH ON

[ ]  Elevator trim .... TAKE OFF POSITION

[ ]  Rudder trim .... TAKE OFF POSITION

[ ]  Cowl flaps .... OPEN

[ ]  Parking brake .... CHECKED & ON

[ ]  Flying controls .... FULL & FREE

Engine Start Check

[ ]  Beacon .... ON

[ ]  Prime .... AS REQUIRED

[ ]  Carb heat .... COLD

[ ]  Mixture .... RICH

[ ]  Propellor .... HIGH RPM

[ ]  Cowl flaps .... FULLY OPEN

[ ]  Throttle .... CRACKED ½"

[ ]  Prop area .... CLEAR

[ ]  Shout .... "CLEAR PROP"

[ ]  Ignition switch .... START, 30S MAX

[ ]  Starter warning light .... OUT

[ ]  Generator .... CHECK

[ ]  Low voltage warning light .... OUT

[ ]  Autopilot .... OFF

[ ]  Radio master switch .... ON

[ ]  Radios .... ON/STANDBY

[ ]  HSI .... HDG FLAG CLEAR & DG SYNCED

NOTE: If engine has been overprimed, start with ¼ to ½ throttle. Reduce to idle when engine fires. After starting, check for oil pressure indication within 30 seconds (60 seconds when cold). If no indication - shut down.

Taxi & Power Checks

[ ]  Engine RPM .... 800 – 1000

[ ]  Brakes .... CHECK

[ ]  Instruments .... CORRECT SENSE

[ ]  Parking brake .... SET

[ ]  Engine gauges .... WITHIN GREEN ARC

[ ]  Throttle .... 1700 RPM

[ ]  Carb heat .... CHECK THEN COLD

[ ]  Magnetos .... CHECK

NOTE: Minimum drop 150 rpm, difference 50 rpm

[ ]  Propellor .... CYCLE THEN SET HIGH

[ ]  Engine gauges .... WITHIN GREEN ARC

[ ]  Suction .... 4" TO 6"

[ ]  Generator .... CHECK CHARGING RATE

[ ]  Throttle .... CHECK IDLE 800 – 1000 RPM

Before Takeoff

[ ]  Fuel selector .... BOTH ON & SUFFICIENT

[ ]  Carb heat .... OFF

[ ]  Throttle friction .... SET

[ ]  Mixture .... FULLY RICH

[ ]  Propellor .... MAX RPM

[ ]  Cowl flaps .... OPEN

[ ]  Engine gauges .... CHECK

[ ]  Flight instruments .... CHECK

[ ]  Flaps .... 0 – 20

[ ]  Trims .... SET FOR TAKE OFF

[ ]  Flying controls .... FULL & FREE

[ ]  Cabin doors .... CLOSED & LOCKED

APPROACHING RUNWAY:

[ ]  Transponder .... SET

Takeoff

NORMAL

[ ]  Flaps .... UP

[ ]  Carb heat .... OFF

[ ]  Power .... FULL & 2600RPM

[ ]  Rotate .... 60MPH

[ ]  Climb .... 90MPH

NOTE: Until clear of obstacles then set up normal climb speed.

MAX PERFORMANCE

[ ]  Flaps .... 20

[ ]  Carb heat .... OFF

[ ]  Brakes .... HOLD

[ ]  Power .... FULL & 2600

[ ]  Brakes .... RELEASE

[ ]  Climb .... 60MPH

NOTE: Until clear of all obstacles then set up max performance climb.

[ ]  Flaps .... UP

NOTE: After obstacles and above 80mph

Climb

NORMAL

[ ]  Airspeed .... 100 – 120MPH

[ ]  Power .... 23" & 2450RPM

[ ]  Mixture .... FULL RICH UNLESS ROUGH

[ ]  Cowl flaps .... AS REQD

MAXIMUM PERFORMANCE

[ ]  Airspeed .... 88 TO 84MPH

[ ]  Power .... FULL & 2600RPM

[ ]  Mixture .... FULL RICH UNLESS ROUGH

[ ]  Cowl flaps .... AS REQD

Cruise

[ ]  Power .... 15 - 23" & 2200 – 2450RPM

[ ]  Cowl flaps .... AS REQD

[ ]  Mixture .... LEAN AS REQD

Fuel tank change

[ ]  Fuel selector .... AS REQD

[ ]  Fuel pressure .... CHECK

Descent

[ ]  Mixture .... RICH

[ ]  Power .... SET, MIN 15"

[ ]  Cowl flaps .... CLOSED

[ ]  Carb heat .... ON, IF REQD

[ ]  Fuel .... SUFFICIENT

[ ]  Instruments .... CHECK

[ ]  Radios .... CHECK

[ ]  Altimeter .... SET

Before Landing

[ ]  Propellor .... HIGH RPM

[ ]  Cowl flaps .... CLOSED

[ ]  Fuel selector .... BOTH

[ ]  Mixture .... RICH

[ ]  Carb heat .... CHECK FOR ICING

[ ]  Brakes .... OFF

[ ]  Airspeed .... 80 – 90MPH

[ ]  Flaps .... 0 – 40 BELOW 110MPH

[ ]  Airspeed .... 70 – 80MPH, FLAPS DOWN

After landing

[ ]  Cowl flaps .... OPEN

[ ]  Flaps .... UP

[ ]  Transponder .... STANDBY

[ ]  Landing light .... OFF

Shutdown

[ ]  Engine gauges .... CHECK

[ ]  Radio master switch .... OFF

[ ]  Power .... 1000RPM

[ ]  Magnetos .... CHECK

[ ]  Mixture .... CUT-OFF

[ ]  Ignition switch .... OFF

[ ]  Beacon .... OFF

[ ]  Master switch .... OFF

[ ]  Fuel selector .... OFF

[ ]  Cowl Flaps .... CLOSE

Quantities

[ ]  Fuel capacity per side .... 32.5 USG / 27 IMP G

[ ]  Usable fuel per side .... 31 USG / 26 IMP G

[ ]  Oil .... 12 QTS

[ ]  Min Oil .... 9 QTS

[ ]  Oil (extended flight) .... 12 QTS

[ ]  Nosewheel pressure .... 45 PSI

[ ]  Mainwheel pressure .... 32 PSI

V-Speeds (IAS MPH)

[ ]  Never exceed .... Vne 193

[ ]  Normal max .... Vno 160

[ ]  Maneuvering .... Va 128

[ ]  T/O safety speed .... Vmca 60 Max MP 2600rpm

[ ]  Climb 20 flap .... Vx 60 Max MP 2600rpm

[ ]  Climb normal .... 90 23" MP 2450rpm

[ ]  Best glide speed .... 80

[ ]  Flap 10 limit .... Vfe 160

[ ]  Flap 10 – 40 .... Vfe 110

[ ]  Stall clean .... Vs1 64

[ ]  Stall 40 flap .... Vso 55

[ ]  Max crosswind .... 15

Field Length

NOTE: Standard day, zero wind, hard surface, sea level & clearing 50' obstacle.

NOTE: TAKE OFF: 10 flap, 28000lb. 625'/195m ground run to 60 mph & 1295'/370m total distance at obstacle.

NOTE: On short dry grass add 100'/30m to both ground run and total distance.

NOTE: LANDING: 40 flap, 28000lb. 590'/180m ground roll from 69 mph approach speed & 1350'/415m total distance from obstacle.

NOTE: On short dry grass 300'/90m to both ground roll and total distance.

Light Signals

NOTE: Signal | On Ground | In Flight

NOTE: Steady Green: Takeoff / Land

NOTE: Flashing Green: Taxi / Return to land

NOTE: Steady Red: Stop / Give way

NOTE: Flashing Red: Clear runway / Do not land

NOTE: Flashing white: Return to ramp / --

WARNING: Red/Green alternating: WARNING – USE CAUTION

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/C182F/Nigel_Sutcliffe_cessna_C182_F_checklist.txt

Rights: licensed

Checklist hash: 6fcef99c14254ff065cec54b270a77bb5405b85e764bca1256a950cec5cc84a1

Verify against the aircraft's own approved documentation before use.