Cessna T210N — Sam_Dawson_T210_ground_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Starting Engine

[ ]  Preflight Inspection .... Complete

[ ]  CRM/Passenger Briefing .... Complete

[ ]  Seats and Seat Belts .... Adjust & Lock

[ ]  Brakes .... Test & Set

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Cowl Flaps .... OPEN

[ ]  Mixture .... RICH

[ ]  Propeller .... HIGH RPM

[ ]  Landing Gear Lever .... DOWN

[ ]  Avionics Master Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Circuit Breakers .... Check In

Starting Engine

[ ]  Master Switch .... ON

[ ]  Landing Gear Lights/Horn .... PRESS TO TEST

[ ]  Exterior Lights .... As Required

[ ]  Throttle .... CLOSED

[ ]  Auxiliary Fuel Pump Switch .... ON

[ ]  Throttle .... ADVANCE 3-5 seconds, then RETURN to IDLE

[ ]  Auxiliary Fuel Pump Switch .... OFF

[ ]  Propeller Area .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Throttle .... ADVANCE slowly

[ ]  Ignition Switch .... RELEASE after start

[ ]  Oil Pressure .... Check

[ ]  Low Voltage Light .... OFF (at about 800 RPM)

Before Taxi

[ ]  Flaps .... Retract

[ ]  Throttle .... 1000 RPM

[ ]  Avionics Master (Radios) .... On

[ ]  ATIS / AWOS .... Receive

[ ]  Clearance .... Obtain

[ ]  Avionics / Transponder .... Set / SBY

[ ]  Flight Instruments .... Check & Set

[ ]  Engine Instruments .... Check

Taxi

[ ]  Brakes & Steering .... Check

[ ]  Flight Instruments .... Check

Before Takeoff

[ ]  Brakes .... SET

[ ]  Cabin Doors .... Closed & Locked

[ ]  Seats / Seat Belts .... Upright & Secured

[ ]  Flight Controls .... Free & Correct

[ ]  Clock .... Check

[ ]  Fuel Quantity .... Check

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Cowl Flaps .... FULL OPEN

[ ]  Elevator and Rudder Trim .... SET for takeoff

[ ]  Flaps .... SET for Takeoff

[ ]  Flight Instruments .... Check & Set

[ ]  Radios / Avionics .... Set for Departure

[ ]  Mixture .... Enrichen

[ ]  Throttle .... 1700 RPM

[ ]  Magnetos .... Check (drop <150, 50 differential)

[ ]  Propeller .... CYCLE

[ ]  Engine Instruments / Ammeter .... CHECK

[ ]  Vacuum Gage .... CHECK

[ ]  Throttle .... Idle / 1000 RPM

[ ]  Autopilot .... OFF

[ ]  Departure Briefing .... Complete - Type T/O: Vr-65, Vy-100, Vx-78, Power, Heading, Altitude

Shutdown and Securing

[ ]  Avionics Master Switch .... Off

[ ]  Exterior Lights (except beacon) .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Idle Cut Off

[ ]  Ignition Switch .... Off

[ ]  Battery and Alternator .... Off

[ ]  Gust Lock .... Install

[ ]  Post Flight Inspection .... Accomplish

[ ]  Tie downs and Chocks .... Install

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cessna/T210N/Sam_Dawson_T210_ground_checklist.txt

Rights: licensed

Checklist hash: 5499ac7316e239f67d58235fac451eff6b905335aa83d2e08d9db7da2ee6461d

Verify against the aircraft's own approved documentation before use.