Cirrus SR20 — alexbruzzano_sr20_arnav_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Preflight — Cabin

[ ]  AROW .... COMPLETED

[ ]  CAPS SAFETY PIN .... REMOVED

[ ]  FIRE EXTINGUISHER .... AVAILABLE

[ ]  EGRESS HAMMER .... AVAILABLE

[ ]  HOBBS METER .... NOTE TIME

[ ]  CIRCUIT BREAKERS .... IN

[ ]  TURN COORDINATOR .... NORMAL

[ ]  AVIONICS POWER .... OFF

[ ]  BATTERY MASTER SWITCH .... ON

[ ]  ANNUNCIATOR LIGHTS .... TEST

[ ]  STANDBY VACUUM .... VERIFY OPS

[ ]  FLAPS .... 100%

[ ]  FUEL QUANTITY .... CHECK

[ ]  FUEL SELECTOR .... FULLEST

[ ]  LIGHTS & PITOT HEAT .... CHECK

[ ]  BATTER MASTER SWITCH .... OFF

Preflight — Left Fuselage

[ ]  COMM 1 & 2 ANTENNA .... CONDITION

[ ]  WING/FUSELAGE FARING .... CHECK

[ ]  BAGGAGE DOOR .... LOCKED

[ ]  STATIC PORT .... CLEAR

[ ]  PARACHUTE COVER .... CONDITION

Preflight — Empennage

[ ]  TIE DOWN .... REMOVED

[ ]  HORIZ/VERT STAB .... CONDITION

[ ]  ELEVATOR .... CONDITION

[ ]  RUDDER .... CONDITION

Preflight — Right Fuselage

[ ]  STATIC PORT .... CLEAR

[ ]  PARACHUTE COVER .... CONDITION

[ ]  WING/FUSELAGE FARING .... CHECK

Preflight — Right Wing

[ ]  FLAP .... CONDITION

[ ]  AILERON .... CONDITION

[ ]  WINGTIP .... CONDITION

[ ]  STROBE/NAV LIGHT .... CONDITION

[ ]  FUEL VENT .... CLEAR

[ ]  LEADING EDGE/STALL STRIPS .... CONDITION

[ ]  FUEL CAP .... QUANTITY/SECURE

[ ]  STALL WARNING .... CLEAR/TEST

[ ]  CABIN AIR VENT .... CLEAR

[ ]  GEAR FARINGS .... CONDITION

[ ]  TIRE (53 psi) .... CONDITION

[ ]  FUEL DRAINS .... SUMP

[ ]  WHEEL BRAKES .... CONDITION

[ ]  CHOCKS .... REMOVE

[ ]  TIE DOWN ROPES .... REMOVE

Preflight — Nose

[ ]  COWLING .... SECURE

[ ]  EXHAUST PIPES .... CONDITION

[ ]  TRANSPONDER ANTENNA .... CONDITION

[ ]  FUEL DRAIN .... SUMP

[ ]  TOW BAR .... REMOVE

[ ]  STRUT .... CONDITION

[ ]  GEAR FARING .... CONDITION

[ ]  TIRE (40 psi) .... CONDITION

[ ]  PROPELLER .... CONDITION

[ ]  SPINNER .... CONDITION/OIL LEAKS

[ ]  AIR INLETS .... CLEAR

[ ]  ALTERNATOR BELT .... CONDITION/TENSION

[ ]  LANDING LIGHT .... CONDITION

[ ]  ENGINE OIL .... CHECK/SECURE

[ ]  EXTERNALN POWER DOOR .... SECURE

Preflight — Left Wing

[ ]  CABIN AIR VENT .... LEAR

[ ]  GEAR FARINGS .... CONDITION TIRE (53 psi)

[ ]  CONDITION .... CONDITION

[ ]  FUEL DRAINS .... SUMP

[ ]  WHEEL BRAKES .... CONDITION

[ ]  CHOCKS .... REMOVE

[ ]  TIE DOWN ROPES .... REMOVE

[ ]  LEADING EDGE/STALL STRIPS .... CONDITION

[ ]  FUEL CAP .... QUANTITY/SECURE

[ ]  FUEL VENT .... CLEAR

[ ]  PITOT TUBE .... CLEAR

[ ]  STROBE/NAV LIGHT .... CONDITION

[ ]  AILERON .... CONDITION

[ ]  FLAP .... CONDITION

Before Starting Engines

[ ]  PREFLIGHT .... COMPLETE

[ ]  PASSENGERS .... BRIEFED

[ ]  SEATS & BELTS .... SECURE

[ ]  BRAKES .... SET

[ ]  STATIC SOURCE .... NORMAL

[ ]  EXTERNAL POWER .... AS REQ

Starting Engine

[ ]  PROPELLER AREA .... CLEAR

[ ]  DOORS .... LATCHED

[ ]  BATTERY MASTER SWITCH .... ON

[ ]  STROBE LIGHTS .... ON

[ ]  MIXTURE .... FULL RICH

[ ]  THROTTLE .... FULL FWD

[ ]  FUEL PUMP .... PRIME THEN BOOST

[ ]  THROTTLE .... IDLE

[ ]  IGNITION SWITCH .... START

[ ]  THROTTLE .... 1000 RPM

[ ]  OIL PRESSURE .... CHECK

[ ]  VACUUM GAUGE .... CHECKED

[ ]  ALTERNATOR .... ON

[ ]  AVIONICS SWITCH .... ON

[ ]  VACUUM ANNUNCIATOR .... OUT

[ ]  AMMETER .... CHECKED

[ ]  ENGINE GAUGES .... CHECKED

[ ]  FUEL PUMP .... OFF

[ ]  EXTERNAL POWER .... DISCONN

[ ]  VACUUM SYSTEM CB .... IN

Cold Start

Cold start

[ ]  MIXTURE .... FULL RICH

[ ]  THROTTLE .... FULL FWD

[ ]  FUEL PUMP .... PRIME 10 SEC THEN BOOST

[ ]  THROTTLE .... OPEN ¼ INCH

[ ]  IGNITION SWITCH .... START

NOTE: continue with normal checklist

Hot Start

Hot start

[ ]  MIXTURE .... FULL RICH

[ ]  THROTTLE .... OPEN ¼ INCH

[ ]  FUEL PUMP .... BOOST

[ ]  IGNITION switch .... START

NOTE: continue with normal checklist

Before Taxi

[ ]  AUTOPILOT .... TEST

[ ]  FLAPS .... UP

[ ]  CABIN HEAT/DEFROST .... SET

[ ]  RADIOS/AVIONICS .... SET

[ ]  LIGHTS .... AS REQUIRED

[ ]  PARKING BRAKE .... RELEASE

Taxi

[ ]  BRAKES & STEERING .... CHECKED

[ ]  INSTRUMENTS .... CHECKED

Before Takeoff

[ ]  BRAKES .... SET

[ ]  SEATS & BELTS .... SECURE

[ ]  DOORS .... LATCHED

[ ]  TRIM .... SET T/OFF

[ ]  FLIGHT CONTROLS .... CHECKED

[ ]  HEADING INDICATOR .... CHECK/SLAVE

[ ]  MIXTURE .... FULL RICH

[ ]  FUEL PUMP .... BOOST

[ ]  FUEL QUANTITY .... CONFIRM

[ ]  FUEL TANK .... SWITCH

[ ]  PROPELLER .... CHECK

NOTE: Oil temp minimum 100°

NOTE: Increase to detent: note 2000 RPM then drop

[ ]  THROTTLE .... 1700 RPM

[ ]  VACUUM GAUGE .... CHECK

[ ]  ENGINE INSTRUMENTS .... CHECK

[ ]  ALTERNATOR .... CHECK

[ ]  MAGNETOS .... CHECK

NOTE: 150 RPM/75 RMP max difference

[ ]  ALTERNATE AIR .... CHECK

[ ]  THROTTLE .... 1000 RPM

[ ]  FLAPS .... 50%

[ ]  COMM/NAV RADIOS .... SET

[ ]  FLIGHT PLAN .... LOADED

[ ]  PITOT HEAT .... AS REQUIRED

[ ]  PARKING BRAKE .... RELEASE

Lineup

[ ]  FINAL .... CLEAR

[ ]  LANDING LIGHT .... ON

[ ]  FLAPS .... SET

[ ]  MIXTURE .... RICH

[ ]  FUEL PUMP .... BOOST

[ ]  TIME OFF .... NOTE

Climb

NOTE: Above 500 AGL

[ ]  FLAPS .... UP

[ ]  ENGINE INSTRUMENTS .... CHECK

[ ]  AIRSPEED .... 105 KIAS

NOTE: Above 1000 AGL

[ ]  FUEL PUMP .... OFF

Cruise

[ ]  CRUISE POWER .... SET

[ ]  MIXTURE .... LEAN

[ ]  ENGINE INSTRUMENTS .... CHECK

[ ]  EXTERNAL LIGHTS .... AS REQUIRED

[ ]  HEADING INDICATOR .... CHECK

Descent

[ ]  BRAKE PRESSURE .... CHECK

[ ]  HEADING INDICATOR .... CHECK

[ ]  ALTIMETER .... SET

[ ]  MIXTURE .... RICH

[ ]  FUEL PUMP .... BOOST

[ ]  FUEL TANK .... FULLEST

[ ]  CABIN HEAT/DEFROST .... AS REQUIRED

VFR Maneuvers

STALL

NOTE: 11"

STEEP TURNS

NOTE: 2100-2300 RPM

NOTE: 120 KIAS

SLOW FLIGHT

NOTE: 11"

Before Landing

[ ]  SEATS & BELTS .... SECURE

[ ]  MIXTURE .... RICH

[ ]  FUEL PUMP .... BOOST

[ ]  FLAPS .... AS REQUIRED

[ ]  LANDING LIGHT .... ON

[ ]  AUTO PILOT .... DISENGAGE

Precision/Non Precision Approach

PRECISION/NON PRECISION APPROACH

NOTE: ►Hold & PT

NOTE: 22" & 120 KIAS

NOTE: ►Pre landing

NOTE: 20" & 100 KIAS

NOTE: ►@ GS INT

NOTE: 17" 50% 100 KIAS

NOTE: ►@ MDA

NOTE: 24" & 100 KIAS

VFR Traffic Pattern

VFR TRAFFIC PATTERN

NOTE: ►Downwind

NOTE: 20" & 100 KIAS

NOTE: ►Abeam

NOTE: 12" 50% 90 KIAS        12" 0% 95 KIAS

NOTE: ►Base

NOTE: 12" 100% 85 KIAS       12" 0% 95 KIAS

NOTE: ►Final

NOTE: 12" 80 KIAS            12" 0% 90 KIAS

After Landing

[ ]  FUEL PUMP .... OFF

[ ]  THROTTLE .... 1000 RPM

[ ]  FLAPS .... UP

[ ]  TRANSPONDER .... STANDBY

[ ]  LIGHTS .... AS REQUIRED

[ ]  PITOT HEAT .... OFF

Engine Shutdown

[ ]  AVIONICS SWITCH .... OFF

[ ]  MIXTURE .... CUTOFF

[ ]  MAGNETOS .... OFF

[ ]  ALL LIGHTS .... OFF

[ ]  BAT & ALT MASTER SWITCH .... OFF

[ ]  CAPS SAFETY PIN .... INSERT

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cirrus/SR20/alexbruzzano_sr20_arnav_checklist.txt

Rights: licensed

Checklist hash: 5ca3c1ecd38e226ee0a050af04a1d5c3d0e96273a5a5ddec4e4e22a63ece0683

Verify against the aircraft's own approved documentation before use.