Cirrus SR20 — Stephen_Shields_SR20_CHECKLIST_v1.3 (Mar-06)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "BEFORE TAKEOFF RUNUP" item count differs between passes (16 vs 18); verify nothing was dropped.

content_gap: Section "STARTING ENGINE" item count differs between passes (19 vs 23); verify nothing was dropped.

BEFORE STARTING ENGINE

[ ]  Preflight .... COMPLETED

[ ]  Place ignition keys on dash .... PLACE IGNITION KEYS ON DASH

[ ]  Hobbs Meter .... NOTE TIME IN LOGBOOK

[ ]  Emergency Equipment .... ON BOARD

[ ]  Passengers .... BRIEFED

[ ]  Seats, Seat Belts & Harnesses .... ADJUST & SECURE

STARTING ENGINE

WARNING: KEEP ALL PERSONNEL & STARTING CABLES FREE OF PROPELLER

[ ]  External Power (if applicable) .... CONNECT

[ ]  Brakes .... HOLD

[ ]  Battery Master Switches .... ON

Check voltage

[ ]  Strobe Lights .... ON

[ ]  Mixture .... FULL RICH

[ ]  Power Lever .... OPEN ¼ INCH

[ ]  Fuel Pump .... PRIME (only on cold first start), then BOOST

NOTE: NO BOOST REQUIRED IF ENGINE IS STILL WARM (off less than 60 min. in warm weather, less than 30 min. in cold weather)

[ ]  Propeller Area .... CLEAR

[ ]  Power Lever .... OPEN ¼ INCH

[ ]  Ignition Switch .... START

[ ]  Power Lever .... RETARD TO 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Alt Master Switches .... ON

[ ]  Avionics .... ON

[ ]  Engine Parameters .... MONITOR

[ ]  External Power (if applicable) .... DISCONNECT

[ ]  Ammeter .... CHECK

BEFORE TAKEOFF RUNUP

NOTE: COMING FROM e-checklist

[ ]  Boost Pump .... CONFIRM ON

[ ]  Power Level .... 1700 RPM

[ ]  Alternators .... CHECK

[ ]  Pitot Heat .... ON

[ ]  Avionics .... ON

[ ]  Navigation Lights .... ON

[ ]  Landing Light .... ON (3-5 seconds)

NOTE: May be left on if desired for safety

[ ]  ALT 1 & ALT .... CAUTION LIGHTS OUT & POSITIVE AMPS

[ ]  Voltage .... CHECK

[ ]  Pitot Heat .... OFF

Check for voltage drop

[ ]  Magnetos .... CHECK LH & RH

NOTE: RPM Drop ≤ 150, difference between magnetos ≤ 75 RPM

[ ]  Power Level .... DECREASE TO 1000 RPM

NOTE: RETURN TO e-checklist

SHUT DOWN

[ ]  Avionics Switch .... OFF

[ ]  Fuel Pump .... OFF

[ ]  Ignition Switch .... TOGGLE ON/OFF

[ ]  Mixture .... CUTOFF

[ ]  Magnetos .... OFF (remove keys)

[ ]  Bat-Alt Master Switches .... OFF (right to left)

[ ]  ELT .... TRANSMIT LIGHT OUT

[ ]  CAPS Handle .... INSERT PIN

[ ]  Hobbs Meter .... NOTE TIME IN LOGBOOK

[ ]  Flight Plan .... CLOSE

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cirrus/SR20/Stephen_Shields_SR20_CHECKLIST_v1.3 (Mar-06).txt

Rights: licensed

Checklist hash: 7abf087ac71adcef31e1eb42bef01331394e31b717bdb064b405e6a065a7e879

Verify against the aircraft's own approved documentation before use.