Cirrus SR22 — sr22_checklist_stephen_shirley

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Initial

[ ]  Weather & Den. Alt.

[ ]  Weight & Balance

[ ]  Performance Req.

[ ]  Flight Plan

[ ]  A.R.R.O.W. .... CHECK

[ ]  Avionics .... OFF

[ ]  Bat 2 Master .... ON

[ ]  Check Volts .... 23-25V

[ ]  Flap Light .... OFF

[ ]  Bat 1 Master .... ON

[ ]  Fuel Quantity

[ ]  Fullest Tank

[ ]  Flaps .... 100%, Light

[ ]  Check Flaps .... 100%

[ ]  Oil Annunciator .... ON

[ ]  Check Ann. Lights .... CHECK

[ ]  Check Volt .... 23-25V

[ ]  Masters .... OFF

[ ]  Alt. Static .... NORMAL

[ ]  Fire Extinguisher Egress Hammer

[ ]  CAPS pin .... IN

[ ]  Hobbs Time

Exterior Summary

[ ]  Check Fuel / Oil

[ ]  Caps/Drains/Vents

[ ]  Prop/Air Intake

[ ]  Exhaust System

[ ]  Surfaces & Controls

[ ]  Pitot/Static Ports

[ ]  Gear/Tires/Brakes

[ ]  Antennas

[ ]  Ties/Chocks/Doors

[ ]  Final Walk Around

Start

[ ]  Brief Passengers .... COMPLETE

[ ]  Remove CAPS pin .... REMOVE

[ ]  Masters .... ON

[ ]  Strobes .... ON

[ ]  Flaps .... UP

[ ]  Mixture .... RICH

[ ]  Power .... FORWARD

[ ]  PRIME .... 8 BUMPS, BOOST ON

[ ]  Power .... CLOSED

[ ]  Power .... 1/4" IF HOT

[ ]  Clear Propeller .... CLEAR

[ ]  Brakes .... HOLD

[ ]  Ignition .... START

AFTER EACH ATTEMPT:

[ ]  Power .... FORWARD

[ ]  Power .... CLOSED

[ ]  Power .... 1000 RPM

[ ]  Check Oil Pressure

[ ]  Monitor Engine

[ ]  Check Alternator 1

[ ]  Flaps .... UP

[ ]  Avionics .... ON

[ ]  GPS & Autopilot

[ ]  Frequencies

[ ]  ATIS/AWOS

[ ]  Altimeter

Taxi

[ ]  Seatbelts .... SECURE

[ ]  Cabin Heat/Defrost

[ ]  Lights .... AS REQUIRED

[ ]  Check Brakes .... CHECK

[ ]  Check HSI/Comp .... CHECK

[ ]  Check AI/TC .... CHECK

Run-Up

[ ]  Brakes .... HOLD

[ ]  Trim .... TAKEOFF

[ ]  Controls .... FREE & CORR

[ ]  Flaps .... 50% & CHECK

[ ]  Instruments .... CHECK

[ ]  Fuel .... FULLEST TANK

[ ]  Power .... 2000 RPM

[ ]  Check Prop. .... CHECK

[ ]  Power .... 1700 RPM

[ ]  Oil Pressure/Temp.

[ ]  Pitot Heat & Lights

[ ]  Check Alts/Volts .... 28

[ ]  Mags .... <150 DROP, <75 DIFF

[ ]  Power .... 1000 RPM

[ ]  Radios/Clearance

[ ]  Transponder Code

[ ]  Baro setting in Altimeter & S-A

[ ]  Initial altitude & alert in S-A

Taking Active

[ ]  Boost Pump .... ON

[ ]  Flaps .... 50%

[ ]  Mixture .... RICH

Takeoff

[ ]  Pitot Heat

[ ]  Flaps .... 50% & CHECK

[ ]  Lights

[ ]  Boost Pump .... ON

[ ]  Power .... FULL FORWARD

[ ]  Adjust Fuel Flow

[ ]  Engine Instruments

[ ]  Release Brakes .... RELEASE

[ ]  Rotate .... 70-73 KIAS

[ ]  Flaps up .... 80 KIAS

[ ]  Vx 80, Vy 100

[ ]  Normal 110-120

Climb

[ ]  Full Power

[ ]  Fuel Flow per list

[ ]  Engine Instruments

[ ]  Boost Pump .... OFF

[ ]  O2 .... ON AT 8000'

Cruise

[ ]  Set Power

[ ]  Lean as Required

[ ]  Engine Instruments

[ ]  Monitor Fuel Flow

[ ]  EGT 75 ROP~1350

[ ]  Oil temp~200

Descent

[ ]  Set Altimeter .... SET

[ ]  Fuel .... FULLEST TANK

[ ]  Mixture .... FULL RICH

[ ]  Marker Beacons

[ ]  Seatbelts .... SECURE

Before Landing

[ ]  Mixture .... FULL RICH

[ ]  Boost Pump .... ON

[ ]  Flaps .... AS REQUIRED

[ ]  Lights .... AS REQUIRED

[ ]  Autopilot .... OFF

[ ]  Downwind 100

[ ]  Base 90

[ ]  Short Final .... 80 KIAS

[ ]  Short Field .... 77 KIAS

[ ]  Active gentle flare

Clear Active

[ ]  Flaps .... UP

[ ]  Power .... 1000 RPM

[ ]  Pitot Heat .... OFF

[ ]  Trim .... TAKEOFF

Shutdown

[ ]  Record Flight Time

[ ]  Avionics .... OFF

[ ]  Boost Pump .... OFF

[ ]  Mixture .... LEAN/CUTOFF

[ ]  Ignition Switch .... OFF

[ ]  Masters .... OFF

[ ]  Lights .... OFF

Securing

[ ]  Check ELT

[ ]  CAPS pin .... REPLACE

[ ]  Hobbs Time

[ ]  Chocks .... IN

[ ]  Secure Doors

[ ]  Tie Downs/Covers

[ ]  Close Flight Plan

Reference Speeds

[ ]  VR Rotation - 70

[ ]  VX Best Angle – 80

[ ]  VY Best Rate – 100

[ ]  VS0 Stall w/flaps – 59

[ ]  VS Stall w/o flaps – 70

[ ]  Best Glide – 90

[ ]  VO Maximum Turbulence – 133

[ ]  VNO Maximum Cruise – 178

[ ]  VNE Never Exceed – 201

[ ]  VFE Flaps Extended – 104 top of white arc

[ ]  VFE ½ Flaps Ext. – 120

[ ]  Maximum X-Wind Demo – 20

Frequencies

[ ]  UNICOM: 122.7 122.8 122.95 123.0 123.05

[ ]  Multicom: 122.9

[ ]  Air-Air: 122.75 122.85

[ ]  Flight Service: 122.2 122.1 122.6 123.6

[ ]  Flight Watch: 122.0

[ ]  Guard: 121.5

Weights & Fluids

[ ]  MGTOW: 3400 lbs

[ ]  Empty Weight: 2290 lbs

[ ]  Maximum Baggage Cpt: 130 lbs

[ ]  Tire Inflation: Nose 40 +2/-0, Mains 62 +2/-0

[ ]  Oil: Exxon Elite 15W-50, Minimum 6 qts.

Power Loss Immediately After Takeoff  [EMERGENCY]

NO RESTART

[ ]  Best Glide [MEMORY] .... 88 KIAS (87 KIAS 2900 LBS)

[ ]  Mixture [MEMORY] .... CUTOFF

[ ]  Fuel Selector .... OFF

[ ]  Flaps .... DOWN/AS REQUIRED

[ ]  Masters & Ignition Switch .... OFF

[ ]  Power Lever .... IDLE

[ ]  Seatbelts .... SECURED

Power Loss In Flight  [EMERGENCY]

[ ]  Best Glide [MEMORY] .... 88 KIAS (87 KIAS 2900 LBS)

[ ]  Mixture .... FULL RICH

[ ]  Fuel Selector .... SWITCH TANKS

[ ]  Fuel Pump .... BOOST

[ ]  Ignition Switch .... CHECK, BOTH

Engine Airstart  [EMERGENCY]

[ ]  Battery Masters .... ON

[ ]  Power Lever .... 1/2" OPEN

[ ]  Mixture .... CUTOFF

[ ]  Fuel Selector .... SWITCH TANKS

[ ]  Ignition Switch .... BOTH

[ ]  Fuel Pump .... BOOST

[ ]  Alt. Masters .... OFF

[ ]  Starter .... ENGAGE (IF NOT WINDMILLING)

[ ]  Mixture .... SLOWLY INCREASE

[ ]  Power Lever .... SLOWLY INCREASE

[ ]  Alt. Masters .... ON

Forced Landing (Engine Out)  [EMERGENCY]

[ ]  Best Glide [MEMORY] .... 88 KIAS (87 KIAS 2900 LBS)

[ ]  Squawk .... 7700

[ ]  Declare Emergency .... GUARD 121.5

[ ]  Seatbelts .... SECURED

[ ]  ELT .... ON

[ ]  Power Lever .... IDLE

[ ]  Mixture .... CUTOFF

[ ]  Fuel Selector .... OFF

[ ]  Ignition .... OFF

[ ]  Boost Pump .... OFF

[ ]  Flaps .... AS NEEDED

[ ]  Master .... OFF

CAPS Deployment  [EMERGENCY]

[ ]  Minimum Possible Airspeed [MEMORY] .... 133 MAX DEMO'D

[ ]  Mixture .... CUTOFF

[ ]  Remove Cover .... REMOVE

[ ]  Remove Safety Pin .... REMOVE

[ ]  Pull Handle [MEMORY] .... DOWN

[ ]  Mixture .... CHECK/CUTOFF

[ ]  Fuel Selector .... OFF

[ ]  Masters .... OFF

[ ]  Ignition .... OFF

[ ]  Boost Pump .... OFF

[ ]  ELT .... ON

[ ]  Seatbelts .... SECURED

[ ]  Loose Items .... SECURED

[ ]  Cross Arms Across Chest

[ ]  Grasp Harness, Keep Torso Erect

[ ]  Evacuate Quickly

Smoke/Fume Elimination  [EMERGENCY]

[ ]  Heater .... OFF

[ ]  Air Vents .... OPEN, FULL COLD

[ ]  Land ASAP

[ ]  Cabin Doors .... UNLATCH (IF NECESSARY)

Engine Fire During Start  [EMERGENCY]

[ ]  Mixture [MEMORY] .... CUTOFF

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Power Lever .... FORWARD

[ ]  Starter .... CRANK

Engine Fire In Flight  [EMERGENCY]

[ ]  Mixture [MEMORY] .... CUTOFF

[ ]  Power Lever .... IDLE

[ ]  Fuel Selector .... OFF

[ ]  Ignition .... OFF

[ ]  Perform Forced Landing Checklist

Wing Fire In Flight  [EMERGENCY]

[ ]  Pitot Heat .... OFF

[ ]  Navigation Lights .... OFF

[ ]  Strobes .... OFF

[ ]  Slip to keep flames away from tank

[ ]  Dive to blow out fire if practicable

[ ]  Land ASAP

Cabin Fire In Flight  [EMERGENCY]

[ ]  Masters .... OFF

[ ]  Heater .... OFF

[ ]  Air Vents .... CLOSED

[ ]  Fire Extinguisher .... ACTIVATE

WHEN FIRE EXTINGUISHED…

[ ]  Air Vents .... OPEN, FULL COLD

[ ]  Avionics .... OFF

[ ]  All other switches .... OFF

[ ]  Land ASAP

[ ]  Master Switches .... ON (ONLY IF REQUIRED)

[ ]  Avionics .... ON (ONLY IF REQUIRED)

Inadvertent Icing  [EMERGENCY]

[ ]  Pitot Heat .... ON

[ ]  Exit Icing Conditions

[ ]  Cabin Heat .... MAXIMUM

[ ]  Windshield Defrost .... FULL OPEN

[ ]  Alternate Induction Air .... ON

[ ]  Land Faster, Consider Reduced/No Flaps

Emergency Descent  [EMERGENCY]

[ ]  Flaps .... UP

[ ]  Power Lever .... IDLE

[ ]  Mixture .... FULL RICH

[ ]  Airspeed .... VNE (201 KIAS)

[ ]  Forward Slip .... IF NECESSARY

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cirrus/SR22/sr22_checklist_stephen_shirley.txt

Rights: licensed

Checklist hash: e33049e6f0c2c53ed5ce6ee184a9898348b7cc023fdcad462d6a7d31498e5a05

Verify against the aircraft's own approved documentation before use.