Cirrus SR22 — sr22_checklist_steve_kahn

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Climb out and level" item count differs between passes (2 vs 4); verify nothing was dropped.

content_gap: Section "Run-up / Pre-take-off" item count differs between passes (17 vs 18); verify nothing was dropped.

Pre-flight Inspection

[ ]  Fuel sump -- 5 drains

[ ]  Fuel tanks -- Check levels

[ ]  O-rings -- Inspect

[ ]  Oil level -- 6 - 8 quarts

[ ]  Tire pressure -- Nose 40; main 62 psi

[ ]  Wheel fairings -- Tight and clean

[ ]  Windshield -- Cleaned

[ ]  Control Surfaces -- Check

[ ]  Prop & spinner -- Check

[ ]  Chocks & ropes -- Remove

[ ]  Pitot cover & cowl plugs -- Off

[ ]  Baggage door -- Closed & locked

[ ]  Pitot heat -- Check

[ ]  Nav, Strobe, landing lights -- Check

Pre-start

[ ]  Circuit Breakers -- In

[ ]  CAPS pin -- Remove

[ ]  Doors locked/Seats -- Secure

[ ]  Seat belts, harnesses -- On

[ ]  Passenger Briefing -- Complete

[ ]  Bat #2 master -- Check 23-25V

[ ]  Flap position light -- Off

[ ]  All Bat-Alt Master switches -- On

[ ]  Strobe light -- On

[ ]  Brakes -- Test and set

Start

[ ]  Mixture -- Rich

[ ]  Throttle -- Full forward

[ ]  Prime -- 5 -10 seconds, then Boost

[ ]  Throttle -- Back to ¼ inch open

[ ]  Propeller area -- Clear

[ ]  Ignition switch -- Start (1000 RPM)

[ ]  Oil pressure -- Check

[ ]  Boost pump -- Off

[ ]  Fuel quantity -- Verify

Pre-taxi

[ ]  Avionics master -- On

[ ]  Ammeter -- Check Alt #1 & Alt #2

[ ]  MFS, Autopilot, GNS 430 -- Check/set

[ ]  ATIS -- Copy

[ ]  Altimeter -- Set

[ ]  Attitude indicator -- Set

[ ]  Parking brake -- Off

[ ]  Flaps -- Check and up

[ ]  Ground Clearance -- Okay

Run-up / Pre-take-off

[ ]  Flight instruments -- Monitor on taxi

[ ]  Flight controls -- Check, full range

[ ]  Elevator/Aileron Trim -- Set for take-off

[ ]  Flaps -- Set to 50%

[ ]  Fuel selector -- Fullest tank

[ ]  Boost pump -- On

[ ]  Throttle -- Increase to detent (2000 RPM)

[ ]  Bring throttle back -- 1700 RPM

[ ]  Load elec. system (w/ pitot & all lights)

[ ]  Check Amps/Volts – both alternators

[ ]  Mags -- 150 rpm max drop; 75 rpm diff.

[ ]  Throttle -- Return to 1000 rpm

[ ]  Transponder -- Set correct code

[ ]  GPS -- Activate flight plan or direct to

[ ]  Autopilot -- Set pre-select as required

[ ]  Pitot heat, lights -- As needed

[ ]  Tower -- Clearance for runway

Climb out and level

NOTE: Note -- Time

[ ]  Compass – Verify runway heading

[ ]  Rotate -- 70 Knots .... 70 KNOTS

[ ]  Flaps up -- 80 Knots .... 80 KNOTS

[ ]  Boost pump -- Off

Climb out and level

[ ]  Cruise alt -- Lean mixture

[ ]  Fuel -- Switch every 30 min; Boost on

Descent & landing

[ ]  ATIS -- Check

[ ]  Mixture -- Rich

[ ]  Auto pilot -- Off

[ ]  Landing lights -- On

[ ]  Flaps & speed -- 50% < 119 knots

[ ]  -100% < 104 knots

After landing

[ ]  Flaps -- Up

[ ]  Boost pump -- Off

[ ]  Flight plan -- Closed

Shutdown

[ ]  Avionics master -- Off

[ ]  Mixture -- Cutoff

[ ]  Magnetos -- Off

[ ]  Bat-Alt Master switches -- Off

[ ]  Lights, de-ice, pitot heat -- Off

[ ]  CAPS pin -- Insert

[ ]  Pitot cover, cowl plugs -- Install

[ ]  Canvass cover / Sun shields -- As needed

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Cirrus/SR22/sr22_checklist_steve_kahn.txt

Rights: licensed

Checklist hash: 1852f23da6eeddd99e9b1f6df2bfb42928066a689d79e2321be4e9e367fe9bcd

Verify against the aircraft's own approved documentation before use.