Commander 1000 Jetprop — James_Kaim_Checklist_1000_Commander

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

BEFORE EXTERIOR

[ ]  GEAR CONTROL LEVER .... DOWN AND LATCHED

[ ]  TORQUE INDICATORS .... 100%

[ ]  DAVTRON CL0CK .... ZEROED

[ ]  ENGINE CONTROL SWITCHES .... ENG OFF

[ ]  OVERHEAD SWITCHES .... OFF OR NORMAL

IF GPU START IS TO BE MADE

[ ]  GROUND START SWITCH .... OFF/SER

[ ]  GPU .... CONNECTED AND ON

[ ]  GROUND START SWITCH .... GPU/PAR

[ ]  RIGHT VOLT/AMMETER .... PUSH FOR VOLTS

NOTE: (26-29 VOLTS LO VOLT LIGHT OUT)

[ ]  BATTERY SWITCH .... ON

[ ]  NAV LIGHTS (NIGHT FLIGHT) .... CHECK THEN OFF

[ ]  TORQUE INDICATORS .... ZERO

[ ]  GEAR POSITION LIGHTS .... 3 GREEN

[ ]  TRIM TABS (3) .... SET TO ZERO

[ ]  HYDRAULIC PRESSURES .... 470 PSI MINIMUM

[ ]  FLAPS .... DOWN

NOTE: FLAPS MUST BE UP FOR ACCURATE HYDRAULIC FLUID LEVEL CHECK AND SYSTEM PRESSURE SHOULD BE ZERO.

[ ]  OIL DOORS .... OPEN

[ ]  PARKING BRAKE .... AS REQUIRED

[ ]  FUEL QUANTITY .... CHECK

[ ]  BATTERY SWITCH .... OFF

[ ]  WINDSHIELD AND WINDOWS .... CHECK

[ ]  EMERGENCY EXIT .... SECURE

[ ]  CABIN COMPARTMENT .... SECURE

[ ]  BEFORE EXTERIOR CHECKLIST .... COMPLETE

EXTERIOR

CLOCKWISE

[ ]  DOOR BAYONET AND SEAL .... CHECK

[ ]  OAT PROBE AND UPPER WING .... CHECK

[ ]  LEFT WINDSHIELD/WIPER .... CHECK

[ ]  NOSE ACCESS .... SECURE

[ ]  LEFT PITOT HEAD .... CHECK

[ ]  LEFT RAM AIR INLET .... CLEAR

[ ]  LEFT LANDING LIGHT .... CHECK

[ ]  NOSE GEAR/DOORS .... CHECK

[ ]  RADOME .... CHECK

[ ]  RIGHT LANDING LIGHT .... CHECK

[ ]  RIGHT RAM AIR INLET .... CLEAR

[ ]  RIGHT PITOT HEAD .... CHECK

[ ]  RIGHT WINDSHIELD/WIPER .... CHECK

[ ]  EMERGENCY EXIT .... SECURE

[ ]  RIGHT INBOARD DEICE BOOT .... CHECK

[ ]  RIGHT WING FUEL SUMP .... DRAIN

[ ]  RIGHT ENGINE AND PROP .... CHECK

[ ]  RIGHT WING .... CHECK

[ ]  RIGHT ENGINE TAILPIPE .... CHECK

[ ]  RIGHT WHEEL WELL .... CHECK

[ ]  RIGHT STRUT, WHEEL AND BRAKE .... CHECK

[ ]  INBOARD FLAP AND HINGE .... CHECK

[ ]  AMBIENT AIR INLET .... CLEAR

[ ]  RIGHT SIDE EJECTOR TUBE .... CLEAR

[ ]  FREON AIR EVAP DRAIN .... CLEAR

[ ]  BATTERY VENT DRAIN .... CHECK

[ ]  ECU INTAKE PORT .... CLEAR

[ ]  RAM AIR INLET SCOOP .... CLEAR

[ ]  OXYGEN BOTTLE .... CHECK AND ON

[ ]  RIGHT STATIC PORTS .... CLEAR

[ ]  ANTENNAS .... CHECK

[ ]  DORSAL FIN .... CHECK

[ ]  VERTICAL AND HORIZONTAL DEICE BOOTS .... CHECK

[ ]  STATIC WICKS .... SECURE

[ ]  ELEVATORS AND TRIM TABS .... CHECK

[ ]  EXTERNAL RUDDER GUST LOCK .... REMOVED

[ ]  RUDDER AND TRIM TAB .... CHECK

[ ]  RUDDER HORN/HEATER .... CLEARANCE

[ ]  TAIL CONE .... CHECK

[ ]  FREON AIR INLET VENT .... CLEAR

[ ]  LEFT SIDE STATIC PORTS .... CLEAR

[ ]  ECU EXHAUST DUCT .... CLEAR

[ ]  FREON AIR EXHAUST .... CLEAR

[ ]  BATTERY VENT INLET .... CLEAR

[ ]  BAGGAGE COMPARTMENT .... CHECK

[ ]  FREON AIR EVAP DRAIN .... CLEAR

[ ]  LEFT INBOARD FLAP HINGE .... CHECK

[ ]  LEFT STRUT, WHEEL, BRAKE AND STRUT SWITCH .... CHECK

[ ]  LEFT WHEEL WELL .... CHECK

[ ]  LEFT TAILPIPE .... CHECK

[ ]  HYDRAULIC QUANTITY .... CHECK

[ ]  LEFT WING .... CHECK

[ ]  LEFT NACELLE, ENGINE AND PROP .... CHECK

[ ]  NITROGEN PRESSURE GAGE .... 425 TO 525 PSI

[ ]  LEFT WING FUEL SUMP .... DRAIN

[ ]  LEFT INBOARD DEICE BOOT .... CHECK

[ ]  EXTERIOR CHECKLIST .... COMPLETE

BEFORE STARTING ENGINES

[ ]  GEAR CONTROL LEVER .... DOWN AND LATCHED

[ ]  ENGINE CONTROL SWITCHES .... ENG OFF

[ ]  FUEL/HYD SWITCHES .... AS REQUIRED

[ ]  GND START SWITCH .... AS REQUIRED

IF GPU START IS TO BE MADE

[ ]  GND START SWITCH .... OFF/SER

[ ]  GPU .... CONNECTED AND ON

[ ]  GND START SWITCH .... GPU/PAR

[ ]  RIGHT VOLT/AMMETER .... PUSH FOR VOLTS

NOTE: (26-29 VOLTS LO VOLT LIGHT OUT)

[ ]  BATTERY SWITCH .... ON

[ ]  OVERHEAD SWITCHES .... SET FOR ENGINE START

[ ]  CABIN BUS .... AS REQUIRED

[ ]  TORQUE LIMITER SWITCHES .... OFF

[ ]  NTS SWITCHES .... TEST

[ ]  PANEL LIGHTS .... AS REQUIRED

[ ]  ANTI-COLLISION LIGHT .... ON

[ ]  BELTS/NO SMOKING SIGN .... ON

[ ]  AUTO START OVERRIDE .... AS REQUIRED

[ ]  CREW MIKE SELECTORS .... AS REQUIRED

[ ]  CREW OXYGEN MASKS .... FITTED, 100%, CHECK FLOW

[ ]  CIRCUIT BREAKER PANELS .... CHECK

[ ]  AURAL WARNING SYSTEMS .... CHECK

[ ]  ENVIRONMENTAL SYSTEM CONTROLS .... SET

[ ]  ENVIR MASTER SWITCH .... NORMAL

[ ]  CABIN DEPRESS SWITCH .... NORMAL

[ ]  TEMP CONTROL KNOB .... AS DESIRED

[ ]  AUTO/OVRD SWITCH .... OFF

[ ]  MAX FLO SWITCH .... OFF

[ ]  BLEED AIR VALVES .... BOTH CLOSED

[ ]  PRESSURIZATION SYSTEM CONTROLS .... SET

[ ]  CABIN RATE CONTROL .... 12 O'CLOCK

[ ]  CABIN ALTITUDE SELECTOR .... 1,000' AGL OR HIGHER

[ ]  CABIN DIFFERENTIAL .... ZERO

[ ]  CABIN ALTITUDE .... FIELD ELEVATION

[ ]  CABIN RATE OF CHANGE .... ZERO

[ ]  LO VOLT LIGHT .... ON WITHOUT GPU

[ ]  CREW AUDIO AND COMPASS PANELS .... SET AND SLAVE

[ ]  ANNUNCIATOR PANEL .... CHECK AND TEST

[ ]  FUEL TOTALIZER .... AS REQUIRED

[ ]  POWER LEVERS .... GND IDLE

[ ]  CONDITION LEVERS .... LO RPM

[ ]  OIL DOORS .... OPEN

[ ]  FLAPS .... UP, INDICATING UP

[ ]  HYD PRESS .... 470 PSI MINIMUM

[ ]  PROP SYNC .... OFF

[ ]  PARKING BRAKE .... AS REQUIRED

[ ]  FUEL QUANTITY .... CHECKED AND BALANCED

[ ]  ELT .... ARMED

[ ]  PASSENGER OXYGEN .... AS REQUIRED

[ ]  OXYGEN PRESSURE .... CHECK

[ ]  BATTERY SWITCH .... IF PAX NOT READY, OFF

[ ]  TAKEOFF DATA .... DETERMINED

[ ]  BEFORE STARTING ENGINES CHECKLIST .... COMPLETE

STARTING ENGINES

[ ]  EXTERNAL GUST LOCK .... STOWED

[ ]  PAX BRIEFING .... AS REQUIRED

[ ]  CABIN DOOR .... CLOSED, LOCKED

[ ]  INTERNAL CONTROL LOCK .... STOWED

[ ]  SEAT BELTS AND HARNESSES .... FASTENED

[ ]  BATTERY SWITCH .... ON

[ ]  DOOR SWITCH .... LOCK, DOOR LIGHT OUT

BATTERY START

[ ]  START MODE .... SELECTED

[ ]  PROPELLER .... CLEAR AND UNFEATHERED

[ ]  EGT .... BELOW 200°C

[ ]  ENGINE CONTROL SWITCH

[ ]  FUEL PUMP ON .... 15 PSI MINIMUM

[ ]  AIR START .... SET LEVER FOR NTS TEST

[ ]  GND START .... NTS LIGHT OUT WITHIN 1 SEC

[ ]  CONDITION LEVER .... READY FOR ABORT

[ ]  EGT AND RPM .... MONITOR

[ ]  OIL PRESSURE .... 40 PSI MIN AT 65% RPM

[ ]  FUEL PRESSURE .... GREEN ARC

[ ]  HYDRAULIC PRESSURE .... 900 TO 1075 PSI

[ ]  ANNUNCIATOR LIGHT .... OFF

[ ]  NTS SWITCH .... OFF

[ ]  TORQUE LIMITER SWITCH .... ON

[ ]  OPERATING ENGINE GENERATOR .... ON

[ ]  ANNUNCIATOR LIGHT .... OFF

[ ]  LO VOLT LIGHT .... OFF

[ ]  AMMETER .... 200 AMPS OR LESS

[ ]  GND START SWITCH .... GPU/PAR

NOTE: REPEAT ITEMS MARKED (**) FOR SECOND ENGINE START

[ ]  OTHER GENERATOR .... ON

[ ]  ANUNCIATOR LIGHT .... OFF

[ ]  LOAD SHARING AND VOLTAGE .... CHECK

GPU START

[ ]  PROPELLER .... CLEAR AND UNFEATHERED

[ ]  EGT .... BELOW 200°C

[ ]  ENGINE CONTROL SWITCH

[ ]  FUEL PUMP ON .... 15 PSI MINIMUM

[ ]  AIR START .... SET LEVER FOR NTS TEST

[ ]  GND START .... LIGHT OUT WITHIN 1 SEC.

[ ]  CONDITION LEVER .... READY FOR ABORT

[ ]  EGT AND RPM .... MONITOR

[ ]  OIL PRESSURE .... 40 PSI MIN AT 65% RPM

[ ]  FUEL PRESSURE .... GREEN ARC

[ ]  HYDRAULIC PRESSURE .... 900 TO 1075

[ ]  ANNUNCIATOR LIGHT .... OFF

[ ]  NTS SWITCH .... NORMAL

[ ]  TORQUE LIMIT SWITCH .... ON

NOTE: REPEAT ITEMS MARKED (**) FOR SECOND ENGINE START

[ ]  GND START SWITCH .... OFF/SER

[ ]  GPU .... DISCONNECTED AND REMOVED

[ ]  GND START SWITCH .... GPU/PAR

[ ]  BOTH GENERATORS .... ON

[ ]  ANNUNCIATOR LIGHTS .... OFF

[ ]  LO VOLT LIGHT .... OUT

[ ]  LOAD SHARING AND VOLTAGE .... CHECK

[ ]  STARTING ENGINES CHECKLIST .... COMPLETE

AFTER START/BEFORE TAXI

[ ]  CABIN BUS .... VERIFY ON

[ ]  RADIO SWITCHES .... ON

[ ]  BUS OFF LIGHTS .... OUT

[ ]  INVERTERS .... CHECKED AND ON

[ ]  AVIONICS SWITCHES .... ON

[ ]  BLEED AIR VALVES .... CHECK AND BOTH OPEN

[ ]  MAX FLO .... AS REQUIRED

[ ]  AUTO/OVRD SWITCH .... AS REQUIRED

[ ]  OIL TEMPERATURE .... GREEN ARC

[ ]  SRL SYSTEM .... CHECK

[ ]  SRL/ P/P SWITCHES .... NORMAL

[ ]  CONDITION LEVERS .... HIGH

[ ]  POWER LEVERS .... 100%

NOTE: OBSERVE RPM JUMP AT 80%

[ ]  EGT .... NOTE VALUE

[ ]  SRL/ P/P SWITCHES .... SRL OFF

[ ]  EGT .... VALUE WITHIN 10 C

[ ]  SRL/ P/P SWITCHES .... NORMAL

[ ]  NTS CHECK VALVE .... CHECK

[ ]  CONDITION LEVERS .... HI RPM

[ ]  POWER LEVERS .... FLT IDLE

[ ]  NTS SWITCH .... TEST

[ ]  NT LIGHTS .... ON

[ ]  NTS TEST LIGHTS .... REMAIN OFF

[ ]  NTS SWITCHES .... OFF

NOTE: TORQUE LIMITER TEST CAN BE PERFORMED AT THIS POINT

[ ]  OVERSPEED GOVERNOR .... CHECK

[ ]  POWER LEVERS (ONE AT A TIME) .... ADVANCE

[ ]  RPM .... 103 TO 105%

CAUTION: DO NOT EXCEED 106% RPM

[ ]  POWER LEVERS .... GND IDLE

[ ]  CONDITION LEVERS .... LO RPM

[ ]  PROPELLERS (ONE AT A TIME) .... UNLOCK

[ ]  POWER LEVERS .... SLIGHTLY INTO REVERSE

[ ]  TORQUE AND FUEL FLOW .... OBSERVE SLIGHT RISE

[ ]  POWER LEVERS .... GND IDLE

[ ]  GYRO PRESSURE .... CHECK

[ ]  HEADING INDICATORS .... CHECKED AND NO FLAGS

[ ]  ATIS AND CLEARANCE .... AS REQUIRED

[ ]  ALTIMETERS AND CLOCKS .... SET

[ ]  AFTER START/BEFORE TAXI CHECKLIST .... COMPLETE

TAXI/BEFORE TAKEOFF

[ ]  TAXI LIGHTS .... AS REQUIRED

[ ]  BRAKES .... CHECK

[ ]  FLIGHT INSTRUMENTS .... CHECKED

[ ]  YAW DAMPER .... CHECKED AND OFF

[ ]  TAXI CHECKLIST .... COMPLETE

BEFORE TAKEOFF

[ ]  FLIGHT CONTROLS .... FREE AND FULL TRAVEL

[ ]  ELECTRIC TRIM AND A/P .... CHECKED AND OFF

[ ]  TRIM TABS (3) .... SET FOR TAKEOFF

[ ]  CIRCUIT BREAKER PANELS .... CHECK

[ ]  MAX FLO SWITCH .... OFF

[ ]  FLAPS .... UP, INDICATING UP

[ ]  FUEL QUANTITY .... RECHECK

[ ]  FUEL INTERCONNECT VALVE .... CHECKED AND CLOSED

[ ]  LOW FUEL PUMPS .... CHECKED AND OFF

[ ]  ENGINE INSTRUMENTS .... CHECK

[ ]  RADIOS AND FLIGHT INSTRUMENTS .... CHECK AND SET

[ ]  ICE PROTECTION .... CHECKED AND OFF

[ ]  CREW BRIEFING .... ACCOMPLISH

[ ]  BEFORE TAKEOFF CHECKLIST .... COMPLETE

LINEUP/TAKEOFF

[ ]  WEATHER RADAR .... AS REQUIRED

[ ]  TRANSPONDER .... ON

[ ]  LANDING AND TAXI LIGHTS .... AS REQUIRED

[ ]  STROBE LIGHTS .... ON

[ ]  UNDERSPEED GOVERNOR .... CHECK

[ ]  CONDITION LEVERS .... HI RPM

[ ]  POWER LEVER .... GND IDLE

[ ]  ENGINE RPM .... 96.5 TO 97.5%

[ ]  POWER LEVERS .... REVERSE SLIGHTLY

[ ]  ENGINE RPM .... 96.5 TO 97.5%

[ ]  TORQUE/TEMP LIMITER TEST .... AS REQUIRED

[ ]  CONDITION LEVERS HIGH .... HI RPM

[ ]  POWER LEVERS .... OBTAIN 100% RPM

[ ]  TORQUE/TEMP LIMITER SWITCHES .... TEST

[ ]  FUEL FLOW AND RPM .... OBSERVE DROP

CAUTION: DO NOT ALLOW RPM TO GO BELOW 65%

[ ]  IGNITION OVERRIDE .... AS REQUIRED

[ ]  PITOT HEAT .... ON

[ ]  ICE PROTECTION .... AS REQUIRED

[ ]  ANNUNCIATOR PANEL .... CONSIDERED

[ ]  LINEUP CHECKLIST .... COMPLETE

TAKEOFF

[ ]  POWER LEVERS .... ADVANCE

[ ]  BETA LIGHTS .... OFF

[ ]  ENGINE RPM .... VERIFY 100%

[ ]  ENGINE INTRUMENTS .... MONITOR

[ ]  ROTATE .... 15° AT 100 KIAS

AFTER TAKEOFF/CLIMB

[ ]  GEAR .... UP AT 200' AGL

[ ]  OIL DOORS .... AS REQUIRED

[ ]  CLIMB POWER .... SET 650 EGT 90% TQ, AND 97 TO 99% RPM

[ ]  PROP SYNC .... ON

[ ]  CLIMB SPEED .... 135 KIAS AT SEA LEVEL DECREASING 1 KT/1,000'

[ ]  IGNITION OVERRIDE .... OFF

[ ]  BELTS/NO SMOKING SIGN .... AS REQUIRED

[ ]  PRESSURIZATION .... CHECK AND ADJUST RATE

[ ]  YAW DAMPER .... AS REQUIRED

[ ]  AFTER TAKEOFF CHECKLIST .... COMPLETE

PASSING 10,000 FEET

[ ]  PRESSURIZATION .... CHECK DIFF AND ALTITUDE

[ ]  LANDING AND TAXI LIGHTS .... OFF

PASSING FL 180

[ ]  ALTIMETERS .... 29.92

[ ]  WINDSHIELD HEAT .... ON

[ ]  OXYGEN SYSTEM .... CHECK

[ ]  PASSENGER OXYGEN .... AUTO

[ ]  OXYGEN PRESSURE .... CHECK

[ ]  CREW MASKS .... CHECK FLOW

CRUISE

[ ]  CRUISE POWER .... SET

[ ]  ANNUNCIATOR PANEL .... TEST PERIODICALLY

[ ]  ICE PROTECTION .... AS REQUIRED

[ ]  OXYGEN SYSTEM .... CHECK

[ ]  PRESSURIZATION .... CHECK

[ ]  LOW FUEL PUMPS .... CHECK AT 600 LBS

DESCENT

[ ]  BELTS SIGN .... ON

[ ]  ICE PROTECTION .... AS REQUIRED

[ ]  WINDSHIELD HEAT .... AS REQUIRED

[ ]  PRESSURIZATION CONTROLS .... SET FOR DESTINATION

[ ]  ATIS INFORMATION .... RECEIVED AND RECORDED

[ ]  LANDING DATA .... DETERMINED

[ ]  ALTIMETER .... SET PASSING FL 180

APPROACH

[ ]  ICE PROTECTION .... AS REQUIRED

[ ]  LANDING LIGHTS .... AS REQUIRED

[ ]  HYDRAULIC PRESURE .... CHECK

[ ]  PRESSURIZATION .... CHECK

[ ]  CREW BRIEFING .... ACCOMPLISH

BEFORE LANDING

[ ]  WINDSHIELD HEAT .... VERIFY OFF

[ ]  IGNITION OVERRIDE .... AS REQUIRED

[ ]  BELTS/NO SMOKING SIGN .... ON

[ ]  RADIO ALTIMETER .... CHECK AND SET

[ ]  CONDITION LEVERS .... HI RPM

[ ]  GEAR .... DOWN AND LOCKED

[ ]  FLAPS .... AS REQUIRED

[ ]  OIL DOORS .... OPEN

[ ]  PROP SYNC (TYPE I) .... OFF

[ ]  YAW DAMPER .... OFF

[ ]  MAX FLO SWITCH .... OFF

AFTER LANDING

[ ]  POWER LEVERS .... GND IDLE

[ ]  CONDITION LEVERS .... LO RPM

[ ]  ICE PROTECTION .... OFF

[ ]  IGNITION OVERRIDE .... OFF

[ ]  STROBE LIGHTS .... OFF

[ ]  LANDING AND TAXI LIGHTS .... AS REQUIRED

[ ]  TRANSPONDER AND WEATHER RADAR .... STANDBY

[ ]  FLAPS .... UP

[ ]  OIL DOORS .... VERIFY OPEN

[ ]  PROP SYNC .... OFF

SHUTDOWN

[ ]  PARKING BRAKE .... AS REQUIRED

[ ]  AUTO/OVRD SWITCH .... OFF

[ ]  AUXILLIARY EQUIPMENT .... OFF

[ ]  AVIONICS SWITCHES .... OFF

[ ]  INVERTER AND RADIO SWITCHES .... OFF

[ ]  GENERATORS .... OFF

[ ]  ENGINES .... SHUTDOWN

[ ]  POWER LEVERS .... GND IDLE

[ ]  ENGINE CONTROL SWITCHES .... ENG OFF

[ ]  POWER LEVERS .... FULL REVERSE AT 50% RPM

[ ]  DOOR SWITCH .... UNLOCK AFTER PROP STOPS

SECURING AIRPLANE

[ ]  OXYGEN SYSTEM .... OFF

[ ]  CREW MASKS .... DISARM AND DISCONNECT

[ ]  OIL DOORS .... CLOSED

[ ]  BATTERY SWITCH .... OFF

[ ]  GND START SWITCH .... OFF/SER

[ ]  PARKING BRAKE .... AS REQUIRED

[ ]  AIRPLANE .... SECURE

[ ]  RUDDER GUST LOCK .... INSTALL

[ ]  TIE DOWN AND CHOCKS .... AS REQUIRED

[ ]  EXTERNAL COVERS .... AS REQUIRED

[ ]  POST FLIGHT INSPECTION .... COMPLETE

[ ]  COURTESY LIGHT .... OBSERVE OFF

GO AROUND

[ ]  F/D GO AROUND MODE .... SELECTED

[ ]  POWER .... MAX ALLOWABLE

[ ]  AIRSPEED .... 100 KIAS

[ ]  FLAPS .... UP-135 KIAS AS FLAPS RETRACT

[ ]  GEAR .... UP WHEN POSITIVE CLIMB

[ ]  OIL DOORS .... AS REQUIRED

[ ]  IGNITION OVERRIDE .... OFF

TAXI BACK

[ ]  FLAPS .... UP

[ ]  TRIM .... 3 TABS SET FOR TAKEOFF

[ ]  RADAR .... STANDBY

[ ]  TRANSPONDER .... STANDBY

[ ]  ICE PROTECTION .... CHECK

QUICK TURN

[ ]  PROPS .... ON LOCKS

[ ]  GND START SWITCH .... PAR

[ ]  EGT .... BELOW 200 C

[ ]  ENGINE .... START

[ ]  GENERATOR .... ON

[ ]  TRIM .... 3 TABS SET FOR TAKEOFF

[ ]  FLAPS .... UP

AIRSPEEDS FOR SAFE OPERATION (KIAS)

CONDITIONS:

NOTE: TAKEOFF WEIGHT-----11,200 LBS

NOTE: LANDING WEIGHT-----10,550 LBS

NOTE: STANDARD DAY-----SEA LEVEL

NOTE: TE CLIMB TO 50 FEET(FLAPS UP/GEAR DOWN)-----102

NOTE: VX (CLEAN)-----95

NOTE: VY (CLEAN)-----135

NOTE: SE CLIMB TO 50 FEET (FLAPS UP/GEAR DOWN)-----98

NOTE: VXSE (CLEAN)-----102

NOTE: VYSE (CLEAN)-----120

NOTE: VSSE-----105

NOTE: VMO-----250

NOTE: VA (10325)-----139

NOTE: THRESHOLD SPEED (FULL FLAPS)-----100

NOTE: TRANSITION TO BALKED LANDING-----100

NOTE: MAX DEMONSTRATED CROSSWIND (TAKEOFF)-----30

NOTE: MAX DEMONSTRATED CROSSWIND (LANDING)-----27

NOTE: VMCA-----93

NOTE: MODERATE TURBULANCE PENETRATION-----180

NOTE: SEVERE/EXTREME TURBULANCE PENETRATION-----VA

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Commander/1000 Jetprop/James_Kaim_Checklist_1000_Commander.txt

Rights: licensed

Checklist hash: ad210bdf5c1f349bd0c98378478838f78a526093f2478ff6969a716bfdcf9a44

Verify against the aircraft's own approved documentation before use.