Diamond DA40 180 — Normal Operating Procedures

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: normal_only | 1 unresolved safety defect(s)

safety_defect: Contains no airspeeds, power settings, fuel quantities, or other numeric limitations — all such values must come from the specific aircraft POH/AFM.

content_gap: Equipment and avionics vary across DA40 production. Verify all items against the specific aircraft POH, supplements, and installed equipment before use.

Preflight Walk-Around

[ ]  Aircraft documents (ARROW) — airworthiness certificate, registration, radio station license, operating handbook/AFM, weight-and-balance data .... ON BOARD AND CURRENT

[ ]  Tie-downs and wheel chocks — all aircraft restraints .... REMOVED

[ ]  Canopy — condition, seals, latch mechanism, and hinge hardware .... UNDAMAGED, SEALS INTACT, LATCH FUNCTIONAL

[ ]  Pitot tube — cover removed, inlet clear of debris and obstruction .... COVER REMOVED, CLEAR

[ ]  Static ports — both sides of fuselage .... CLEAR AND UNOBSTRUCTED

[ ]  Fuel — left tank visual quantity check; verify cap seated and locked .... QUANTITY NOTED, CAP SECURE

[ ]  Fuel — left tank sump drain; sample for water, sediment, and correct color .... CLEAR, CORRECT COLOR, NO CONTAMINATION

[ ]  Left wing leading edge, skin, and composite surfaces .... NO DELAMINATION, CRACKS, OR IMPACT DAMAGE

[ ]  Left aileron — attachment at hinge points, surface condition, and full range of motion .... SECURE, UNDAMAGED, FREE MOVEMENT

[ ]  Left flap — hinge attachment and surface condition .... SECURE AND UNDAMAGED

[ ]  Left wingtip and navigation light .... UNDAMAGED, LENS INTACT

[ ]  Left main landing gear — strut extension, tire condition and inflation, brake disc and pad visible wear .... STRUT NORMAL, TIRE SERVICEABLE, BRAKES ADEQUATE

[ ]  Nose section — composite skin for impact damage or cracks .... UNDAMAGED

[ ]  Engine cowling — all latches and fasteners .... SECURE

[ ]  Engine oil level — dipstick; within acceptable operating range per POH .... WITHIN RANGE; CAP REINSTALLED AND SECURE

[ ]  Engine compartment — fuel and oil lines for leaks, wiring for chafing, no foreign objects .... CLEAR, NO LEAKS, SECURE

[ ]  Induction air inlet — unobstructed; IO-360 is fuel-injected, no carburetor heat system .... INLET CLEAR

[ ]  Exhaust stacks and visible exhaust system .... NO CRACKS, CORROSION, OR LOOSE CLAMPS

[ ]  Propeller blades — each blade for nicks, gouges, cracks, and delamination; spinner condition and security .... BLADES SERVICEABLE, SPINNER SECURE

[ ]  Nose gear strut — extension, shimmy damper, and tow bar removal .... STRUT EXTENDED NORMALLY, TOW BAR REMOVED

[ ]  Nose gear tire — condition, inflation, and tread .... SERVICEABLE

[ ]  T-tail — horizontal stabilizer, elevator, and elevator trim tab; inspect composite surfaces and hinge hardware .... UNDAMAGED, HINGES SECURE, NO DELAMINATION

[ ]  Vertical stabilizer and rudder — composite skin condition, hinge security, and free movement .... UNDAMAGED, RUDDER FREE

[ ]  Right main landing gear — strut, tire, and brake condition .... STRUT NORMAL, TIRE SERVICEABLE, BRAKES ADEQUATE

[ ]  Right wing and fuel tank — leading edge, skin for delamination, fuel cap security .... UNDAMAGED, FUEL CAP SECURE

[ ]  Fuel — right tank sump drain; sample for water and contamination .... CLEAR, CORRECT COLOR, NO CONTAMINATION

[ ]  Right aileron and right flap — hinge attachment, surface condition .... SECURE, FREE, UNDAMAGED

[ ]  Right wingtip and navigation light .... UNDAMAGED, LENS INTACT

[ ]  Antennas — all externally visible antennas .... SECURE AND UNDAMAGED

[ ]  Overall fuel quantity — both tanks combined; adequate for planned flight with required reserves .... VERIFIED ADEQUATE PER POH

Cockpit Preparation / Before Start

[ ]  Canopy — open for entry; inspect seal and latch before closing .... INSPECTED

[ ]  Seats — fore-aft position and lock; ensure seat is secure before flight .... ADJUSTED AND LOCKED

[ ]  Seat belts and shoulder harnesses — pilot and all occupants .... FASTENED AND SNUG

[ ]  Canopy — close and verify latched; check both latch points .... CLOSED AND LATCHED

[ ]  Passenger briefing — canopy latch operation, seatbelts, sterile cockpit, no smoking, emergency procedures .... COMPLETED

[ ]  Sidestick and rudder pedals — full range of motion; correct response direction .... FULL AND FREE, CORRECT SENSE

[ ]  Parking brake .... SET

[ ]  Fuel selector — confirm position .... BOTH (or selected tank per POH)

[ ]  All electrical switches — lights, avionics master, pitot heat .... OFF

[ ]  Circuit breakers .... ALL IN

[ ]  Flaps — set to takeoff position per POH .... SET PER POH

[ ]  Elevator trim — set to takeoff position .... SET TO TAKEOFF

[ ]  Mixture .... RICH (or per altitude per POH)

[ ]  Propeller control — confirm lever position .... FULL FORWARD (HIGH RPM)

[ ]  Throttle .... CLOSED / IDLE

Engine Start

[ ]  Propeller arc — visually confirm area clear of personnel and objects; announce intent .... CLEAR; 'CLEAR PROP' ANNOUNCED

[ ]  Battery master switch .... ON

[ ]  Alternator switch .... ON

[ ]  Beacon / anti-collision light .... ON

[ ]  Avionics master .... OFF (protect during cranking)

[ ]  Fuel boost pump — prime per POH procedure for cold or hot start condition .... PRIMED PER POH

[ ]  Throttle — position for start per POH .... SET PER POH

[ ]  Ignition switch — START; release when engine fires .... ENGAGED, RELEASED ON START

[ ]  Throttle — reduce to idle after start and allow engine to stabilize .... IDLE, STABILIZING

[ ]  Oil pressure — verify positive rise within first 30 seconds; shut down if pressure does not rise .... RISING NORMALLY

[ ]  Avionics master — on after engine is running and stable .... ON

[ ]  Engine instruments — oil temperature rising, all gauges scanning into normal range .... NORMAL

Before Taxi / After Start

[ ]  G1000 / avionics — complete self-tests, verify database currency, GPS acquiring .... SELF-TESTS PASSED, DATABASES CURRENT

[ ]  Altimeter — set current ATIS/ASOS altimeter setting; verify field elevation indication within tolerance .... SET, ELEVATION CONFIRMED

[ ]  Heading indicator / HSI — verify alignment with magnetic compass .... ALIGNED

[ ]  ATIS or AWOS — obtain current weather, active runway, and altimeter .... OBTAINED AND NOTED

[ ]  Transponder — set assigned squawk code; select ALT mode .... CODE SET, ALT

[ ]  Taxi clearance or CTAF self-announce .... RECEIVED / ANNOUNCED

[ ]  Taxi and landing light — as required for taxi operations .... ON AS APPROPRIATE

[ ]  Parking brake .... RELEASE WHEN READY TO TAXI

Run-Up / Before Takeoff

[ ]  Parking brake .... SET

[ ]  Engine warm-up — verify oil temperature and pressure within normal range before run-up .... TEMPS AND PRESSURES NORMAL

[ ]  Throttle — advance to run-up power setting per POH .... RUN-UP POWER SET

[ ]  Magnetos — test LEFT then RIGHT; note RPM drop on each; verify drop within limits and differential within limits per POH .... DROP WITHIN LIMITS, DIFFERENTIAL WITHIN LIMITS

[ ]  Propeller cycle — advance prop control to full forward; smoothly reduce RPM; note governor responding and RPM drop; advance back to high RPM. Repeat as directed by POH to circulate warm oil through governor. .... GOVERNOR RESPONSE CONFIRMED

[ ]  Engine instruments — oil temperature, oil pressure, fuel flow, EGT/CHT within normal operating range .... ALL NORMAL

[ ]  Throttle — reduce to idle; verify smooth idle; advance back to run-up .... SMOOTH IDLE CONFIRMED

[ ]  Throttle — return to idle for final checks .... IDLE

[ ]  Fuel selector .... BOTH (or as directed by POH)

[ ]  Mixture .... RICH (or as required for field elevation per POH)

[ ]  Propeller control .... FULL FORWARD

[ ]  Flaps — confirm takeoff setting .... SET PER POH

[ ]  Trim — elevator at takeoff position .... SET

[ ]  Flight controls — final check of sidestick full travel and correct response .... FREE AND CORRECT

[ ]  Canopy — fully latched at both latch points; no canopy unlock indication .... LATCHED AND VERIFIED

[ ]  Seat belts and shoulder harnesses .... FASTENED ALL OCCUPANTS

[ ]  Approach and departure briefing — abort criteria, emergency return, traffic pattern .... BRIEFED

Takeoff

[ ]  Runway — confirm clear of traffic; obtain clearance or announce on CTAF .... CLEAR, CLEARED OR ANNOUNCED

[ ]  Strobes / landing light .... ON

[ ]  Transponder .... ALT MODE CONFIRMED

[ ]  Throttle — smoothly advance to full power .... FULL POWER

[ ]  Engine instruments — scan at full power; abort if any indication abnormal .... NORMAL

[ ]  Directional control on roll — use rudder pedals; nose-wheel steering lightly; sidestick neutral .... TRACKING CENTERLINE

[ ]  Rotation — at Vr per POH; smooth back pressure on sidestick .... ROTATE AT Vr PER POH

[ ]  Initial climb attitude — establish per POH; verify positive rate of climb on altimeter and VSI .... POSITIVE CLIMB

Climb

[ ]  Airspeed — establish climb speed per POH (Vy for best rate, or Vx for obstacle clearance) .... CLIMB SPEED PER POH

[ ]  Flaps — retract per POH when clear of obstacles and at safe speed .... RETRACTED PER POH

[ ]  Propeller — reduce to climb power setting per POH if appropriate .... PER POH

[ ]  Throttle — set climb power per POH .... CLIMB POWER SET

[ ]  Mixture — lean as required for altitude per POH .... PER POH

[ ]  Engine instruments — CHT, oil temperature, oil pressure, fuel flow; verify all within limits .... WITHIN LIMITS

[ ]  Fuel selector — verify BOTH or per POH fuel management procedure .... PER POH

[ ]  Altimeter — confirm set to current altimeter setting; change to 29.92 when assigned by ATC or above transition altitude .... SET CORRECTLY

Cruise

[ ]  Cruise altitude reached — level off; allow airspeed to build before reducing power .... LEVEL AT CRUISE ALTITUDE

[ ]  Propeller — set cruise RPM per POH power setting chart .... CRUISE RPM SET PER POH

[ ]  Throttle — set cruise manifold pressure or power per POH chart .... CRUISE POWER SET

[ ]  Mixture — lean to peak EGT then enrichen as directed by POH for power setting and altitude .... LEANED PER POH

[ ]  Fuel selector — confirm BOTH or manage tanks per POH .... PER POH

[ ]  Engine instruments — CHT, EGT, oil temperature and pressure, fuel flow; scan and verify in normal range .... ALL NORMAL

[ ]  Fuel quantity — monitor both tanks; compare against fuel flow and elapsed time for reasonableness .... MONITORED, CONSISTENT WITH PLAN

[ ]  Altimeter setting — update as new reports available .... CURRENT SETTING

[ ]  Autopilot (if equipped) — engage and monitor; do not become complacent; continue scanning engine instruments .... AS DESIRED; MONITORING ACTIVE

Descent

[ ]  ATIS / AWOS / ASOS — obtain destination weather and current altimeter setting .... OBTAINED

[ ]  Altimeter — set destination altimeter setting .... SET

[ ]  Throttle — reduce to descent power; avoid rapid large throttle reductions that can shock-cool cylinders .... REDUCED GRADUALLY PER POH

[ ]  Propeller — adjust per POH for descent configuration .... PER POH

[ ]  Mixture — enrichen as altitude decreases; monitor CHT during descent .... ENRICHENED AS REQUIRED

[ ]  Fuel selector — confirm BOTH .... BOTH

[ ]  Engine instruments — continue monitoring; CHT should not drop rapidly; manage with power as needed .... WITHIN LIMITS, MONITORING

[ ]  G1000 / avionics — program arrival, STAR, and approach as applicable .... PROGRAMMED AND BRIEFED

Before Landing

[ ]  Fuel selector .... BOTH

[ ]  Mixture .... RICH

[ ]  Propeller control — advance to full forward (high RPM) to prepare for go-around .... FULL FORWARD

[ ]  Canopy — confirm latched; no unlock warning .... LATCHED

[ ]  Seat belts and shoulder harnesses — all occupants .... FASTENED AND SNUG

[ ]  Landing light .... ON

[ ]  Flaps — extend to approach/landing position per POH when at or below flap extension speed .... SET PER POH

[ ]  Airspeed — establish approach speed per POH for conditions and flap setting .... APPROACH SPEED PER POH

[ ]  Go-around capability — mentally confirm power-in go-around is available; note any obstacles .... CONFIRMED

Landing

[ ]  Final approach — maintain stable approach speed and glidepath; do not let airspeed decay below approach speed .... STABLE, ON SPEED

[ ]  Threshold — aim for touchdown zone; T-tail requires nose-high attitude; guard against over-rotation and tail strike .... ON CENTERLINE, NOSE HIGH

[ ]  Flare — smooth progressive back pressure on sidestick; allow aircraft to settle .... SMOOTH FLARE

[ ]  Touchdown — main gear first; hold nose off as long as practical, then lower nose gently .... MAIN GEAR FIRST, NOSE LOWERED GENTLY

[ ]  Braking — apply smooth, progressive braking after nose is on ground; avoid hard braking that stresses nose gear .... SMOOTH PROGRESSIVE BRAKING

[ ]  Directional control — use rudder pedals on rollout; keep tracking centerline .... ON CENTERLINE

Go-Around

[ ]  Throttle — smoothly advance to full power without hesitation .... FULL POWER

[ ]  Establish climb attitude — pitch for positive rate; sidestick back to climb attitude; control T-tail pitch sensitivity .... CLIMB ATTITUDE

[ ]  Flaps — retract incrementally per POH as airspeed increases and climb is confirmed .... RETRACTING PER POH

[ ]  Airspeed — accelerate to Vy or Vx as required; do not allow airspeed to decrease below safe value .... CLIMB SPEED PER POH

[ ]  Announce go-around intentions on frequency .... ANNOUNCED

[ ]  Propeller control .... FULL FORWARD IF NOT ALREADY

[ ]  Obstacle clearance — track runway centerline extended until clear of obstacles, then turn as required .... OBSTACLES CLEAR

After Landing / Shutdown and Securing

[ ]  Flaps — retract after clearing runway .... RETRACTED

[ ]  Landing and strobe lights — as appropriate after clearing active runway .... AS APPROPRIATE

[ ]  Transponder — set to standby or ground mode per local procedure .... STANDBY OR GROUND

[ ]  Taxi to parking — taxi at safe speed; use differential braking as required for DA40's ground handling .... TAXIING SAFELY

[ ]  Parking brake — set upon reaching parking position .... SET

[ ]  Avionics master — off before shutdown .... OFF

[ ]  Throttle — idle for brief engine cooling period per POH .... IDLE, COOLING

[ ]  Mixture — pull to idle cut-off; allow engine to stop on fuel starvation .... IDLE CUT-OFF; ENGINE STOPS

[ ]  Ignition — off after engine stops .... OFF

[ ]  All electrical switches and lighting .... OFF

[ ]  Battery master and alternator switch .... OFF

[ ]  Fuel selector .... LEFT or RIGHT (off BOTH) per POH recommendation to prevent fuel siphoning

[ ]  Canopy — open; canopy gas strut check for proper hold-open position .... OPEN AND SECURED

[ ]  Control lock — install on sidestick to prevent wind damage .... INSTALLED

[ ]  Tie-downs and chocks — secure aircraft per local procedures .... AIRCRAFT SECURED

[ ]  Pitot cover — install to prevent insect ingestion and moisture .... INSTALLED

[ ]  Fuel caps — verify both tanks properly capped .... SECURED

[ ]  Post-flight inspection — walk around to check for fluid leaks, damage, or anomalies noted during flight .... COMPLETED, NO DISCREPANCIES (OR NOTED)

NOT FOR FLIGHT — source fidelity: not_applicable | operational review: none | completeness: normal_only | 1 unresolved safety defect(s)

Source: authored — no source document

Rights: original_expression CC-BY-4.0

Checklist hash: afcb0011eaf3c4c25392162b9bc173d227471698b3aed63f4e3f0f82636e7e3b

Verify against the aircraft's own approved documentation before use.