Diamond DA40 Star — da40_checklist_philip_greenspun

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "AFTER TAKE-OFF (500’ AGL)" item count differs between passes (6 vs 3); verify nothing was dropped.

content_gap: Section "RUN UP" item count differs between passes (21 vs 18); verify nothing was dropped.

content_gap: Section "STARTING ENGINE" item count differs between passes (37 vs 25); verify nothing was dropped.

content_gap: Section "TAKE-OFF" item count differs between passes (5 vs 4); verify nothing was dropped.

PRE-START

[ ]  Documents/Preflight Inspection .... CHECK

[ ]  Weight/Balance/Dens. Alt .... CHECK

[ ]  Hobbs Time .... LOGGED

[ ]  Baggage .... STOWED

[ ]  Rudder Pedals .... ADJUSTED/LOCKED

[ ]  Passenger Briefing .... PERFORMED

[ ]  Rear Door .... CLOSED AND LOCKED

[ ]  Harness .... FASTENED AND ADJUSTED

[ ]  Parking Brake .... SET

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Fuel Valve .... EMPTIEST TANK

[ ]  Trim Wheel .... T/O

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Prop Control .... FULL FORWARD

[ ]  Throttle .... FREE & IDLE POSITION

[ ]  Cabin Heat .... OFF

[ ]  Circuit Breakers .... CHECK IN

[ ]  Alternate Air .... CLOSED

[ ]  Alternate Static Valve .... CLOSED

[ ]  Avionics Master .... OFF

[ ]  Ess. Bus Switch .... OFF

[ ]  Electrics .... OFF

[ ]  ALT/BAT Master Switch .... ON

[ ]  Canopy .... CLOSED AND SECURE

[ ]  Strobes .... ON

STARTING ENGINE

COLD START

[ ]  Throttle .... SET 1 INCH

[ ]  Propeller .... FULL FORWARD

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Brakes .... HOLD

[ ]  Propeller Area .... CLEAR

[ ]  Fuel Pump .... ON

[ ]  Mixture .... FULL RICH (3 - 5 SEC.) IDLE CUT-OFF

[ ]  Fuel Pump .... OFF

[ ]  Throttle .... SET TO 1/2 INCH

[ ]  Ignition .... START POSITION

[ ]  Mixture .... ADVANCE

NOTE: Engage starter 10 sec MAX and allow 20 sec. to cool

WARM START

[ ]  Throttle .... SET 1 INCH

[ ]  Propeller .... FULL FORWARD

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Brakes .... HOLD

[ ]  Propeller Area .... CLEAR

[ ]  Fuel Pump .... ON

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Throttle .... 1/2 INCH OPEN

[ ]  Ignition .... START POSITION

[ ]  Mixture .... ADVANCE

NOTE: Engage starter 10 sec MAX and allow 20 sec to cool

UPON ENGINE START

[ ]  Throttle .... MIN 1000 RPM

[ ]  Fuel Pump .... OFF

[ ]  Oil Pressure .... CHECK GREEN

[ ]  Ammeter .... CHECK GREEN

[ ]  Fuel Pressure .... MIN 14 PSI

[ ]  Oil Pressure .... GREEN IN 15 SEC. If not shutdown

[ ]  Annunciator Panel .... CHECK CLEAR

[ ]  Engine Gauges .... GREEN

[ ]  Mixture .... LEAN 1”+

[ ]  Fuel Pump .... OFF

[ ]  Fuel Computer .... UPDATE

BEFORE TAXIING

[ ]  Avionics Master Switch .... ON

[ ]  Aircraft Lighting .... AS REQUIRED

[ ]  ATIS .... COPY

[ ]  Flight Instruments .... SET

[ ]  HSI .... TO COMPASS

[ ]  Radios/GPS .... SET

[ ]  Transponder .... STANDBY/1200

[ ]  Engine Gauges .... CHECK GREEN

[ ]  Annunciator Panel .... CHECK CLEAR

[ ]  Autopilot .... ON, OVERPOWER

[ ]  Autopilot .... ON, STICK BUTTON DISC.

[ ]  Fuel Valve .... SWITCH TANKS

[ ]  Taxi Clearance .... OBTAIN

[ ]  Parking Brake .... RELEASE

TAXIING

[ ]  Brakes .... CHECK

[ ]  Flight Instruments .... TAXI CHECK

RUN UP

[ ]  Line Up Into Wind

[ ]  Parking Brake .... SET

[ ]  Nose Wheel .... STRAIGHT

[ ]  Brakes .... HOLD

[ ]  Oil Temp. .... CHECK GREEN

[ ]  Oil Pressure .... CHECK GREEN

[ ]  Propeller .... FULL FORWARD

[ ]  Throttle .... 2000 RPM

[ ]  Mixture .... MAX POWER GUESS

[ ]  Check Ignition .... LEFT/BOTH/RIGHT/BOTH

[ ]  MAX DROP .... 175 RPM

[ ]  MAX DIFF .... 50 RPM

[ ]  Propeller .... CYCLE 3 TIMES

[ ]  Max Drop .... 500 RPM

[ ]  Alternator Load .... CHECK (1/DAY)

[ ]  Mixture .... MAX POWER + 1”

[ ]  Engine Gauges .... CHECK GREEN

[ ]  Throttle .... IDLE TEST

[ ]  Throttle .... 1000 RPM

[ ]  Alternate Air .... TEST

[ ]  Annunciator Panel .... CHECK CLEAR

BEFORE TAKE-OFF

[ ]  Safety Belts .... FASTENED

[ ]  Canopies .... F LOCK, R LOCK, LIGHT

[ ]  Fuel Valve .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Mixture .... MAX POWER+

[ ]  Propeller .... FULL FORWARD

[ ]  ALT/BAT Master Switch .... ON

[ ]  Ignition Switch .... BOTH

[ ]  Engine Instruments .... CHECK

[ ]  Flight Instruments .... SET

[ ]  Flaps .... VERIFY T/O POSITION

[ ]  Trim .... SET FOR T/O

[ ]  Autopilot .... OFF

[ ]  Annunciator Panel .... CHECK CLEAR

[ ]  Flight Controls .... FREE/CORRECT

[ ]  Fuel Pump .... ON

[ ]  Landing Light .... ON

[ ]  Transponder .... 1200/STANDBY

[ ]  Strobes (Canada) .... ON

[ ]  Parking Brake .... RELEASE

TAKE-OFF

[ ]  Throttle .... FULL POWER

NOTE: Min 2660 RPM

[ ]  Engine Temp/Pressure .... CHECK

[ ]  Rotate .... 59 KTS

[ ]  Climb Speed .... 66 KTS (60 SOLO)

AFTER TAKE-OFF (500’ AGL)

[ ]  Prop .... 2400 RPM

[ ]  Flaps .... UP

[ ]  Climb Speed .... 73 KTS (68 SOLO)

1000’ AGL

[ ]  Fuel Pump .... OFF

[ ]  Landing Light .... OFF

CRUISE

[ ]  Throttle .... SET AS DESIRED (22”)

[ ]  Propeller .... 1800-2400 RPM

[ ]  Mixture .... LEAN (AS REQUIRED)

[ ]  Fuel Valve .... FULLEST TANK

[ ]  Annunciator Panel .... CHECK CLEAR

BEFORE LANDING

[ ]  GEN/BAT Master Switch .... ON

[ ]  Ignition .... BOTH

[ ]  Gauges .... GREEN

[ ]  Breakers .... CHECK IN

[ ]  Landing Light .... ON

[ ]  GUMPS

[ ]  Fuel Valve .... FULLEST TANK

[ ]  Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Prop .... 2400 RPM

[ ]  Seatbelts .... FASTENED

FINAL APPROACH

[ ]  Flaps .... LANDING

[ ]  Airspeed .... 67 (63 SOLO)

[ ]  Mixture .... RICH

[ ]  Propeller .... FULL FORWARD

GO-AROUND

[ ]  Throttle .... FULL

[ ]  Flaps .... T/O

[ ]  Airspeed .... 66 (60 SOLO)

CLEAR OF RUNWAY

[ ]  Throttle .... 1000 RPM

[ ]  Flaps .... UP

[ ]  Strobes .... OFF (CANADA)

[ ]  Landing Light .... OFF

[ ]  Fuel Pump .... OFF

[ ]  Mixture .... LEAN 1”+

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Diamond/DA40 Star/da40_checklist_philip_greenspun.txt

Rights: licensed

Checklist hash: 8a5b995cb868237c197a952ab863ad617325ae5c8054773f0d9646a1c8536d9d

Verify against the aircraft's own approved documentation before use.