Diamond DA40 Star — Jacob_Esparza_Diamond_Star_DA40_Normal_Operating_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Landing Check" item count differs between passes (15 vs 16); verify nothing was dropped.

BEFORE START CHECK

[ ]  Preflight .... COMPLETE

[ ]  Pedals .... ADJUSTED AND LOCKED

[ ]  Passengers .... BRIEFED

[ ]  Seat Belts .... SECURED

[ ]  Rear Hatch .... CLOSED AND LOCKED

[ ]  Canopy .... CLOSED AND LOCKED

[ ]  Parking Brake .... SET

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Trim .... SET T/O

[ ]  Circuit Breakers .... CHECK

[ ]  Throttle .... IDLE

[ ]  Prop .... FULL FORWARD

[ ]  Mixture .... LEAN

[ ]  Friction Device .... ADJUSTED

[ ]  Alternate Engine Air .... CLOSED

[ ]  Alternate Static .... CLOSED

[ ]  Avionics Master .... OFF

[ ]  Essential Bus .... OFF

[ ]  Master Switch (BAT) .... ON

[ ]  Annunciator Panel .... TEST

[ ]  Fuel Tank Selector .... FULLEST TANK

[ ]  Strobes .... ON

COLD START CHECK

[ ]  Fuel Pump .... ON

[ ]  Throttle .... 1 1/2 INCH FORWARD

Source reads '1 ½ ” Forward'; the 1/2 inch symbol is a transcription reading.

[ ]  Mixture .... RICH 3-5 SEC, THEN LEAN

[ ]  Throttle .... 1/2 INCH FORWARD

Source reads '½” Forward'.

[ ]  Starter .... ENGAGE

NOTE: After Engine Fires

[ ]  Mixture .... FULL RICH

[ ]  Oil Pressure .... GREEN WITHIN 15 SEC

[ ]  Fuel Pump .... OFF

[ ]  Master Switch (ALT) .... ON

[ ]  Ammeter .... CHECK

[ ]  Fuel Pressure .... CHECK 14-35 PSI

[ ]  Annunciator Panel .... CHECK

HOT START CHECK

[ ]  Fuel Pump .... ON

[ ]  Throttle .... 1 1/2 INCH

Source reads '1 ½"'.

[ ]  Mixture .... RICH 1-3 SEC, THEN LEAN

[ ]  Starter .... ENGAGE

NOTE: After Engine Fires

[ ]  Mixture .... FULL RICH

[ ]  Oil Pressure .... GREEN WITHIN 15 SEC

[ ]  Fuel Pump .... OFF

[ ]  Master Switch (ALT) .... ON

[ ]  Ammeter .... CHECK

[ ]  Fuel Pressure .... CHECK 14-35 PSI

[ ]  Annunciator Panel .... CHECK

FLOODED ENGINE START

[ ]  Fuel Pump .... OFF

[ ]  Throttle .... HALF WAY FORWARD

[ ]  Mixture .... FULL LEAN

[ ]  Starter .... ENGAGE

[ ]  Throttle .... RETARD AFTER ENGINE FIRES

NOTE: Refer To After Engine Fires Above

AFTER START CHECK

[ ]  Avionics Master .... ON

[ ]  Electrical Equipment .... AS REQUIRED

[ ]  Flaps .... UP-T/O-LDG-T/O

[ ]  Weather .... OBTAIN

[ ]  Flight Instruments .... SET

[ ]  Avionics and Transponder .... SET

[ ]  Ammeter .... CHECK

[ ]  Fuel Tank Selector .... SWITCH TANKS

[ ]  Verify Engine Runs on Both Tanks .... VERIFY

[ ]  Pitot Heat .... TEST (AMMETER MUST RISE)

AFTER START CHECK (CONT)

[ ]  Pitot Heat .... OFF OR AS NEEDED

[ ]  Lights .... AS REQUIRED

[ ]  RPM .... CHECK IDLE 600-800 RPM

[ ]  Brakes .... RELEASE AND TEST

[ ]  Gyro Instruments .... CHECK DURING TAXI

Run-Up Check

[ ]  Align Aircraft Into Wind

[ ]  Parking Brake .... SET

[ ]  Seats and Belts .... SECURE

[ ]  Front Canopy .... CLOSED AND LOCKED

[ ]  Rear Canopy .... CLOSED AND LOCKED

[ ]  Door Warning Lights .... OFF

[ ]  Fuel Tank Selector .... FULLEST TANK

[ ]  Engine Instruments .... GREEN

[ ]  Circuit Breakers .... CHECK IN

[ ]  Fuel Pressure .... 15-25 PSI

[ ]  Fuel Pump .... ON

[ ]  Mixture .... ADJUSTED (4 BARS)

[ ]  Flaps .... VERIFY T/O

[ ]  Trim .... VERIFY T/O

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Throttle .... 2000 RPM

[ ]  Prop .... CYCLE 3 TIMES

[ ]  Decrease in RPM

[ ]  Decrease in Oil Pressure

[ ]  Increase in Manifold Pressure

[ ]  Magnetos .... CHECK

NOTE: Max RPM Drop: 175 RPM

NOTE: Max Difference: 50 RPM

[ ]  Throttle .... IDLE

[ ]  Alternate Air .... CLOSED

[ ]  Lights .... AS REQUIRED

[ ]  Transponder .... ON/ALT

[ ]  Traffic .... CHECK BASE, FINAL AND RWY

[ ]  Parking Brake .... RELEASE

Take Off Check

[ ]  Prop .... FULL FORWARD

[ ]  Throttle .... SMOOTHLY INCREASE

NOTE: If 2680 +/- 20 RPM not obtained, Abort

[ ]  Engine Instruments .... MONITOR

[ ]  Rotate Vr .... 60 KTS

NOTE: Best Rate Vy: 65 kts

NOTE: Best Angle Vx: 60 kts

500' AGL or Safe Altitude

[ ]  Prop .... 2400 RPM

[ ]  Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Flaps .... UP

Cruise Check

[ ]  Throttle .... SET TO POWER TABLE

[ ]  Landing Light .... VERIFY OFF

[ ]  Fuel Pump .... VERIFY OFF

NOTE: Cruise Climb: 70-85 kts

[ ]  Prop .... 1800-2400 RPM

[ ]  CHT .... 150-400 F

[ ]  Oil Temp .... 165-220 F

[ ]  Mixture .... ADJUST

[ ]  Trim .... AS NEEDED

[ ]  Fuel Tank Selector .... FULLEST TANK

NOTE: Turn Fuel Pump On Before Switching

NOTE: Max Difference in Tanks: 10 Gal

Reference Speeds

NOTE: Vso (dirty stall speed): 49 kts

NOTE: Vs1 (clean stall speed): 52 kts

NOTE: Vr (rotation speed): 60 kts

NOTE: Vx (best angle): 60 kts

NOTE: Vy (best rate): 65 kts

NOTE: Vref (approach speed): 65 kts

NOTE: Vglide (best glide speed): 75 kts

NOTE: Vfe (flap extend speed): T/O 108 kts

NOTE: Vfe (flap extend speed): LDG 90 kts

Decent and Landing Checks on Back

NOTE: Diamond Star DA40-180 N607C Page 2 of 2 Normal Operating Checklist

Decent Check

[ ]  Mixture .... ADJUST

[ ]  Prop .... 1800-2400 RPM

[ ]  Throttle .... AS NEEDED

[ ]  Landing Light .... ON

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Fuel Pump .... ON

[ ]  Seatbelts .... SECURED

[ ]  Destination Weather .... OBTAIN

[ ]  IFR Approach .... BRIEF

[ ]  Avionics .... SET FOR APPROACH

Landing Check

APPROACH

[ ]  Fuel Selector .... VERIFY FULLEST TANK

[ ]  Fuel Pump .... VERIFY ON

[ ]  Seat Belts .... SECURED

[ ]  Airspeed .... AT OR BELOW VFE

[ ]  Flaps .... T/O

[ ]  Trim .... AS NEEDED

[ ]  Lights .... AS REQUIRED

BEFORE LANDING

[ ]  Mixture .... FULL RICH

[ ]  Prop .... FULL FORWARD

[ ]  Throttle .... 13 INCH MP

Source reads '13” MP'.

[ ]  Flaps .... LDG

[ ]  Airspeed .... 65-70 KTS

NOTE: Note: If landing in strong wind and turbulence use a higher approach speed and/or use less flaps.

Go-Around Check

[ ]  Throttle .... FULL FORWARD

[ ]  Airspeed .... 60 KTS

[ ]  Flaps .... T/O

AT 500' AGL OR SAFE ALTITUDE

[ ]  Prop .... 2400 RPM

[ ]  Airspeed .... 70 KTS

[ ]  Flaps .... UP

[ ]  Fuel Pump .... OFF

[ ]  Landing .... REFER TO LANDING CHECK

Clear Runway Check

[ ]  Throttle .... IDLE

[ ]  Brakes .... AS NEEDED

[ ]  Fuel Pump .... OFF

[ ]  Transponder .... STBY/OFF

[ ]  Pitot Heat .... OFF

[ ]  Avionics .... AS NEEDED

[ ]  Landing Light .... OFF

[ ]  Other Lights .... AS NEEDED

[ ]  Flaps .... UP

Engine Shutdown Check

[ ]  Parking Brake .... SET

[ ]  Engine Instruments .... CHECK

[ ]  Avionics Master .... OFF

[ ]  All Electrical Equip .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Ignition .... OFF TILL DROP IN RPM THEN BOTH

[ ]  Mixture .... FULL LEAN

[ ]  Ignition .... OFF

[ ]  Tach. Reading .... OBTAIN

[ ]  Master Switch .... OFF

[ ]  Discrepancies .... LOG

[ ]  Control Lock .... SECURE

[ ]  Pitot and Stall Cover .... ON

[ ]  Canopy .... CLOSED AND LOCKED

[ ]  Wheel Chocks/Tie downs .... SECURE

[ ]  Flight Plan .... CLOSED

Performance T/O and LDG - Short Field Takeoff

[ ]  Flaps .... T/O

[ ]  Brakes .... HOLD

[ ]  Prop .... FULL FORWARD

[ ]  Throttle .... FULL FORWARD

[ ]  Rotate .... 60 KTS

[ ]  Climb .... 65 KTS

[ ]  Obstacle Cleared .... 70 KTS

NOTE: 500' AGL or Safe Altitude

[ ]  Prop .... 2400 RPM

[ ]  Flaps .... UP

Soft Field Takeoff

[ ]  Elevator .... FULL AFT

[ ]  Flaps .... T/O

[ ]  Prop .... FULL FORWARD

[ ]  Throttle .... FULL FORWARD

[ ]  Airspeed .... ACCELERATE IN GROUND EFFECT

[ ]  Climb .... VX UNTIL OBSTACLE CLEARED THEN VY

NOTE: 500' AGL or Safe Altitude

[ ]  Prop .... 2400

[ ]  Flaps .... UP

[ ]  Landing Light .... OFF

[ ]  Fuel Pump .... OFF

Short Field Landing

[ ]  Landing Light .... ON

[ ]  Fuel Pump .... ON

[ ]  Touchdown Point .... ACQUIRE

[ ]  Prop .... FULL FORWARD

[ ]  Mixture .... RICH

[ ]  Power .... AS NEEDED

[ ]  Seat Belts .... SECURED

[ ]  Flaps .... LDG

[ ]  Airspeed .... 60-65 KTS

Soft Field Landing

[ ]  Landing Light .... ON

[ ]  Fuel Pump .... ON

[ ]  Mixture .... FULL RICH

[ ]  Prop .... FULL FORWARD

[ ]  Throttle .... AS NEEDED

[ ]  Seat Belts .... SECURE

[ ]  Flaps .... LDG

[ ]  Airspeed .... 60-65 KTS

[ ]  At touchdown hold nose wheel off as long as possible

General Information

NOTE: Fuel Indicator: Indicates no more than 15 Gallons per tank

Source line is broken across two lines; meaning is clear.

NOTE: Fuel Capacity: 40 gallons

NOTE: Usable Fuel: 39 Gallons

NOTE: Max difference in tanks: 10 Gallons

NOTE: Oil: Min 4 qts, Max 8 qts, Rec. 6 qts

NOTE: Do not operate starter more than 10 sec at a time. After 10 seconds, let cool 20 seconds. After 6 attempts let cool 30 mins.

NOTE: Max CHT: 500°F

NOTE: CHT cooling rate: less than 50° per min

NOTE: No Baggage allowed w/o baggage net

NOTE: Tire Pressure Nose Wheel: 29 psi

NOTE: Tire Pressure Main Wheels: 36 psi

Local Freq.'s

ALBUQUERQUE SUNPORT (KABQ)

NOTE: ATIS: 118.00

NOTE: Clearance: 119.20

NOTE: Ground: 121.90

NOTE: Tower: 120.30

NOTE: North Depart/Apprch: 127.40

NOTE: South Depart/Apprch: 123.90

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Diamond/DA40 Star/Jacob_Esparza_Diamond_Star_DA40_Normal_Operating_Checklist.txt

Rights: licensed

Checklist hash: 3fe20475d36fd8404c318229ceb518d0c6130a7bf4c76306f251e3395738e3b3

Verify against the aircraft's own approved documentation before use.