Diamond DA40 Star — Jacob_Esparza_Diamond_Star_DA40_Pre-Flight_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Front Fuselage" item count differs between passes (9 vs 11); verify nothing was dropped.

Cabin Check

[ ]  Maintenance Check

[ ]  Weight and Balance .... COMPUTED

[ ]  Airplane Documents .... ARROW ON BOARD

[ ]  Ignition Key .... REMOVED

[ ]  Front and Rear canopy .... UNDAMAGED/CLEAN

[ ]  All Electrical Equipment .... OFF

[ ]  Circuit Breakers In

[ ]  Engine Control Levers .... CHECK CONDITION

[ ]  Throttle .... IDLE

[ ]  Mixture .... FULL LEAN

[ ]  Prop .... FULL FORWARD

[ ]  Master Switch (BAT) .... ON

[ ]  Annunciator Panel

[ ]  Fuel Quantity .... CHECK

[ ]  Lights .... ON/CHECK

[ ]  Master Switch (BAT) .... OFF

[ ]  Loose Items Secure

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Baggage .... SECURED AND STOWED

Left Main Landing Gear

[ ]  Landing Gear Strut

[ ]  Wheel Fairing

[ ]  Tire Pressure .... CHECK 36 P.S.I

[ ]  Tire Wear

[ ]  Tire, Wheel, Brake

[ ]  Brake Line

[ ]  Slip Marks

[ ]  Chocks .... REMOVED

Left Wing

[ ]  Wing Surface

[ ]  Step

[ ]  Lower Surface Air Intake

[ ]  Lower Surface Openings .... CHECK FOR FUEL

[ ]  Fuel Sump

[ ]  Stall Warning

[ ]  Tank Filler .... VISUALLY CHECK

[ ]  2 Stall Strips on Wing .... CHECK

[ ]  Pitot Tube .... CLEAN ORFICES OPEN

[ ]  Landing/Taxi Light

[ ]  Wing Tip

[ ]  Strobes and Position Lights

[ ]  Mooring .... CHECK CLEAR

[ ]  Aileron and Linkages

[ ]  Foreign Objects in Aileron Paddle .... REMOVE

[ ]  Flap and Linkages

[ ]  Flap Hinges and Safety Pin

Left Fuselage

[ ]  Canopy, left side .... CHECK

[ ]  Rear Cabin Door and Window

[ ]  Fuselage Skin

[ ]  Antennas

Empennage

[ ]  Stabilizers and control surfaces

[ ]  Hinges

[ ]  Elevator Trim Tab

[ ]  Rudder Trim Tab

[ ]  Mooring on fin .... CHECK CLEAR

[ ]  Tail Skid and lower fin

Right Fuselage

[ ]  Fuselage Skin

[ ]  Window

[ ]  Canopy, Right Side .... CHECK

Right Wing

[ ]  Flap and Linkage

[ ]  Flap hinges and safety pin

[ ]  Aileron and Linkages

[ ]  Aileron hinges and safety pin

[ ]  Foreign objects in aileron paddle .... REMOVE

[ ]  Wing tip

[ ]  Position and Strobe Lights

[ ]  Mooring .... CHECK CLEAR

[ ]  Wing Surface

[ ]  2 Stall Strips on Wing .... CHECK

[ ]  Tank Filler .... VISUALLY CHECK

[ ]  Lower Surface Openings .... CHECK FOR FUEL

[ ]  Fuel Sump

[ ]  Step

Right Main Landing Gear

[ ]  Landing Gear Strut

[ ]  Wheel Fairing

[ ]  Tire Inflation .... CHECK 36 PSI

[ ]  Tire Wear and Depth

[ ]  Tire Wheel Brake

[ ]  Brake Line .... CHECK FOR LEAKS

[ ]  Slip Marks

[ ]  Chocks .... REMOVE

Front Fuselage

[ ]  Oil Level .... MIN 4 QTS FOR VFR / MIN 6 QTS FOR IFR

[ ]  Cowling

[ ]  3 Air Intakes .... CLEAR

[ ]  Propeller .... MAX BLADE SHAKE 3 MM / MAX ANGULAR PLAY 2°

[ ]  Spinner and Screws

[ ]  Nose Gear

[ ]  Tire and Wheel

[ ]  Slip Marks

[ ]  Tire Wear and Depth

Front Fuselage (cont)

[ ]  Wheel Fairing

[ ]  Tow Bar .... REMOVED

[ ]  Tire Pressure .... CHECK 29 PSI

[ ]  Chocks .... REMOVE

[ ]  Exhaust

Underside

[ ]  Antennas (if fitted) .... CHECK

[ ]  Gascolator .... STRAIN

[ ]  Venting Pipes .... CHECK FOR BLOCKAGE

[ ]  Fuselage Underside .... CHECK FOR FLUIDS

Happy Flying!!!

NOTE: Happy Flying!!!

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Diamond/DA40 Star/Jacob_Esparza_Diamond_Star_DA40_Pre-Flight_Checklist.txt

Rights: licensed

Checklist hash: a1d4a37ef85d256cbfb9e356d8443a9e4bef0df9cb3b8556fd2195cd81b8ea67

Verify against the aircraft's own approved documentation before use.