Diamond DA40 Star — Sam_Dawson_DA40_g1000_checklist_singlesheet

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "CRUISE" item count differs between passes (12 vs 5); verify nothing was dropped.

PRE-START

[ ]  Documents/Preflight Inspection .... CHECK

[ ]  Weight/Balance/Dens. Alt .... CHECK

[ ]  Baggage .... STOWED

[ ]  Rudder Pedals .... ADJUSTED/LOCKED

[ ]  Passenger Briefing .... PERFORMED

[ ]  Rear Door .... CLOSED and LOCKED

[ ]  Harness .... FASTENED AND ADJUSTED

[ ]  Parking Brake .... SET

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Fuel Valve .... EMPTIEST TANK

[ ]  Trim Wheel .... T/O

[ ]  Throttle Friction .... Adjusted

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Prop Control .... FULL FORWARD

[ ]  Throttle .... IDLE POSITION

[ ]  Cabin Heat .... OFF

[ ]  Circuit Breakers .... CHECK IN

[ ]  Alternate Air .... CLOSED

[ ]  Alternate Static Valve .... CLOSED

[ ]  Avionics Master .... OFF

[ ]  Ess. Bus Switch .... OFF

[ ]  ALT/BAT Master Switch .... ON

[ ]  Canopy .... Position 1 or 2

[ ]  Strobes .... ON

STARTING ENGINE

NOTE: engage starter 10 sec MAX; 20 Sec to cool

COLD START

[ ]  Throttle .... FULL FORWARD

[ ]  Propeller .... FULL FORWARD

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Brakes .... HOLD

[ ]  Propeller Area .... CLEAR

[ ]  Fuel Pump .... ON

[ ]  Mixture .... FULL RICH  (3 - 5 Sec.) IDLE CUT-OFF

[ ]  Fuel Pump .... OFF

[ ]  Throttle .... SET TO ½ INCH

[ ]  Ignition .... START POSITION

[ ]  Mixture .... ADVANCE

WARM START

[ ]  Propeller .... FULL FORWARD

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Brakes .... HOLD

[ ]  Propeller Area .... CLEAR

[ ]  Fuel Pump .... ON

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Pump .... OFF

[ ]  Throttle .... ½ inch OPEN

[ ]  Ignition .... START POSITION

[ ]  Mixture .... ADVANCE

UPON ENGINE START

[ ]  Throttle .... min 1000 RPM

[ ]  Oil Pressure .... CHECK GREEN

[ ]  Ammeter .... CHECK GREEN

[ ]  Alternator .... ON

[ ]  Oil Pressure .... GREEN IN 15 SEC.

[ ]  If not shutdown

[ ]  Alerts lower right PFD .... CHECK CLEAR

[ ]  Engine Gauges .... GREEN

[ ]  Mixture .... LEAN 1”+

[ ]  Fuel Computer .... UPDATE

BEFORE TAXIING

[ ]  Avionics Master Switch .... ON

[ ]  PFD .... No Autopilot Annunciation

[ ]  Autopilot Disconnect Tone .... Note

[ ]  Flaps .... UP-T/O-LDG-T/O (check)

[ ]  Ammeter .... Check

[ ]  Aircraft Lighting .... AS REQUIRED

[ ]  ATIS .... COPY

[ ]  Flight Instruments Altimeters .... SET

[ ]  Radios/GPS .... SET

[ ]  Engine Gauges .... CHECK GREEN

[ ]  Annunciator Panel .... CHECK CLEAR

[ ]  Fuel Valve .... SWITCH TANKS To FULLEST

[ ]  Clearance .... OBTAIN

[ ]  Parking Brake .... RELEASE

TAXIING

[ ]  Brakes .... Check

[ ]  Flight Instruments .... TAXI CHECK

[ ]  TRAFFIC/MUTE annunciator .... press twice

RUN UP

[ ]  Parking Brake .... SET

[ ]  Safety Harnesses .... Fastened

[ ]  Rear Door .... Check closed/locked

[ ]  Flight Controls .... FREE/CORRECT

[ ]  Trim .... SET FOR T/O

[ ]  Fuel Valve .... CHECK fullest tank

[ ]  Circuit Breakers .... IN

[ ]  Engine Instruments .... Green

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Pressure .... No MSG

[ ]  Flight Instruments .... SET

[ ]  Electric Pump .... ON

[ ]  Propeller .... FULL FORWARD

[ ]  Mixture .... Lean above 5000’ DA

[ ]  Throttle .... 2000 RPM

[ ]  Check Ignition .... LEFT/BOTH/RIGHT/BOTH

[ ]  MAX DROP .... 175 RPM

[ ]  MAX DIFF .... 50 RPM

[ ]  Propeller .... CYCLE 3 TIMES

[ ]  Max Drop .... 500 RPM

[ ]  Alternator Load .... CHECK (1/day)

[ ]  Mixture .... MAX POWER + 1”

[ ]  Engine Gauges .... CHECK GREEN

[ ]  Throttle .... 1500 RPM

[ ]  Alternate Air .... TEST

[ ]  Throttle .... IDLE TEST

[ ]  Throttle .... 1000 RPM

[ ]  Alerts lower right PFD .... CHECK CLEAR

BEFORE TAKE – OFF

[ ]  Flaps .... T/O

[ ]  Canopies .... F LOCK, R LOCK, LIGHT

[ ]  Alerts lower right PFD .... CHECK CLEAR

[ ]  Altimeter .... check w/ elevation

[ ]  Landing Light .... ON

[ ]  Strobes .... ON

[ ]  Heading .... Check aligned with runway

[ ]  Time .... note

TAKE-OFF

[ ]  Throttle .... FULL POWER

NOTE: min 2660 RPM

[ ]  Engine Temp/Pressure .... CHECK

[ ]  Rotate .... 59 kts

[ ]  Climb Speed .... 66 Kts (60 solo)

AFTER TAKE-OFF (500’ AGL)

[ ]  Flaps .... UP

[ ]  Climb Speed .... 73 Kts (68 solo)

Climb Check (1000’ AGL)

[ ]  Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  RPM .... as required

[ ]  Throttle .... as required

[ ]  Mixture .... Lean as necessary

[ ]  Engine Instruments .... in green

[ ]  Airspeed .... 80 KIAS

CRUISE

[ ]  Throttle .... SET AS DESIRED (22”)

[ ]  Propeller .... 1800-2700 RPM (MT)

[ ]  Mixture .... LEAN (as required)

[ ]  Fuel Valve .... FULLEST TANK

[ ]  Annunciator Panel .... CHECK CLEAR

IN RANGE CHECK

[ ]  Mixture .... Adjust

[ ]  Electrical Pump .... ON at high altitude

[ ]  ATIS .... Obtain

[ ]  Avionics .... Set

[ ]  Approach Brief .... Complete

[ ]  Lights .... ON

BEFORE LANDING CHECK

[ ]  Fuel Valve .... FULLEST TANK

[ ]  Fuel Pump .... ON

[ ]  Safety Harness .... Fastened

[ ]  Airspeed .... Reduce to flap speed

[ ]  Flaps .... T/O (if desired)

[ ]  Trim .... As Required

[ ]  Landing Light .... ON

FINAL APPROACH

[ ]  Mixture .... RICH (below 5000’ DA)

[ ]  RPM .... High RPM

[ ]  Flaps .... LDG

[ ]  Approach Speed .... 71/67/63/58

GO-AROUND

[ ]  Throttle .... FULL

[ ]  GA Button .... Press; verify GA/GA in PFD

[ ]  Flaps .... T/O

[ ]  Airspeed .... 66 (60 solo)

[ ]  Altitude Pre-select .... Set

[ ]  GPS .... Set as desired

CLEAR OF RUNWAY

[ ]  Throttle .... 1000 RPM

[ ]  Flaps .... UP

[ ]  Landing Light .... OFF

[ ]  Fuel Pump .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Mixture .... Lean if necessary

Source Attribution

NOTE: Blackhawk Aviation

NOTE: El Paso, Texas

NOTE: Samdawsoncfi.com

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Diamond/DA40 Star/Sam_Dawson_DA40_g1000_checklist_singlesheet.txt

Rights: licensed

Checklist hash: a3bb0489d1d47c45cfd3be2235c2b41ed9259fd64b7c416b8ac777b2e275cfc9

Verify against the aircraft's own approved documentation before use.