Diamond DA40 Star — Sam_Dawson_DA40_Ground_Checklist_jan07

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Starting Engine

[ ]  Preflight Inspection .... Complete

[ ]  Rudder Pedals .... Adjusted/locked

[ ]  CRM/Passenger Brief .... Complete

[ ]  Safety Harnesses .... On/Fastened

[ ]  Baggage .... Check/secured

[ ]  Rear Door .... Closed/secured

[ ]  Door Lock .... Unlocked, key removed

[ ]  Front canopy .... Position 1 or 2

[ ]  Canopy Lock .... Unblocked

[ ]  Parking Brake .... Set

[ ]  Flight Controls .... Free

[ ]  Trim Wheel .... TO

[ ]  Throttle .... Idle

[ ]  RPM Lever .... HIGH RPM

[ ]  Mixture Control Lever .... Lean

[ ]  Friction device, throttle .... Adjusted

[ ]  Alternate Air .... Closed

[ ]  Alternate Static Valve .... Closed

[ ]  Avionics Master .... OFF

[ ]  Essential Bus Switch .... OFF

[ ]  Master Switch (BAT) .... ON

[ ]  Fuel Tank Selector .... Fullest Tank

Starting Engine (cold)

[ ]  Strobe Light .... ON

[ ]  Electrical Fuel Pump .... ON (check noise)

[ ]  Throttle .... 1.2” forward

[ ]  Mixture Control Lever .... Rich 3-5 sec, then LEAN

[ ]  Throttle .... IDLE

[ ]  Ignition Switch .... START

WHEN ENGINE STARTS:

[ ]  Mixture Control Lever .... Rapidly to RICH

[ ]  Oil Pressure .... Green within 15 Seconds

[ ]  Electric Fuel Pump .... OFF

[ ]  ALTernator Switch .... ON

[ ]  Ammeter .... Check

[ ]  Fuel Pressure .... Check No MSG

[ ]  Annunciator Section of PFD .... Check

Starting Engine (warm)

[ ]  Strobe Light .... ON

[ ]  Electrical Fuel Pump .... ON (check noise)

[ ]  Throttle .... 1.2” forward

[ ]  Mixture Control Lever .... Rich 1-3 sec, then LEAN

[ ]  Ignition Switch .... START

WHEN ENGINE STARTS:

[ ]  Mixture Control Lever .... Rapidly to RICH

[ ]  Oil Pressure .... Green within 15 Seconds

[ ]  Electric Fuel Pump .... OFF

[ ]  ALTernator Switch .... ON

[ ]  Ammeter .... Check

[ ]  Fuel Pressure .... Check No MSG

[ ]  Annunciator Section of PFD .... Check

Before Taxi Check

[ ]  Avionics Master .... ON

[ ]  PFD .... NO AUTOPILOT ANNUCIATIONS

[ ]  Autopilot Disconnect Tone .... NOTE

[ ]  Electrical Equipment .... On as required

[ ]  Flaps .... UP-T/O-LDG-T/O (indicator/visual check)

[ ]  Flood light .... On, Test as Req.

[ ]  Ammeter .... Check

[ ]  Fuel Tank Selector .... Change tanks, confirm engine runs 1 min @1500RPM

[ ]  Pitot Heating .... On, test (no annunciator)

[ ]  Pitot Heating .... OFF

[ ]  Lights .... Check

[ ]  Throttle .... Check 600-800 RPM

[ ]  ATIS/AWOS/ASOS .... Obtain

[ ]  Flight Instruments/avionics .... Set

[ ]  Altimeters .... Set (both)

[ ]  Clearance .... Obtain

[ ]  Fuel Totalizer .... Check, set

[ ]  Flight Plan .... Set, check

Taxi Check

[ ]  Brakes .... Test

[ ]  Flight Instruments .... Check

Run Up

[ ]  Parking Brake .... Set

[ ]  Rear Door .... Check closed/locked

[ ]  Front Canopy .... Closed, locked

[ ]  Safety Harnesses .... ON/Fastened

[ ]  Flight Controls .... Free, correct

[ ]  Trim .... Check T/O

[ ]  Fuel Tank Selector .... Fullest Tank

[ ]  Circuit Breakers .... IN

[ ]  Flaps .... Check T/O

[ ]  Engine Instruments .... Green Sector

[ ]  Door Warning Light .... Check ext.

[ ]  Fuel Pressure .... Check NO MSG

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture Control Lever .... RICH (below 5000’DA)

[ ]  Throttle .... 2000 RPM

[ ]  Magneto .... Check (175±50 RPM)

[ ]  RPM Lever .... Cycle 3x (max 500RPM)

[ ]  Throttle .... 1000 RPM

[ ]  Parking Brake .... Release

[ ]  Lights .... On as required

Engine Shut-Down

[ ]  Parking Brake .... Set

[ ]  Engine Instruments .... Check

[ ]  Avionics Master Switch .... OFF

[ ]  All Electrical Equipment .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Ignition Check .... OFF until RPM Drop; the BOTH

[ ]  Mixture Control Lever .... LEAN- shut engine off

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Diamond/DA40 Star/Sam_Dawson_DA40_Ground_Checklist_jan07.txt

Rights: licensed

Checklist hash: 2d3cc88e5257b67305d97e661163915e15db7b85ef925bc431418ec4f26bf147

Verify against the aircraft's own approved documentation before use.