Diamond DA40 Star — Sam_Dawson_DA40_InFlight_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "After Landing" item count differs between passes (8 vs 7); verify nothing was dropped.

BEFORE TAKEOFF (FINAL ITEMS)

[ ]  Heading .... Aligned w/ runway

[ ]  Altitude .... Correct

[ ]  Transponder .... ALT

[ ]  Taxi, Landing/Strobe Lights .... On

[ ]  Time .... Note

CLIMB CHECK

[ ]  Flaps .... UP

[ ]  Airspeed .... 80 KIAS

[ ]  RPM .... 2400 RPM

[ ]  Throttle .... MAX POWER

[ ]  Mixture .... RICH; above 5000’ hold EGT constant

[ ]  Engine Instruments .... In Green

[ ]  Trim .... As Required

[ ]  Electrical Fuel Pump .... ON at high Altitude

CRUISE CHECK

[ ]  Flaps .... UP

[ ]  Throttle .... Set

[ ]  RPM .... 1800-2400 RPM

[ ]  Mixture .... SET IAW p. 4A.3.10

[ ]  Trim .... As Required

[ ]  Fuel Tank Selector .... As Required

[ ]  Electrical Fuel Pump .... On at high altitude

DESCENT CHECK

[ ]  Mixture .... Adjust

[ ]  RPM .... 1800-2400

[ ]  Throttle .... As Required

[ ]  Electrical fuel pump .... ON at high altitude

[ ]  ATIS .... Obtain

[ ]  LIGHTS .... On

[ ]  Avionics .... Set

[ ]  Approach Briefing .... Complete

TURN-TIME-TWIST-TAP-THROTTLE-TALK

NOTE: Vr- 59, Vx-66, Vy-73, Vfe-108/91, VA-108/94, Vref-71/67, Glide-73, VNO-129, VNE- 178

NOTE: 22 January 2007

Approach Brief (bold items for VFR)

[ ]  Approach .... Type, Runway

[ ]  GPS .... Set

[ ]  Frequency .... Set, ID

[ ]  Final Approach Course

[ ]  FAF

[ ]  DH/MDA

[ ]  Approach Lights/PAPI/VASI .... Pilot activated?

[ ]  TDZE, HAT/HAA

[ ]  MAP- time, DME, GPS WP? .... Note if GPS WP or DME

[ ]  Missed Approach Procedure .... Review

[ ]  Notes .... Review

[ ]  Runway exit/taxi plan .... Review Direction

[ ]  Terrain .... Highest/MSA

[ ]  Before Landing Check .... To the line

[ ]  Min. Alt., MAP & Climb Out .... Complete

BEFORE LANDING CHECK

[ ]  Fuel Selector .... Fullest Tank

[ ]  Electrical Pump .... ON

[ ]  Safety Harness .... fastened

[ ]  Airspeed .... Reduce to flaps speed

[ ]  Flaps .... T/O (if desired)

[ ]  Trim .... As required

[ ]  Landing light .... As Required

----------------------------SHORT FINAL---------------------------------------

[ ]  Mixture .... Rich (below 5000’ DA)

[ ]  RPM .... HIGH RPM

[ ]  Airspeed .... Reduce to flap speed

[ ]  Flaps .... LDG

[ ]  Approach Speed .... 71/67/63/58

GO AROUND

[ ]  Throttle .... MAX PWR, Press GA, Verify GA/GA in PFD

[ ]  Airspeed .... 66 KIAS

[ ]  Flaps .... TO

[ ]  RPM Lever .... 2400

[ ]  Airspeed .... 73

[ ]  Flaps .... UP

[ ]  Electrical Pump .... OFF

[ ]  Altitude Pre-select .... SET

[ ]  GPS .... Set

After Landing

[ ]  Fuel Pump .... Off

[ ]  Transponder .... Confirm OFF

[ ]  Pitot Heat .... Off

[ ]  Avionics .... As required

[ ]  Lights .... As Required

[ ]  Flaps .... UP

TURN- TIME-TWIST-TAP-THROTTLE-TALK

NOTE: 22 January 2007

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Diamond/DA40 Star/Sam_Dawson_DA40_InFlight_Checklist.txt

Rights: licensed

Checklist hash: f8a776af0801b6961afcf55b32db38ef4e5f6421ad1de89207dedc9868ef46a7

Verify against the aircraft's own approved documentation before use.