Embraer Emb-120 Brasilia — MAX_Emb120_ Check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Before Start

[ ]  Fasten Belts .... ON

[ ]  External Lights .... AS REQUIRED

[ ]  APU .... AS REQUIRED

[ ]  Flaps Bit .... CHECKED

[ ]  Air Conditioning .... AS REQUIRED

[ ]  Radio Master .... ON

[ ]  Instruments .... SET / X-CHECKED

[ ]  Radios/Radar/XPDR .... SET / STAND BY

[ ]  Fuel Totalizer .... SET

[ ]  Fuel Quantity .... CHECKED

Before Start 2

[ ]  Flight Recorder .... SET

[ ]  Power Levers .... GRD IDLE

[ ]  Condition Levers .... FUEL CUT OFF

[ ]  Press sys .... SET

[ ]  Parking Brake .... ON

[ ]  Flight Idle Stop CB .... CHECKED

Cleared to Start

[ ]  Windows .... CLOSED

[ ]  Door Lights .... OFF

[ ]  Rotating Beacon .... ON

[ ]  Fuel Pumps .... AS REQUIRED

[ ]  Radio Master .... OFF

After Start

[ ]  Electrical .... CHECKED / SET

[ ]  APU .... AS REQUIRED

[ ]  Fuel .... CHECHED / SET

After Start 2

[ ]  Auto Feather .... TESTED / ON

[ ]  Prop sync .... ON

[ ]  EEC .... CHECKED / TO or ON

[ ]  Hydraulic Power .... CHECKED / AUTO

[ ]  Ice/Rain Protection .... AS REQUIRED

[ ]  Pitot Static .... ON

[ ]  Air Conditioning .... SET

[ ]  Radio Master .... ON

Taxi

[ ]  Taxi Light .... ON

[ ]  Condition Levers .... FEATHER ONCE

[ ]  Take-off Data .... CHECKED

[ ]  Elev & Aileron Trim .... SET

[ ]  Take-off Briefing .... COMPLETED

[ ]  Flaps .... 15°

[ ]  Flight Controls .... CHECKED

[ ]  Cabin .... CLEAR

Cleared into Position

[ ]  Cab. ATT. Advisory .... GIVEN

[ ]  Landing Lights .... ON

[ ]  Strobe Lights .... ON

[ ]  Air Conditioning .... AS REQUIRED

[ ]  Transponder .... ALT

[ ]  Condition Levers .... MAX RPM

[ ]  Rudder Trim .... SET

After Take-off

[ ]  Gear .... UP - NO LIGHTS

[ ]  Flaps .... ZERO

[ ]  APU .... OFF or AS REQUIRED

[ ]  Auto Feather .... OFF

[ ]  Air Conditioning .... SET

[ ]  Altimeters .... 1013

[ ]  Pressurisation .... CHECKED

[ ]  Fasten Belts .... AS REQUIRED

[ ]  Landing Lights .... OFF

Descend

[ ]  W/S Heat .... AS REQUIRED

[ ]  W/S Defog .... ON

[ ]  Air Conditioning .... AS REQUIRED

[ ]  Landing Data .... CHECK

[ ]  Pressurization .... SET

[ ]  Approach Briefing .... COMPLETED

[ ]  Fasten Belts .... ON

[ ]  Landing Lights .... ON

Approach

[ ]  APU .... AS REQUIRED

[ ]  Autofeather .... ON

[ ]  Air Conditioning .... AS REQUIRED

[ ]  Altimeters .... SET

[ ]  Instruments & Radios .... SET

[ ]  Pressurization .... CHECKED

[ ]  Cab. Att. Advisory .... GIVEN

Before Landing

[ ]  W/S Heat .... AS REQUIRED

[ ]  Radar .... AS REQUIRED

[ ]  Gear .... DOWN - GREEN

[ ]  Flaps .... CHECKED - °/c Vref

[ ]  Condition Levers .... MAX RPM

[ ]  Yaw Damper .... OFF

[ ]  Cabin .... CLEAR

After Landing

[ ]  Landing Lights .... OFF

[ ]  Strobe Lights .... OFF

[ ]  Fuel Pumps .... AS REQUIRED

[ ]  Auto Feather .... OFF

[ ]  Prop sync .... OFF

[ ]  Ice/Rain Protection .... OFF

[ ]  Radar .... STANDBY

[ ]  Transponder .... STANDBY

[ ]  Trim Controls .... ZERO

[ ]  Pressurzation .... CHECKED

[ ]  Flaps .... ZERO

Shut-Down

[ ]  Inverters .... AS REQUIRED

[ ]  Main/Aux Generator .... OFF

[ ]  Fasten Belts .... OFF

[ ]  Emerg Lights .... AS REQUIRED

[ ]  External Lights .... AS REQUIRED

[ ]  Electric Hydraulic Pumps .... OFF

[ ]  Air Conditioning .... AS REQUIRED

[ ]  Stanby Horizon .... AS REQUIRED

[ ]  Back-up Battery .... AS REQUIRED

[ ]  Radio Master .... AS REQUIRED

Shut-Down 2

[ ]  Power Levers .... GRD IDLE

[ ]  Condition Levers .... FUEL CUT OFF

[ ]  Parking Brake .... ON

[ ]  PWR Select .... AS REQUIRED

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Embraer/Emb-120 Brasilia/MAX_Emb120_ Check.txt

Rights: licensed

Checklist hash: 093213185338e9696cc47f8f4772195d4cf81da17dc4ec59806432052db8dfe1

Verify against the aircraft's own approved documentation before use.