Fokker 100 — MAX_Fokker100-650_Check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "TAKEOFF" item count differs between passes (18 vs 19); verify nothing was dropped.

BEFORE START

[ ]  Seat Belt - No SMKG .... ON

[ ]  Fuel Quantity .... CHECKED

[ ]  Altimetars .... SET

[ ]  Ship's Papers .... ON BOARD

[ ]  FMP .... SET

[ ]  L-R FMS .... SET

[ ]  MFDS/TRP .... SET

[ ]  Nav/Com .... SET

[ ]  Parking Brake .... SET

[ ]  L-R Windows And Doors .... CLOSED

CLEARED FOR ENGINE START

[ ]  ENG Anti-Icing .... Check OFF

[ ]  Beacon .... ON

[ ]  Engine Panel .... SET

[ ]  Fuel Pumps .... ON

[ ]  Windows And Probe Heat .... ON

[ ]  Bleed Press .... CHECK

[ ]  Thrust Levers .... FWD IDLE

[ ]  Start Engines

AFTER START

[ ]  Anti-Icing .... SET

[ ]  Engine Panel .... CHECKED

[ ]  Alert Lights .... OUT

[ ]  Flaps .... SET FOR TAKEOFF

[ ]  Fuel Levers .... CHECKED OPEN

[ ]  Trims .... SET

[ ]  Door Selector .... AUTOMATIC

DURING TAXI/BEFORE TAKEOFF

[ ]  Flight Instruments .... CHECKED

[ ]  FMP .... SET

[ ]  ABS .... RTO

[ ]  Brake Temperatures .... CHECKED

[ ]  TO Data And Breifing .... COMPLETED

[ ]  Flight Controls .... CHECKED

[ ]  Radar And Transponder .... AS REQUIRED

CLEARED FOR TAKEOFF

[ ]  APU .... AS REQUIRED

[ ]  TO Config .... CHECKED

TAKEOFF

[ ]  Brakes .... RELEASE

[ ]  Thrust Levers .... ADVANCE SMOOTHLY TO 40% N1

[ ]  Thrust Levers .... ADVANCE SMOOTHLY TO 100% N1

--OR--

[ ]  TO/GA Mode

Engage

[ ]  Engine Instruments .... MONITOR

[ ]  Airspeed 80 KIAS .... CALLOUT "80 KNOTS"

[ ]  Airspeed V1 .... CALLOUT "V1"

[ ]  Airspeed VR .... CALLOUT "ROTATE"

[ ]  Airspeed V2 .... CALLOUT "V2"

[ ]  Landing Gear .... UP (WHEN POSITIVE CLIMB ESTABLISHED)

[ ]  Airspeed .... 250 KIAS AT 3,000' AGL

MAINTAIN V2+20 KIAS

[ ]  Autopilot .... ENGAGE AT 1,000' AGL

[ ]  Flaps .... UP AT 210 KIAS

[ ]  Autothrottle .... OFF

[ ]  Thrust Levers .... 90% N1

[ ]  Airspeed .... 250 KIAS AT 3,000' AGL

[ ]  Landing Gear Lever .... Verify OFF

AFTER TAKEOFF

[ ]  MFDS .... CHECKED

[ ]  Altimeters .... SET

CLIMB

[ ]  Airspeed .... MAINTAIN 250 KIAS

[ ]  Landing Lights .... OFF ABOVE 10,000' MSL

[ ]  Airspeed .... 280 KIAS ABOVE 10,000' MSL

[ ]  Seat Belt .... OFF ABOVE 10,000' MSL

CRUISE

[ ]  Thrust Levers .... AS DESIRED

[ ]  Trim .... AS NECESSARY

BEFORE APPROACH/DESCENT

[ ]  Seat Belt .... ON BELOW 10,000' MSL

[ ]  Landing Alt .... CHECKED

[ ]  Ignition .... AS REQUIRED

[ ]  ABS .... AS REQUIRED

[ ]  MFDS .... CHECKED

[ ]  Altimeters .... SET

[ ]  Landing Data/Approach Briefing .... COMPLETED

[ ]  Airspeed .... <250 KIAS BELOW 10,000' MSL

[ ]  Landing Lights .... ON BELOW 10,000' MSL

APPROACH

[ ]  Airspeed .... AS DESIRED

[ ]  Thrust Levers .... AS DESIRED

[ ]  Flaps .... AS DESIRED

[ ]  Autopilot .... AS DESIRED

LANDING

[ ]  Landing Gear .... DOWN

[ ]  Flaps .... SET

[ ]  Lift Dampers .... ARMED

NOTE: (use Shift+T)

LANDING ROLL

[ ]  Thrust Levers .... IDLE

[ ]  Autopilot .... CHECK DISENGAGED

[ ]  Autothrottle .... CHECK OFF

[ ]  Lift Dumpers .... CHECK FULL UP

[ ]  Brake .... AS REQUIRED

[ ]  Thrust Levers .... REVERSE

[ ]  Thrust Levers .... IDLE AT 60 KIAS

TAXI-IN

[ ]  Lift Dumpers .... DOWN

[ ]  Flap Lever .... UP

[ ]  Pitot Heat .... OFF

[ ]  Landing Lights .... AS DESIRED

[ ]  Flight Director .... OFF

[ ]  Transponder .... AS REQUIRED

PARKING

[ ]  Parking Brake .... AS REQUIRED

[ ]  Door Selector .... MANUAL

[ ]  APU .... AVAILABLE

[ ]  Speed Brake And Lift Dumpers .... IN

[ ]  Flaps .... SET

[ ]  RADAR .... CHECK OFF

[ ]  Anti-Icing .... OFF

[ ]  Fuel Levers .... SHUT

[ ]  Flight Controls .... LOCKED

[ ]  Seat Belt .... OFF

[ ]  Exterior Lights .... CHECK

[ ]  Fuel Pumps .... OFF

[ ]  Air Conditioning .... SET

[ ]  EFIS .... OFF

[ ]  MFDS .... CHECKED

FINAL STOP

[ ]  APU .... OFF

[ ]  BATTERIES .... OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Fokker/100/MAX_Fokker100-650_Check.txt

Rights: licensed

Checklist hash: 879fc8d50b38bc67aa38dd1133f3f5b3089412f1cabca24f4535c191aff46eeb

Verify against the aircraft's own approved documentation before use.