Fuji FA-200 — fuji_fa200_KARI_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Engine Start (COLD)" item count differs between passes (23 vs 17); verify nothing was dropped.

content_gap: Section "Normal Operating Speeds at MTOW 2335lb" item count differs between passes (39 vs 22); verify nothing was dropped.

Normal Operating Speeds at MTOW 2335lb

TAKE OFF 15° FLAP (NORMAL)

[ ]  VR .... 48 kts

CLIMB OUT CLEAN BEST RATE

[ ]  VY .... 71 kts

CRUISE %POWER

[ ]  Keep engine temp below 200°C .... 50-70

[ ]  Max rpm .... 2700

LANDING BASE LEG 15° FLAP

[ ]  Landing Base Leg 15° Flap .... 78 kts

FINAL 35° FLAP

[ ]  Final 35° Flap .... 70 kts

THRESHOLD

[ ]  Threshold .... 65 kts

MAX CROSSWIND

[ ]  Max Crosswind .... 15 kts

STALLING 35° FLAPS

[ ]  Stalling 35° Flaps .... 50 kts

25° FLAPS

[ ]  VS1 .... 52 kts

CLEAN

[ ]  VSO .... 59 kts

NOTE: Warner operates @ 4-9 kts > stall

NOTE: Caution - Stall Speed increases sharply with bank angle

25° FLAP STALL SPEED @ 45° BANK

[ ]  25° Flap stall speed @ 45° bank .... 63 kts

CLEAN STALL SPEED @ 45° BANK

[ ]  Clean stall speed @ 45° bank .... 70 kts

MINIMUM WINDMILL GLIDE

[ ]  Minimum Windmill Glide .... 82 kts

25° OR 35° FLAPS EXTENDED

[ ]  VFE .... 103 kts

DESIGN MANEUVERING SPEED

[ ]  VA .... 116 kts

15° FLAPS EXTENDED - WHITE ARC

[ ]  VFO .... 121 kts

NORMAL OPERATIONS – GREEN ARC

[ ]  VNO .... 128 kts

NEVER EXCEED – RED LINE

[ ]  VNE .... 145 kts

Engine Failure During Takeoff Run  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... APPLY

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

Engine Failure Immediately After Takeoff  [EMERGENCY]

[ ]  Airspeed .... MAINTAIN MIN 85 kts

[ ]  Fuel Selector Valve .... SWITCH TANKS

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Carburettor Heat .... ON

Engine Failure During Flight  [EMERGENCY]

[ ]  Airspeed .... MIN 85 kts

[ ]  Carburettor Heat .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Fuel Selector Valve .... SWITCH TANKS

[ ]  Mixture .... RICH

[ ]  Magnetos .... BOTH

[ ]  Primer .... IN & LOCKED

[ ]  Master Switch .... ON

NOTE: If engine failure was caused by fuel exhaustion, power will not be regained after tanks are switched until empty fuel lines are filled, which may require up to ten seconds.

Landing w/o Engine Power  [EMERGENCY]

[ ]  Airspeed .... MIN 85 kts ON FINALS

[ ]  Radio .... MAYDAY on 121.5 MHz

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Door .... OPEN

NOTE: Touchdown should be made at lowest possible airspeed with full flaps.

Electrical Fire in Flight  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Vents .... OPEN

[ ]  Cabin Heat and Air .... OFF

[ ]  All Electrics .... OFF

[ ]  Master Switch .... ON

[ ]  Switch on Essential Eqpt .... ONE BY ONE

[ ]  Fire .... CHECK AFTER EACH ITEM

[ ]  Land .... AS SOON AS POSSIBLE

Engine Fire in Flight  [EMERGENCY]

[ ]  Fuel Selector Valve .... OFF

[ ]  Throttle .... CLOSED

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Cabin Heat and Air .... OFF

[ ]  Defroster .... OFF

[ ]  Forced Landing .... EXECUTE

Engine Fire During Start On Ground  [EMERGENCY]

[ ]  Cranking .... CONTINUE

IF ENGINE STARTS:

[ ]  Power .... 1800 RPM for a few minutes

[ ]  Engine .... SHUTDOWN & INSPECT

[ ]  a. Fuel Pump .... OFF

[ ]  b. Fuel Selector .... OFF

[ ]  c. Master Switch .... OFF

[ ]  d. Magnetos .... OFF

IF ENGINE FAILS TO START:

[ ]  Passengers .... EVACUATE

[ ]  Engine .... SECURE

[ ]  a. Fuel Pump .... OFF

[ ]  b. Mixture .... IDLE CUT-OFF

[ ]  c. Master Switch .... OFF

[ ]  d. Magnetos .... OFF

[ ]  e. Fuel Selector Valve .... OFF

[ ]  Fire .... EXTINGUISH

Cockpit

[ ]  Tech Log + Weight / Balance .... CHECK

[ ]  Tacho Hours .... CHECK & RECORD

[ ]  Transparencies .... CLEAN

[ ]  Control Locks .... REMOVE

[ ]  Fire Extinguisher .... SECURE / GREEN ARC

[ ]  First Aid Kit .... PRESENT / SECURE

[ ]  Master Switch .... ON

[ ]  Fuel Quantity Gauges .... CHECK

[ ]  Anti-Collision Beacon .... CHECK

[ ]  Landing / Taxi Lights .... CHECK

[ ]  Nav Lights .... CHECK

[ ]  Stall Warner .... CHECK

[ ]  All Lighting Switches .... OFF

[ ]  Master Switch .... OFF

[ ]  Magnetos .... OFF, KEYS REMOVED

[ ]  Flaps .... 40°

Port Wing

[ ]  Fuel Tank .... DIP, CAP SECURE

[ ]  Tank Drain .... DRAIN, SECURE

[ ]  Fuel Sample .... PROPER COLOR

[ ]  Gear, Spat .... SECURE / UNDAMAGED

[ ]  Tyre .... INFLATED – MARK ALIGNED

[ ]  Chocks .... REMOVED

[ ]  Wing Tip & Light .... UNDAMAGED

[ ]  Tiedown .... REMOVED

[ ]  Pitot Mast .... CLEAR / UNDAMAGED

[ ]  Aileron .... FREEDOM OF MOVEMENT

[ ]  Flap .... SECURE & UNDAMAGED

Empennage

[ ]  Aerials .... SECURE

[ ]  Static Vent (each side) .... CLEAR

[ ]  Stabilators .... FREEDOM OF MOVEMENT

[ ]  Rudder .... VISUAL CHECK ONLY

[ ]  Anti-Balance / Trim .... SECURE, DAMAGE

[ ]  Tail Cone & Light .... SECURE, UNDAMAGED

[ ]  Tie Down .... REMOVED

Starboard Wing

[ ]  Flap .... SECURE / UNDAMAGED

[ ]  Aileron .... FREEDOM OF MOVEMENT

[ ]  Wing Tip & Light .... UNDAMAGED

[ ]  Tiedown .... REMOVED

[ ]  Fuel Tank .... DIP, CAP SECURE

[ ]  Tank Drain .... DRAIN, SECURE

[ ]  Fuel Sample .... PROPER COLOR

[ ]  Gear, Spat .... SECURE / UNDAMAGED

[ ]  Tyre .... INFLATED – MARK ALIGNED

[ ]  Chocks .... REMOVED

Nose

[ ]  Engine Oil Level .... 6 Qts. MIN, 8 Qts MAX

[ ]  Engine Oil Dipstick .... SECURE

[ ]  Cowling .... SECURE, UNDAMAGED

[ ]  Prop / Spinner .... SECURE, UNDAMAGED

[ ]  Alternator Belt .... PROPER TENSION

[ ]  Nose Oleo .... NO LEAKS / NORMAL

[ ]  Spat / Tyre .... CONDITION / DAMAGE

[ ]  Tow Bar .... REMOVED & STOWED

[ ]  Chocks .... REMOVED

[ ]  Fuel Sump Drain .... DRAIN, SECURE

Before Starting Engine

[ ]  Preflight Inspection .... COMPLETE

[ ]  Flaps .... UP IN STAGES

[ ]  Canopy .... CLOSED / LOCKED

[ ]  Controls .... FULL & FREE MOVEMENT

[ ]  Elevator Trim .... FULL TRAVEL – NEUTRAL

[ ]  Fuel Selector Valve .... EMPTIEST TANK

[ ]  Seats, Harnesses .... ADJUSTED, LOCKED

[ ]  Radio, Electrical Equipment .... OFF

[ ]  Parking Brake .... SET

[ ]  Circuit Breakers .... CHECK

[ ]  Anti-Collision Beacon .... ON

Engine Start (COLD)

[ ]  Master Switch .... ON

[ ]  Mixture .... FULL RICH

[ ]  Fuel Pump .... ON

[ ]  Fuel Pressure .... CHECK

[ ]  Carburettor Heat .... FREE MOVEMENT - COLD

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Propeller .... CLEAR

[ ]  Magnetos .... START

[ ]  Starter Engaged Light .... OUT

[ ]  Oil Pressure .... CHECK 25 psi MIN

[ ]  Engine .... WARM UP 1200 RPM

[ ]  Brakes .... HOLDING

[ ]  Low Volts Light .... OUT

[ ]  Ammeter .... CHARGING

[ ]  Suction .... REGISTERING

[ ]  Radios .... COM 1 ON

[ ]  NAV 1 .... ON

[ ]  COM 2 .... ON

[ ]  DME .... ON

[ ]  Transponder .... STANDBY

[ ]  NAV 2 .... ON

[ ]  GPS .... ON

Before Taxi

[ ]  Radios .... CHECK VOLUME

[ ]  Gyros .... ERECTED

[ ]  DI .... ALIGN WITH COMPASS

[ ]  Altimeters .... SET QFE / QNH

[ ]  Nav Lights .... AS REQUIRED

[ ]  Taxi Light .... ON

[ ]  Radio Check / Taxi Clearance .... OBTAIN

[ ]  Parking Brake .... OFF

Taxi Checks

[ ]  Brakes / Nosewheel Steering .... TEST

[ ]  Compass .... SWINGING FREELY

[ ]  DI .... TURNING CORRECTLY

[ ]  AI .... MOVING CORRECTLY

[ ]  Turn / Slip .... MOVING CORRECTLY

[ ]  ASI .... REGISTERING

[ ]  Nav Aids .... REGISTERING

Power Checks

[ ]  Parking Brake .... SET

[ ]  Fuel Pump .... ON

[ ]  Fuel Pressure .... GREEN ARC

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Fuel Pump .... OFF

[ ]  Fuel Pressure .... GREEN ARC

[ ]  Oil Temperature .... YELLOW ARC MIN

[ ]  Oil Pressure .... CHECK 25 psi MIN

[ ]  Area Behind .... CLEAR

[ ]  Throttle .... GENTLY INCREASE - 2000 RPM

[ ]  Carburettor Heat .... HOT, CHECK RPM, COLD

[ ]  Magnetos .... CHECK (150 RPM Max drop, 50 RPM diff.)

[ ]  Suction .... IN WHITE ARC

[ ]  Oil Temperature .... YELLOW ARC MIN

[ ]  Oil Pressure .... CHECK 25 psi MIN

[ ]  Throttle - Idle .... 500 – 700 RPM

[ ]  Throttle .... 1200 RPM SET

Pre-Takeoff

[ ]  Elevator Trim .... SET NEUTRAL

[ ]  Flaps .... 15° (NORMAL)

[ ]  Fuel Contents .... SUFFICIENT

[ ]  Carburettor Heat .... COLD

[ ]  Primer .... LOCKED

[ ]  Throttle Friction .... FINGER TIGHT

[ ]  Mixture .... FULL RICH

[ ]  Magnetos .... BOTH

[ ]  DI .... ALIGN WITH COMPASS

[ ]  Altimeters .... SET QFE / QNH

[ ]  Radio Aids .... TUNE / IDENT

[ ]  Engine Temps & Pressure .... NORMAL

[ ]  Fuel Pump .... ON

[ ]  Pitot Heater .... AS REQUIRED

[ ]  Nav Lights .... AS REQUIRED

[ ]  Flight Controls .... FULL & FREE

[ ]  Hatches and Harnesses .... SECURE

[ ]  Parking Brake .... OFF

[ ]  Transponder .... ON-ALT

[ ]  Departure Clearance .... OBTAIN

Normal Takeoff

[ ]  Flaps .... 15° (NORMAL)

[ ]  Carburettor Heat .... COLD

[ ]  Fuel Pump .... ON

[ ]  DI .... ALIGNED WITH RUNWAY

[ ]  Engine Temps & Pressure .... NORMAL

[ ]  Throttle .... FULL OPEN

[ ]  Rotate .... 55 kts

[ ]  Climb Speed .... 71 kts

[ ]  Flaps .... RETRACT 300ft MIN

[ ]  Top of Climb .... FREDAL CHECKS

Short Field / Obstacle Clearance

[ ]  Flaps .... 25° (Second Notch)

[ ]  Carburetor Heat .... COLD

[ ]  Fuel Pump .... ON

[ ]  Throttle .... FULL OPEN

[ ]  Rotate .... 55-60 kts

[ ]  Initial Climb Speed .... 71 kts

Climb

[ ]  Normal Climb .... 75 kts

[ ]  VY – Best Rate of Climb Speed .... 71 kts

[ ]  VX – Best Angle of Climb Speed .... 65 kts

Cruise

[ ]  Fuel Pump .... OFF

[ ]  Power .... 2200-2600 RPM

[ ]  Elevator Trim .... SET

[ ]  Taxi Light .... OFF

[ ]  Mixture .... LEAN (MAX 200°C EGT)

[ ]  Fuel Selector .... SWITCH AS NEEDED

Before Landing

[ ]  Parking Brake .... OFF

[ ]  Mixture .... FULL RICH

[ ]  Magnetos .... BOTH

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Engine Temps & Pressure .... NORMAL

[ ]  Carburettor Heat .... HOT

[ ]  Seats / Harnesses .... ADJUSTED / LOCKED

[ ]  Airspeed .... IN WHITE ARC

[ ]  Flaps .... AS REQUIRED

[ ]  Airspeed .... 80 kts

[ ]  Elevator Trim .... SET

[ ]  Landing Light .... ON AS REQUIRED

Finals Checks

[ ]  Carburettor Heat .... COLD

[ ]  Runway .... CLEAR

[ ]  Approach .... SPEED & ANGLE OK

[ ]  Permissions .... LANDING CLEARANCE

After Landing

[ ]  Flaps .... UP

[ ]  Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Carburettor Heat .... OFF

[ ]  Transponder .... OFF

Shut-Down, Securing Aeroplane

[ ]  Magnetos .... CHECK LEFT / RIGHT

[ ]  Electrical Equipment, Lights .... OFF

[ ]  COM 1/2, NAV 1/2, DME, Xpdr, GPS .... OFF

[ ]  Throttle .... IDLE

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition .... OFF

[ ]  Master Switch .... OFF

[ ]  Parking Brake .... SET (IF REQUIRED)

[ ]  Chocks/Tiedowns .... INSTALLED

Specifications

[ ]  Maximum Gross Weight .... 2335 lbs

[ ]  Basic Empty Weight .... 1575 lbs

[ ]  Payload With Full Fuel .... 448 lbs

[ ]  Fuel Flow @ 75% Power .... 8 gph

[ ]  Total Fuel Capacity .... 52 gal

[ ]  Fuel Octane .... 100LL (Blue)

[ ]  Oil Sump Capacity .... 8 qts

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Fuji/FA-200/fuji_fa200_KARI_Checklist.txt

Rights: licensed

Checklist hash: 7cf7a72688a320a412e912f1a7f2bdc9701e6a4d3ceb1bf6b46fe7cabdfe0bb1

Verify against the aircraft's own approved documentation before use.