Lancair 320 — L320-3 - Climb Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Climb Checklist

[ ]  Power .... 25"Hg, 2,500 RPM

[ ]  Flaps & Gear .... CHECK

NOTE: Cruise Climb 125 KIAS. Check power in climb, adjust Mixture for 10.0 GPH hot days, 9.5 GPH cool days. Max 1400( EGT.

Cruise Checklist

[ ]  Power .... 22"Hg, 2,400 RPM

[ ]  Mixture .... ADJUST

[ ]  Fuel .... TIME & MANAGE

NOTE: At 8,500 MSL, normal EGT = 1400(, FF = 7.6 GPH.

Descent Checklist

[ ]  Power .... 14"Hg, 2,300 RPM

[ ]  Mixture .... INCR. AS REQUIRED

[ ]  Approach Briefing .... COMPLETE

NOTE: Descent Planning: 4 to 1, + 5 Miles to decelerate.

Before Landing Checklist

[ ]  Mixture .... FULL RICH

[ ]  Prop .... HIGH RPM

[ ]  Gear [120 KIAS] .... DOWN, 3 GREEN

[ ]  Flaps [100 KIAS] .... AS REQUIRED

[ ]  Landing Clearance .... RECEIVED

NOTE: Approach Speed 75 - 80 KIAS Full Flaps. MFA Additive:  ½ Headwind + All Gusts Up To 90 KIAS.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Lancair/320/L320-3 - Climb Checklist.txt

Rights: licensed

Checklist hash: 55d6129ce6f9400ff3933d57cd1e8c919feee1e7bbdbf436b078dc3a0b1da599

Verify against the aircraft's own approved documentation before use.