Lancair 320 — L320-5 - Emergency Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Engine Out Checklist  [EMERGENCY]

[ ]  MAX GLIDE .... 104 KIAS

[ ]  Flaps and Gear .... UP

[ ]  Landing Area .... DETERMINE

Engine Restart  [EMERGENCY]

[ ]  Magneto Switches .... ON

[ ]  Master Switch .... ON

[ ]  Aux Fuel Pump .... ON

[ ]  Prop Lever .... HIGH RPM

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... ½” – 1” OPEN

[ ]  Start Button .... PRESS

Engine Secure / Landing  [EMERGENCY]

[ ]  Electrical Switches .... OFF

[ ]  Magneto Switches .... OFF

[ ]  Master Switches (3) .... OFF

[ ]  Mixture .... CUTOFF

[ ]  Canopy Latches .... UNLATCH

[ ]  Gear and Flaps (When Landing Assured) .... AS REQUIRED

[ ]  SPEED .... 70 KIAS MIN.

Emergency Gear Extension  [EMERGENCY]

[ ]  SPEED .... 90 KIAS

[ ]  Gear Circuit Breaker .... PULL

[ ]  Gear Handle .... DOWN

[ ]  Hydraulic Valve (Pilot’s Right Knee) .... MOVE DOWN TO VERTICAL POSITION

[ ]  Gear Lights .... ON, 3 GREEN

[ ]  Hydraulic Valve .... MOVE UP TO HORIZONTAL POSITION

[ ]  Gear Circuit Breaker .... PUSH IN

NOTE: If Main Gear fails to lock, yaw aircraft in direction of unlocked gear. If Nose Gear fails to lock, reduce speed to 80 - 85 KIAS, exert positive G forces until all Gear Lights illuminate.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Lancair/320/L320-5 - Emergency Checklist.txt

Rights: licensed

Checklist hash: 39ddd235f06611bb8d7450f77d61cf59b408a6ebc77f46be2c8ea5b4aab99933

Verify against the aircraft's own approved documentation before use.