Lancair 350 — WM_Columbia_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Speeds" item count differs between passes (6 vs 5); verify nothing was dropped.

Starting Engine

[ ]  Mixture .... FULL RICH

[ ]  Prop .... FULL POWER

[ ]  Vapor Suppression .... OFF

[ ]  Induction Heated Air .... OFF

[ ]  Strobe Lights .... ON

[ ]  Throttle .... OPEN 1”

[ ]  Crosstie .... OFF

[ ]  Bus Switches .... ON

[ ]  Primer .... PRESS

[ ]  Throttle .... OPEN 1/8”

[ ]  Prop Area .... CLEAR

[ ]  Ignition .... ENGAGE

[ ]  Oil Pressure

[ ]  Engine Instruments

[ ]  Ammeter .... SELECT AMM

[ ]  Avionics Master .... ON

Before Taxi

[ ]  Crosstie Operation .... VERIFY

[ ]  Speedbrakes .... TEST/STOW

[ ]  Flaps .... TAKEOFF

[ ]  Flight Controls .... FREE/CLEAR

[ ]  Trim .... CENTERED

[ ]  Radios .... ON/SET

[ ]  IFR Clearance .... AS REQ’D

[ ]  GPS Flight Plan .... INPUT

[ ]  Transponder .... SET CODE

[ ]  Autopilot Master .... AP/FD

[ ]  Navaids/CDI .... SET/CHECK

[ ]  Headings .... VERIFY

[ ]  Heading/Altitude/VSI Bugs .... SET

[ ]  Autopilot Test .... COMPLETE

[ ]  Fuel Gauges/ .... VERIFY/CHECK LIGHT

[ ]  Fuel Selector .... TEST/SET FULLEST

[ ]  Chart Page .... SELECT DEPARTURE

[ ]  Map Page .... SET 5-20NM

[ ]  Engine Page .... SELECT/MONITOR

[ ]  Engine Instruments

[ ]  Annunciator Panel/CO Alert .... TEST/CLEAR

[ ]  Ammeters .... CHECK

[ ]  Lights .... AS REQ’D

Before Takeoff

[ ]  Radios .... TWR/DEPX

[ ]  Doors .... CLOSE/LATCH/UNLOCKED

[ ]  Crosstie .... OFF

[ ]  Engine Runup .... 1700 RPM

[ ]  Magnetos .... CHECK 25-150 RPM DROP

[ ]  Prop .... CHECK

[ ]  Fuel Selector .... CHECK DETENT/WARN

[ ]  Flight Controls .... FREE/CLEAR

[ ]  Annunciator Panel .... TEST/CLEAR

[ ]  Door Seals .... ON

[ ]  Brakes .... OFF

Crossing Hold Short

[ ]  Lights .... AS REQUIRED

[ ]  Pitot Heat .... AS REQUIRED

[ ]  Boost Pump .... ARMED

[ ]  Flaps .... TAKEOFF

[ ]  Speedbrakes .... STOWED

[ ]  Trim .... CENTERED

After Takeoff – 500 AGL

[ ]  Flaps .... UP

[ ]  Prop .... 2500-2700 RPM

[ ]  Landing Light .... OFF

[ ]  Engine Page .... NORMALIZE

[ ]  Map Page .... SELECT

Climb – 5000 AGL

[ ]  Boost Pump .... DEARMED

[ ]  Lights .... AS REQ’D

[ ]  Mixture .... LEAN AS REQ’D

[ ]  Pitot Heat .... AS REQ’D

[ ]  Deice .... AS REQ’D

[ ]  Fuel Selector .... SWITCH TANKS

Cruise

[ ]  Boost Pump .... CHECK DEARMED

[ ]  Throttle .... AS REQ’D

[ ]  Prop .... AS REQ’D

[ ]  Mixture .... LEAN ASSIST

Speeds

NOTE: Best Angle (Vx): 80 KIAS (SL)/84 KIAS (10k MSL)

NOTE: Best Rate (Vy): 106 KIAS (SL)/93 KIAS (10k MSL)

NOTE: Maneuver (Va): 148 KIAS (3400#)/127 KIAS (2500#)

NOTE: Approach (VRef): 90 KIAS

NOTE: Stall Flaps Up (VS0): 71 KIAS

NOTE: Stall Flaps Land (VS1): 57 KIAS

Descent

[ ]  ATIS .... OBTAIN

[ ]  Altimeter .... SET

[ ]  Fuel Tank .... SELECT

[ ]  Power Settings .... ADJUST

[ ]  Engine Page .... NORMALIZE

[ ]  Mixture .... ADJUST

[ ]  Chart Page .... SELECT APPROACH

Approach

[ ]  GPS Approach .... VERIFY PROGRAMMED

[ ]  Navaids .... TUNE/VERIFY

[ ]  CDI/NAV2 .... SELECT/VERIFY

[ ]  Radios .... SET

[ ]  GPS Approach .... ACTIVATE WHEN NEEDED

[ ]  Landing Light .... ON

[ ]  Fuel Tank .... SELECT

[ ]  Boost Pump .... CHECK DEARMED

[ ]  Prop .... ADJUST

[ ]  Mixture .... ADJUST

[ ]  Seat Belts .... FASTENED

Landing

[ ]  Fuel Tank .... SELECT

[ ]  Mixture .... RICH

[ ]  Prop .... MAX RPM

[ ]  Speedbrakes .... STOWED

[ ]  Flaps .... T/O or LAND

Go-Around/Missed Approach

[ ]  Throttle .... MAX POWER

[ ]  Speedbrakes .... VERIFY STOWED

[ ]  Flaps .... SET TAKEOFF

[ ]  Positive Rate of Climb .... ESTABLISH

[ ]  Flaps .... UP

[ ]  Chart Page .... SELECT

[ ]  GPS OBS Button .... PRESS/RELEASE

[ ]  Missed Approach Instructions .... FOLLOW

Clearing Runway

[ ]  Lights .... OFF/AS REQ’D

[ ]  Flaps .... UP

[ ]  Speedbrakes .... STOW

[ ]  Door Seal .... OFF

[ ]  Pitot Heat/Deice .... OFF

[ ]  Autopilot Master .... OFF

[ ]  Time .... NOTE

EMERGENCIES — Speeds  [EMERGENCY]

NOTE: Engine Out (Flaps 0): 106 KIAS

NOTE: Engine Out (Flaps 10): 93 KIAS

NOTE: Precautionary Landing (Flaps 40): 78 KIAS

NOTE: Power-off Approach (Flaps 0): 106 KIAS

NOTE: Power-off Approach (Flaps 40): 90 KIAS

Engine Failure Below 400 AGL  [EMERGENCY]

[ ]  Airspeed .... 90 KIAS

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Ignition .... OFF

[ ]  Flaps .... LAND

[ ]  Master Switches .... OFF

Engine Failure on Takeoff Above 400 AGL  [EMERGENCY]

[ ]  Airspeed .... 106 KIAS

[ ]  Flaps .... UP

[ ]  Mixture/Throttle .... RICH/FULL OPEN

[ ]  Boost Pump .... ARMED

Engine Failure in Flight (Restart)  [EMERGENCY]

[ ]  Airspeed .... 106 KIAS

[ ]  Landing Site .... SELECT

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Mixture/Throttle .... RICH/FULL OPEN

[ ]  Boost Pump .... ARMED

[ ]  Ignition .... BOTH

Engine Fire in Flight  [EMERGENCY]

[ ]  Throttle .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Heating/Ventilation System .... OFF

[ ]  Master Switches .... OFF

[ ]  Nav/Comm Bypass .... ON

[ ]  Airspeed .... 179 KIAS to Extinguish

[ ]  Landing .... POWER-OFF LANDING

Electrical Fire in Flight  [EMERGENCY]

[ ]  Heating/Ventilation Controls .... FULL OPEN

[ ]  Master Switches .... OFF

[ ]  Oxygen .... OFF

[ ]  Avionics Master .... OFF

[ ]  Trim System Switch .... OFF

[ ]  Nav/Comm Bypass .... ON

[ ]  Book Procedures .... PERFORM IF ABLE

[ ]  Landing .... PRECAUTIONARY LANDING

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Lancair/350/WM_Columbia_Checklist.txt

Rights: licensed

Checklist hash: 60ff6250bf729b2c10e18eea05115a359abe8b80101c56dda5f7f0eb9f101dd2

Verify against the aircraft's own approved documentation before use.