Liberty Aerospace XL2 — LibertyXL2_Checklist_Version2_dogsaviation

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

START

[ ]  Fuel Selector .... ON

[ ]  Parking Brake .... SET

[ ]  Alternate Air .... OFF

[ ]  Fuel Boost Pump .... .AUTO

[ ]  Master Switch (BAT & ALT) .... ON

[ ]  Annunciator Lights .... TEST/HIGH

[ ]  VM1000 .... VOLTS > 11.4v

[ ]  FADEC A & B .... ON

[ ]  Ignition Key .... (L), boost pump on

[ ]  HSA .... .TEST/HIGH

[ ]  Throttle .... CHECK WOT, ¼”

[ ]  Ignition Key .... START

ENGINE FIRE DURING START  [EMERGENCY]

Engine fire during start

[ ]  1. Ignition Switch [MEMORY] .... START

NOTE: Continue turning the engine over.

NOTE: If Engine Starts:

[ ]  2. Power .... .1700 RPM (2 min.)

[ ]  3. Engine .... SHUT DOWN

NOTE: If Engine does NOT Start:

[ ]  4. Ignition Switch .... START

[ ]  5. Throttle .... .FULL FORWARD

[ ]  6. FADEC A and B PWR Switches .... .OFF

[ ]  7. Fuel Selector Valve .... OFF

[ ]  8. Cranking .... TERMINATE

[ ]  9. Ignition Switch .... .OFF

[ ]  10. Master Switch .... .OFF

[ ]  11. Airplane .... .EVACUATE

[ ]  12. Fire .... .EXTINGUISH

[ ]  13. Fire Damage .... .INSPECT

AFTER START

[ ]  Throttle .... 800-1000RPM

[ ]  VM 1000 .... .IN LIMITS

[ ]  Cockpit Rheostat switches .... SET

[ ]  Lights .... AS REQUIRED

[ ]  Avionics Master .... ON

[ ]  Flaps .... 0◦

[ ]  Transponder .... STBY

[ ]  Flight Instruments .... .CHECK/SET

[ ]  Comm/Nav Aids .... CHECK/SET

[ ]  Fuel Boost Pump .... AS REQUIRED

RUN-UP

[ ]  Fuel Boost Pump .... AUTO

[ ]  Fuel Selector .... ON

[ ]  Parking Brake .... SET

[ ]  Canopy Doors .... CLOSED/SECURE

[ ]  Throttle .... 1700RPM

[ ]  Alternate Air .... CHECK

[ ]  Fuel Boost Pump .... ON/OFF/AUTO

[ ]  Annunciator Lights .... TEST/HIGH

[ ]  HSA .... TEST/HIGH

[ ]  VM1000 .... IN LIMITS

[ ]  FADEC Power B .... CHECK

[ ]  FADEC Power A .... CHECK

[ ]  Ignition L/R .... DROP 10-150 RPM

CAUTION: FADEC CAUTION

[ ]  Throttle .... IDLE

BEFORE TAKEOFF

[ ]  Seatbelts/Harnesses .... FASTENED

[ ]  Fuel Quantity .... .VERIFY

[ ]  Flight Controls .... FREE /CORRECT

[ ]  Flight Instruments .... CHECK/SET

[ ]  Nav/Comm .... CHECK/SET

[ ]  Flaps .... 20°

[ ]  Trim .... SET for TAKEOFF

[ ]  Transponder .... ALT

[ ]  Parking Brake .... RELEASE

AFTER LANDING

[ ]  Flaps .... RETRACT

[ ]  Transponder .... STANDBY

[ ]  Lights .... .AS REQUIRED

SHUTDOWN

[ ]  Avionics Master .... .OFF

[ ]  Ignition Key .... OFF

[ ]  FADEC A & B .... .OFF

[ ]  All Light Switches .... OFF

[ ]  All Electrical Switches .... .OFF

[ ]  Master Switch .... OFF

[ ]  Parking Brake .... .AS REQUIRED

SPEEDS AND APPROACH

NOTE: Vr :55; Vx: 70 ; Vy: 80; Best Glide: 80

NOTE: Approach: Flaps 30: 65-70;

NOTE:       Flaps 20: 70-75; Flaps Up: 80-85

NOTE: Va 100 KIAS (1653 lbs) / 94 (1450 lbs)

ENGINE FAILURE AFTER TAKEOFF  [EMERGENCY]

Engine failure after takeoff

[ ]  1. Best Glide or Landing Speed (as appropriate) [MEMORY] .... ESTABLISH

[ ]  2. Flaps [MEMORY] .... AS REQUIRED

NOTE: If time permits:

[ ]  3. Ignition Switch .... OFF

[ ]  4. FADEC A and B PWR Switches .... OFF

[ ]  5. Master Switch .... OFF

[ ]  6. Throttle .... IDLE

[ ]  7. Fuel Selector Valve .... OFF

[ ]  8. Seat Belts/Shoulder Harness .... .SECURE

FORCED LANDING  [EMERGENCY]

Forced landing

[ ]  1. Establish Best Glide Attitude [MEMORY] .... 80 KIAS

[ ]  2. Radio .... MAYDAY 121.5 MHz)

[ ]  3. Transponder .... .SQUAWK 7700

[ ]  4. ELT (if off airport) .... ACTIVATE

[ ]  5. Ignition Switch .... OFF

[ ]  6. FADEC A and B PWR Switches .... OFF

[ ]  7. Fuel Boost Pump Mode Switch .... OFF

[ ]  8. Throttle .... IDLE

[ ]  9. Fuel Selector Valve .... OFF

[ ]  10. Flaps (when field is made) .... . AS REQUIRED or 30°

[ ]  11. Master Switch .... OFF

[ ]  12. Belts/ Harness .... SECURE

WARNING: WARNING: Flaps will not operate when Master Switch is off.

ENGINE FAILURE IN FLIGHT  [EMERGENCY]

Engine failure in flight

[ ]  1. Best Glide Attitude [MEMORY] .... 80 KIAS

[ ]  2. Fuel Selector Valve .... CHECK ON

[ ]  3. Throttle .... OPEN ¾ INCH

[ ]  4. Fuel Boost Pump .... ON

[ ]  5. FADEC A and B PWR Switches .... . RECYCLE SIMULTANIOUSLY

[ ]  6. Ignition Switch .... CHECK BOTH

[ ]  7. Ignition Switch (NOT Windmilling) .... START

[ ]  8. Throttle .... BEST OPERATION

[ ]  9. Repeat steps 2 – 6 prop windmilling.

[ ]  10. No Start .... Forced Landing checklist.

ENGINE PARTIAL POWER LOSS  [ABNORMAL]

Engine partial power loss

[ ]  1. Fuel Boost Pump Mode Switch .... ON

[ ]  2. Fuel Selector Valve .... CHECK ON

[ ]  3. Alternate Induction Air .... PULL ON

[ ]  4. Ignition Switch .... .CHECK R, L, BOTH

[ ]  5. LAND .... AS SOON AS PRACTICAL

ENGINE FIRE IN FLIGHT  [EMERGENCY]

Engine fire in flight

[ ]  1. Fuel Selector Valve [MEMORY] .... .OFF (lift knob to turn off)

[ ]  2. Throttle .... IDLE

[ ]  3. Cabin Heat .... OFF

[ ]  4. Fuel Boost Pump Mode Switch .... OFF

[ ]  5. FADEC A & B PWR Switches .... OFF

[ ]  6. Ignition Switch .... OFF

[ ]  7. Perform Forced Landing checklist

WARNING: WARNING : No restart.

CABIN FIRE IN FLIGHT  [EMERGENCY]

Cabin fire in flight

[ ]  1. Master Switch [MEMORY] .... .OFF

[ ]  2. FADEC PWR A Switch .... .OFF

[ ]  3. Avionics Master Switch .... OFF

[ ]  4. All Electrical Switches .... OFF (Except FADEC PWR B Switch)

WARNING: WARNING: Turning off FADEC PWR B switch when Master Switch is OFF will cause immediate loss of engine power.

[ ]  5. Cabin Heat .... OFF

[ ]  6. Fire Extinguisher .... ACTIVATE

[ ]  7. Air Vents .... OPEN TO VENT CABIN

[ ]  8. LAND .... AS SOON AS PRACTICAL

SPIN RECOVERY  [EMERGENCY]

Spin

[ ]  1. Throttle [MEMORY] .... IDLE

[ ]  2. Ailerons .... NEUTRAL

[ ]  3. Rudder .... FULL OPPOSITE DIRECTION OF SPIN

[ ]  4. Control Stick .... FORWARD

[ ]  5. Rudder .... Neutralize

[ ]  6. Recover from dive

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Liberty Aerospace/XL2/LibertyXL2_Checklist_Version2_dogsaviation.txt

Rights: licensed

Checklist hash: a9646431d6d3c1e3b5c7ffbc21f781f19feee4ff7a6d98980a184d377ac385f7

Verify against the aircraft's own approved documentation before use.