Mooney M20C — doug_sperber_M20C

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Engine Start" item count differs between passes (25 vs 22); verify nothing was dropped.

Engine Start

[ ]  Tach Time

[ ]  Step .... Raise

[ ]  Seatbelts .... Fastened

[ ]  Circuit Breakers .... All checked in

[ ]  Comm. .... ATIS and Clearance obtained

[ ]  Electrical .... switches, generator and radio master off (exc. fuel and beacon)

[ ]  Gas selector .... Desired tank

[ ]  Flaps .... Up and confirm visually

[ ]  Carb Heat .... Off

[ ]  Cowl Flaps .... Open

[ ]  Mixture .... Rich

[ ]  Prop .... High RPM

[ ]  Master Switch .... On

[ ]  Boost Pump .... On (Check Fuel Pressure), then Off

[ ]  Throttle .... Pump 1 to 3 times, then 1/4” open

[ ]  Beacon or Strobes .... On (Nav at night)

[ ]  Brakes .... Set

[ ]  Area & Prop .... “Clear”

[ ]  Starter .... Engage

[ ]  Throttle .... 1000 RPM

[ ]  Engine Instruments .... Check (Oil Pressure & Temp in green)

[ ]  Mixture .... Leaned for Taxi

[ ]  Electrical .... On (Avionics, Transponder, Etc.)

[ ]  Transponder .... Standby

[ ]  Parking Brake .... Release if engaged, Check Brakes before Taxi

Engine Run-Up

[ ]  Area .... Clear

[ ]  Boost Pump .... On (Check Fuel Pressure)

[ ]  Brakes .... Set

[ ]  Throttle .... 2000 RPM for 30 seconds, Lean to burn carbon deposits

[ ]  Prop .... Cycle 2-3 times (Check for RPM and Oil Pressure drop)

[ ]  Throttle .... Back to 1700 RPM

[ ]  Boost pump .... Off (Check Fuel Pressure)

[ ]  Engine Instruments .... Check (Oil Pressure & Temp in green)

[ ]  Magnetos .... Check (125 Max. Drop, 50 Max. Diff.)

[ ]  Carb Heat .... On, Check for RPM Drop, then Off

[ ]  Throttle .... Check idle with Carb Heat, then 1000RPM

[ ]  Parking Brake .... Release if engaged

NOTE: Perform Pre-Takeoff Checklist

Pre-Takeoff

[ ]  Instruments .... Check/Set (Altimeter, HI, AI, T&B, VSI, ASI)

[ ]  Gear / Annunciator Lights .... Test

[ ]  Gas Selector .... to desired tank for takeoff

[ ]  Controls .... Free and correct

[ ]  Trim .... Set for takeoff

[ ]  Flaps .... Set for Takeoff (2 Pumps)

[ ]  Gear Handle Area .... clear and un-obstructed

[ ]  Comm. and Nav Radios .... Set for departure

[ ]  Transponder .... Set and Standby

[ ]  Seatbelts .... Fastened

[ ]  Door .... Closed and locked (lever forward) - Window closed

Takeoff (Cleared for Takeoff)

[ ]  “LIGHTS” - Strobe Light .... On

[ ]  “CAMERA” - Landing Light .... On

[ ]  “ACTION” - Transponder .... On ALT

[ ]  Boost Pump .... On

[ ]  Mixture .... RICH

[ ]  Prop .... HIGH RPM

[ ]  Doors, Windows, Seatbelts .... Secure

[ ]  Time .... Time

Climb-out

[ ]  Climb .... Positive rate

[ ]  Gear .... Up at 80 MIAS (Apply brakes to stop tire rotation)

[ ]  Flaps .... Up at 90 MIAS

[ ]  Airspeed .... 105-120 MIAS for climb-out

[ ]  Power .... MP 25”, RPM 2500 for Climb

[ ]  Boost Pump .... Off (Check fuel pressure)

[ ]  Landing Light .... Off

[ ]  Engine Gauges .... Check (Oil Pressure and Temp in green)

[ ]  Mixture .... Lean as neccessary

[ ]  Cowl Flaps .... Close when transitioning to cruise

Approach to Landing

[ ]  Cowl Flaps .... Closed

[ ]  Boost Pump .... On

[ ]  Gas Selector .... Fullest tank

[ ]  Mixture .... Set as desired

[ ]  Seatbelts .... Fastened

[ ]  Brakes .... Check for Pressure

[ ]  Landing Light .... On

[ ]  Engine Guages .... Check

[ ]  Carb Heat .... On

[ ]  Gear .... Down when less than 120 MIAS

[ ]  GUMPS Check (Approach to Land, Downwind, Base, and Final)

Post Landing (When clear of the runway)

[ ]  Transponder .... Standby

[ ]  Boost Pump .... Off

[ ]  Landing Light .... Off

[ ]  Flaps .... Up

[ ]  Cowl Flaps .... Open

[ ]  Mixture .... Lean for Taxi

Engine Shut-down

[ ]  Switches .... Off (Avionics, Lights)

[ ]  Throttle .... 1000 RPM

[ ]  Magnetos .... Grounding Check (Off, then BOTH)

[ ]  Mixture .... LEAN

[ ]  Throttle .... Full Idle (When engine starts to stop)

[ ]  Master Switch .... Off

[ ]  Ignition .... Off and Key out

Preflight

[ ]  clean windscreen with vertical strokes only

[ ]  1.mags off

[ ]  2.right Tank - fuel level and fuel drain

[ ]  3.left Tank - fuel level and fuel drain

[ ]  4.Pitot Tube - clear

[ ]  5.left Control Surfaces - attach bolts secure.

[ ]  6.left Brake and Tyre

[ ]  7.Tailplane Control Surfaces - attach bolts & "bungees" secure.

[ ]  8.Static Ports - clear

[ ]  9.Flap Attach Bolts - view both flaps from wing root

[ ]  10.right Control Surfaces - attach bolts secure

[ ]  11.right Brake and Tyre

[ ]  12.front Tyre

[ ]  13.Prop - undamaged, cowling free of debris first flight of day

[ ]  14.Oil aeroshell - look behind rear seats - 6 qt. min.

[ ]  15.Fuel - water contamination - where appropriate, check cowl strainer last

V-Speeds (MPH IAS)

[ ]  Vs: 67

[ ]  Vs0: 57

[ ]  Vlo/le: 120

[ ]  Vfe: 100

[ ]  Vno: 150

[ ]  Vm: 132

[ ]  Vne: 189

[ ]  Vy (Used): 105

[ ]  Vy (charts): 80-103

[ ]  Vx: 80

[ ]  Rotate: 65-75

[ ]  Cruise Climb: 115-120

[ ]  Approach: 90, 80 on short final

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20C/doug_sperber_M20C.txt

Rights: licensed

Checklist hash: b167985a5d7f4e9ad0f67ba3a0b8e827f1a88b65ad514aebe40db3d9c0b809df

Verify against the aircraft's own approved documentation before use.