Mooney M20C — jim_allen_9128v

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Fuel Tank Measurements" item count differs between passes (17 vs 16); verify nothing was dropped.

content_gap: Section "Preflight Check" item count differs between passes (49 vs 47); verify nothing was dropped.

content_gap: Section "Takeoff" item count differs between passes (6 vs 7); verify nothing was dropped.

Preflight Check

1.

[ ]  Ignition Switch

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... ON to check outside lights, then OFF

[ ]  Fuel Selector Drain — Selector handle on R; pull ring and hold for 5 seconds. Repeat procedure with selector handle on L. .... DRAIN

[ ]  Instrument Static Port .... UNOBSTRUCTED

[ ]  Tail tiedown .... REMOVE

[ ]  Empennage .... CHECK

NOTE: Remove ice, snow, or frost.

[ ]  Tail Cone Access Door .... SECURE

[ ]  Instrument Static Port .... UNOBSTRUCTED

[ ]  Static Drain System .... CHECK

[ ]  Wing Skins .... CHECK

[ ]  Flap and Attach Points .... CHECK

[ ]  Aileron and Attach Points .... CHECK

[ ]  Wing Tip and Navigation Light .... CHECK

NOTE: Remove ice, snow, or frost.

[ ]  Left Wing Leading Edge .... CHECK

[ ]  Pitot Tube and Stall Switch Vane .... UNOBSTRUCTED

[ ]  Fuel Tank .... CHECK QUANTITY; SECURE CAP

[ ]  Chock and Tiedown .... REMOVE

[ ]  Left Main Gear Shock Discs and Tire .... CHECK

[ ]  Fuel Tank Sump Drain .... SAMPLE

[ ]  Pitot System Drain .... CHECK

[ ]  Tank Vent .... UNOBSTRUCTED

[ ]  Fuel Selector Drain Valve .... CLOSED

[ ]  Windshields .... CLEAN

[ ]  Left Side Engine Cowl Fasteners .... SECURE

[ ]  Propeller .... CHECK for nicks and cracks

[ ]  Forward Engine Components — Starter, alternator belt, etc. .... CHECK

[ ]  Induction Air Filter .... CHECK clean and sealed

[ ]  Landing Light .... CHECK

[ ]  Nose Gear — Tire; check for towing damage .... CHECK

[ ]  Shock Discs .... CHECK

[ ]  Right Side Engine Cowl Fasteners .... SECURE

[ ]  Engine Oil Level .... CHECK (Full for Extended Flight)

[ ]  Windshield .... CLEAN

[ ]  Fuel Tank Sump Drain .... SAMPLE

[ ]  Tank Vent .... UNOBSTRUCTED

[ ]  Chock and Tiedown .... REMOVE

[ ]  Right Main Gear Shock Discs and Tire .... CHECK

[ ]  Fuel Tank .... CHECK QUANTITY; SECURE CAP

[ ]  Right Wing Leading Edge .... CHECK

[ ]  Wing Skins .... CHECK

[ ]  Wing Tip and Navigation Light .... CHECK

[ ]  Aileron and Attach Points .... CHECK

[ ]  Flap and Attach Points .... CHECK

NOTE: Remove ice, snow, or frost.

[ ]  Baggage Door .... SECURE

Before Starting

[ ]  Fuel Selector Handle .... SET for fuller tank

[ ]  Parking Brake .... PULL ON

[ ]  Ignition and Master Switches .... OFF

[ ]  Carburetor Heat .... OFF

[ ]  Landing Gear Switch .... DOWN

[ ]  Mixture Control .... IDLE CUTOFF

[ ]  Propeller .... HIGH RPM

[ ]  Throttle .... CLOSE

[ ]  Boost Pump .... OFF

[ ]  Anti-collision Light .... OFF

[ ]  Navigation Lights .... OFF

[ ]  Landing Light .... OFF

[ ]  Radios .... ALL OFF

[ ]  Cabin Heat .... OFF

[ ]  Main Circuit Breaker Panel .... CHECK

[ ]  Auxiliary Circuit Breaker Panel .... CHECK

Starting Check

[ ]  Master Switch .... ON

[ ]  Fuel Selector Indicators .... CHECK for conformity to observed quantity

[ ]  Annunciator Lights .... PRESS to TEST

[ ]  Landing GEAR DN Light .... GREEN

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture Control .... OPEN to FULL RICH

[ ]  Throttle .... PUMP TWICE to prime engine; then open approximately one-quarter

[ ]  Ignition-Starter Switch .... turn to START and PRESS forward. When engine fires, release to BOTH.

[ ]  Throttle .... Set for 1000 to 1200 RPM

[ ]  Oil Pressure Gage .... 25 PSI MIN

[ ]  Carburetor Heat .... ON momentarily to check operation

[ ]  Fuel Pressure Gage .... GREEN ARC

[ ]  Lights .... As required

[ ]  Anticollision Light .... ON

[ ]  Pitot Heat .... CHECK and note ammeter deflection

[ ]  Radios .... ON and CHECK

[ ]  Stabilizer Trim Indicator .... TAKEOFF

[ ]  Fuel Selector Handle .... CHECK right and left

Flooded Engine Clearing  [ABNORMAL]

[ ]  Throttle .... FULL OPEN

[ ]  Mixture Control .... IDLE CUTOFF

[ ]  Electric Fuel Pump .... OFF

[ ]  Ignition Starter Switch .... turn to START and PRESS forward

[ ]  Throttle .... RETARD when engine starts

[ ]  Mixture Control .... OPEN slowly to FULL RICH

Cold Weather Starting

NOTE: HAND CRANKING

Before-Takeoff Check

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Fuel Selector Handle .... SET to fuller tank

[ ]  Altimeter .... SET to field elevation or field altimeter setting

[ ]  Directional Gyro .... SET to magnetic compass

[ ]  Flight Instruments

[ ]  Engine Instruments

[ ]  Clock .... SET and wind as needed

[ ]  Stabilizer Trim .... SET for TAKEOFF

[ ]  Propeller .... CYCLE and CHECK at 1700 RPM

[ ]  Magnetos .... CHECK at 1700 RPM

[ ]  Wing Flaps .... SET for TAKEOFF or as desired

[ ]  Seats .... LOCKED

[ ]  Seat Belts .... FASTENED

[ ]  Door and Pilot Window .... LATCHED CLOSED

UPON TAKING RUNWAY

[ ]  Fuel Pump .... ON

[ ]  Transponder .... ALT 1200 or as assigned

[ ]  Mixture .... LEANED for Best Power

[ ]  Propeller .... FULL INCREASE

[ ]  Flaps .... TAKEOFF or as desired

[ ]  Fuel Selector Handle .... FULLER TANK

WARNING: Do not change fuel tanks immediately before takeoff.

Takeoff

NOTE: At 65-75 MPH increase back pressure, then relax slightly after A/C breaks ground to keep nose on the horizon. For crosswind takeoff, hold A/C on the ground to a higher speed, then pull up abruptly and make a coordinated turn into wind to correct for drift when clear of ground.

[ ]  Landing gear .... Retract as soon as safely airborne and in good control

[ ]  Flaps .... Retract upon clearing obstacles and IAS of 80 to 90 MPH

[ ]  Throttle .... Reduce to 25" MP w/i 2 minutes

[ ]  Fuel Pump .... Turn off at a safe altitude

NOTE: Climb at 115-120 for visibility and cooling. Best rate of climb speed is 100 MPH at sea level, decreasing approx. 1 MPH per 1000 feet of altitude. Maximum Angle of climb is about 80 MPH gear and flaps up.

Inflight Restarting  [ABNORMAL]

[ ]  Propeller .... HIGH RPM

[ ]  Fuel Selector .... Fuller Tank

[ ]  Mixture Control .... IDLE CUTOFF

[ ]  Boost Pump .... ON

[ ]  Throttle .... OPEN 1/4 travel

[ ]  Ignition Switch .... BOTH

[ ]  Mixture Control .... Move slowly and smoothly to FULL RICH

[ ]  Power and RPM .... Re-establish cruise power and RPM, then lean mixture

Descent Prior to Landing

[ ]  Reduce Power gradually to avoid rapid engine cooling.

[ ]  Plan descent early.

[ ]  GEAR and FLAP SPEED .... 120 MPH. DO NOT EXCEED!!

Before Landing Check

[ ]  Seat Belts .... FASTENED

[ ]  Fuel Selector Handle .... SET to fuller tank

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture Control .... FULL RICH

[ ]  Carburetor Heat .... FULL ON

[ ]  Airspeed .... REDUCE to 120 MPH

[ ]  Propeller .... FULL INCREASE

[ ]  Landing Gear .... DOWN and LOCKED; green annunciator light ON

[ ]  Flaps .... As required

[ ]  Trim .... As required

After Landing and Taxi Check

[ ]  Electric Fuel Pump .... OFF

[ ]  Carburetor Heat .... OFF

[ ]  Wing Flaps .... RETRACT

[ ]  Transponder .... STANDBY

[ ]  Stabilizer Trim .... TAKEOFF

[ ]  Throttle .... 1000 to 1200 RPM

Shutdown Check

[ ]  Throttle .... IDLE at 1000 to 1200 RPM until CHT starts to drop

[ ]  Radios .... OFF

[ ]  Electrical Switches .... OFF

[ ]  Mixture Control .... IDLE CUTOFF

[ ]  Throttle .... RETARD as engine stops firing

[ ]  Ignition Starter Switch .... OFF when propeller stops

[ ]  Trim .... TAKEOFF

[ ]  Flaps .... RETRACTED

[ ]  Master Switch .... OFF

[ ]  Overhead Air Scoop .... CLOSED

[ ]  Seat belts and equipment .... STOWED

[ ]  Wheel Chocks and Tiedown .... As required

Fuel Tank Measurements

INCHES | MARK | GALLONS

NOTE: 0 in — None — 4 gal

NOTE: 2 in — 1 — 5 gal

NOTE: 2.5 in — 2 — 6 gal

NOTE: 3 in — 3 — 7 gal

NOTE: 3.75 in — 4 — 8 gal

NOTE: 4.5 in — 5 — 9 gal

NOTE: 5 in — 6 — 10 gal

NOTE: 5.5 in — 7 — 11 gal

NOTE: 6 in — 8 — 12 gal

NOTE: 6.125 in — 9 — 13 gal

NOTE: 7.5 in — 10 — 14 gal

NOTE: 8.5 in — 11 — 16 gal

NOTE: 9.75 in — 12 — 18 gal

NOTE: 11 in — 13 — 20 gal

NOTE: 12.25 in — 14 — 22 gal

NOTE: 13.5 in — 15 — 24 gal

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20C/jim_allen_9128v.txt

Rights: licensed

Checklist hash: ef79506cd95850ad22021ed70df04ad025336a7a93c368b551b224cf1792547b

Verify against the aircraft's own approved documentation before use.