Mooney M20J_MSE — mooney_m20j_checklist_mwaugh

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Start" item count differs between passes (165 vs 16); verify nothing was dropped.

Before Start

[ ]  Preflight .... COMPLETE

[ ]  Fuel Quantity .... ____ G

[ ]  Paperwork .... COMPLETE

[ ]  Radio Master .... OFF

[ ]  Transponder .... STAND-BY

[ ]  Mixture .... CUT-OFF

[ ]  Propeller .... HIGH

[ ]  Seats/Seat Belts .... SECURE

[ ]  Fuel Selector .... CORRECT TANK

[ ]  Briefing .... Complete

ELECTRICAL POWER ON

[ ]  Master .... ON

[ ]  Anti-Collision .... ON

[ ]  Annunciators .... TEST

[ ]  Electric Stand-by Vac. .... CHECK

[ ]  Brakes .... SET

STARTING

[ ]  Fuel Pump .... ON

[ ]  Mixture .... RICH (7 secs)

[ ]  Fuel Pump .... OFF

[ ]  Throttle .... OPEN ¼”

[ ]  Area .... CLEAR

[ ]  Engine .... START

[ ]  Mixture .... RICH

AFTER START

[ ]  Engine/Inst. .... NORMAL

[ ]  Mixture .... LEAN

[ ]  Trim .... ON

[ ]  Lights .... AS REQUIRED

[ ]  Radios .... ON/SET

[ ]  Time Out .... ______

[ ]  Autopilot .... CHECK

[ ]  Stand-by Vacuum .... CHECK

[ ]  ATIS/AWOS .... OBTAIN

[ ]  Altimeter .... SET

[ ]  Heading .... SET

TAXI CHECKS

[ ]  Brakes .... CHECK

[ ]  Flight Instruments. .... CORRECT

RUNUP

[ ]  Mixture .... RICH

[ ]  Engine .... 1900 RPM

[ ]  Magnetos .... CHECKED

[ ]  Propeller .... CHECKED

[ ]  Electrical .... CHECKED

[ ]  Vacuum .... CHECKED

BEFORE TAKEOFF

[ ]  Controls .... FREE/CORRECT

[ ]  Seats/Seat Belts .... SECURE

[ ]  Trim .... TAKEOFF

[ ]  Flaps .... TAKEOFF

[ ]  Autopilot .... OFF

[ ]  Annunciator Lts .... CHECKED

[ ]  Door/Window .... LATCHED

[ ]  Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Transponder .... ALT

[ ]  Pitot Heat .... AS REQUIRED

[ ]  Lights .... ON

TAKING RUNWAY

[ ]  Flaps .... Takeoff

[ ]  Heading .... RUNWAY

[ ]  Time Off .... ______

TAKEOFF

[ ]  Rotate .... 63 K

[ ]  Climb .... 85 K

[ ]  Gear .... UP (No Runway)

[ ]  Flaps .... UP

500 AGL

[ ]  Throttle .... 25”

[ ]  Propeller .... 2500 RPM

[ ]  Climb .... 100 K

CLIMB

[ ]  Gear .... UP

[ ]  Flaps .... UP

[ ]  Boost Pump .... OFF

[ ]  Power/RPM .... 25”/2500

[ ]  Mixture .... LEAN (if needed)

[ ]  Cowl Flaps .... TRAIL

CRUISE

[ ]  Power/RPM .... SET

[ ]  Cowl Flaps .... CLOSE

[ ]  Ram Air .... AS NEEDED

[ ]  Mixture .... LEAN

[ ]  Landing Light .... OFF

[ ]  Engine Inst. .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Vacuum .... CHECK

APPROACH

[ ]  Seats/Belts .... SECURE

[ ]  Fuel Quantity .... ______ G

[ ]  Ram Air .... OFF

[ ]  ATIS/AWOS .... OBTAINED

[ ]  Briefing .... COMPLETE

[ ]  Radios .... SET

[ ]  Heading .... SET

[ ]  Altimeter .... SET

[ ]  Boost Pump .... ON

[ ]  Lights .... ON

FINAL APPROACH FIX

[ ]  Instruments .... NO FLAGS

[ ]  Gear .... DOWN

[ ]  Flaps .... AS REQUIRED

LANDING

[ ]  Boost Pump .... ON

[ ]  Gear .... DOWN

[ ]  Mixture .... RICH

[ ]  Propeller .... ADVANCED

[ ]  Cowl Flaps .... OPEN

[ ]  Flaps .... LANDING

APPROACH SPEEDS

[ ]  Downwind .... 100 K

[ ]  Gear/Flaps .... 80 K

[ ]  Short Final .... 71 K

SPEEDS

[ ]  VX .... 66 K

[ ]  VY .... 85 K

[ ]  Best Glide .... 90 K

[ ]  VA .... 120K

[ ]  VFE .... 115 K

[ ]  VLO (Retract) .... 105 K

[ ]  VLO (Extend) .... 130 K

[ ]  VLE .... 130 K

POWER SETTINGS

CONDITION | MP | RPM | GPH

[ ]  Takeoff .... Full | 2700 | 18

[ ]  Initial Climb .... 25” | 2500 | 14

[ ]  Cruise Climb .... 24” | 2500 | 10

[ ]  Cruise .... 24 | 2400 | 9.6

[ ]  Cruise .... 22 | 2400 | 8.2

[ ]  Cruise .... 22 | 2200 | 7.8

[ ]  Cruise .... 18 | 2200 | 6.8

[ ]  Descent .... 16 | 2200/2400 | 6.1

AFTER LANDING

[ ]  Time Down .... ______

[ ]  Flaps .... UP

[ ]  Cowl Flaps .... OPEN

[ ]  Mixture .... LEAN

[ ]  Transponder .... STANDBY

[ ]  Pitot Heat .... OFF

[ ]  Boost Pump .... OFF

[ ]  Lights .... AS REQUIRED

[ ]  Trim .... TAKEOFF

SHUTDOWN

[ ]  Radios .... OFF

[ ]  Switches .... OFF

[ ]  Mixture .... CUT-OFF

[ ]  Throttle .... CLOSE

[ ]  Magnetos .... OFF

[ ]  Master .... OFF

SECURING

[ ]  Flight Controls .... SECURE

[ ]  Window .... CLOSED

[ ]  Sun Shields .... INSTALLED

[ ]  Tie-downs .... INSTALLED

[ ]  Baggage Lights .... OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20J_MSE/mooney_m20j_checklist_mwaugh.txt

Rights: licensed

Checklist hash: c8345a4a9e9429131b8a0419f1acfbaaa3146bfe7d083604d0e7fbc2592ca7f4

Verify against the aircraft's own approved documentation before use.