Mooney M20J_MSE — mooney_m20j_procedures_mwaugh

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

TAKEOFF

NORMAL TAKEOFF – FLAPS 15°

[ ]  Full Power .... SET

[ ]  Rotate VR 62K – Takeoff VLOF 70K .... ROTATE

[ ]  Climb VY 85K .... CLIMB

[ ]  No Runway – Positive Rate – GEAR UP .... GEAR UP

[ ]  500' AGL – 105K .... 105K

[ ]  500' AGL – 25" / 2500 RPM – Takeoff Checklist

[ ]  1000' AGL – Takeoff/Climb Checklist

MAXIMUM PERFORMACE TAKEOFF – FLAPS 15°

[ ]  Hold Brakes .... HOLD

[ ]  Full Power .... SET

[ ]  Release Brakes .... RELEASE

[ ]  Rotate VR 62K – VLOF 66K .... ROTATE

[ ]  Climb VX 66K to 50' .... CLIMB

[ ]  Climb VY 85K .... CLIMB

[ ]  No Runway – Positive Rate – GEAR UP .... GEAR UP

[ ]  500' AGL – 105K .... 105K

[ ]  500' AGL – 25" / 2500 RPM – Takeoff Checklist

[ ]  1000' AGL – Takeoff/Climb Checklist

APPROACH

NORMAL APPROACH

[ ]  Approach Checklist

[ ]  Downwind 105K (15") .... 105K (15")

[ ]  GEAR DOWN / FLAPS 30° .... GEAR DOWN / FLAPS 30°

[ ]  Descend – 80K – 2000 RPM .... 80K – 2000 RPM

[ ]  “GEAR DOWN – CHECKLIST” .... CALLED

[ ]  Base – “GUMPFF” .... CALLED

[ ]  400' AGL – “GEAR DOWN STABILIZED” .... CALLED

[ ]  Short Final – Power to IDLE – 71K .... POWER IDLE – 71K

[ ]  Touchdown 71K .... 71K

MAXIMUM PERFORMANCE APPROACH

[ ]  As Normal Approach

INSTRUMENT APPROACHES

ILS APPROACH

[ ]  Approach Checklist

[ ]  105K to Intercept (15") .... 105K (15")

[ ]  G/S Int. – GEAR DOWN .... GEAR DOWN

[ ]  G/S Intercept – “NO FLAGS” .... CALLED

[ ]  G/S Descent - 90K (15") .... 90K (15")

[ ]  “GEAR DOWN – CHECKLIST” .... CALLED

[ ]  1000' AGL “GUMPFF” .... CALLED

[ ]  400' AGL “GEAR DOWN – STABILIZED” .... CALLED

[ ]  100' ABOVE MINS “100 TO GO” .... CALLED

[ ]  “LANDING” – FLAPS 30° .... FLAPS 30°

[ ]  Short Final – POWER IDLE – 71K .... POWER IDLE – 71K

[ ]  Touchdown 71K .... 71K

NON-PRECISION APPROACH

[ ]  Approach Checklist

[ ]  105K Procedure Turn (15") .... 105K (15")

[ ]  2E – 2Miles FAF GEAR DOWN .... GEAR DOWN

[ ]  FAF – “NO FLAGS” .... CALLED

[ ]  FAF Descent – 90K (15") .... 90K (15")

[ ]  “GEAR DOWN – CHECKLIST” .... CALLED

[ ]  1000' AGL “GUMPFF” .... CALLED

[ ]  100' ABOVE MINS “100 TO GO” .... CALLED

[ ]  MDA – Level (18") .... LEVEL (18")

[ ]  Leaving MDA – FLAPS 30° .... FLAPS 30°

[ ]  400' AGL “GEAR DOWN – STABILIZED” .... CALLED

[ ]  Short Final – POWER IDLE – 71K .... POWER IDLE – 71K

[ ]  Touchdown 71K .... 71K

CIRCLE TO LAND

[ ]  Turning Final, Leaving MDA – FLAPS 25° .... FLAPS 25°

[ ]  Short Final – POWER IDLE – 71K .... POWER IDLE – 71K

[ ]  Touchdown 71K .... 71K

MISSED APPROACH

[ ]  Full Power – PITCH 85K .... FULL POWER – 85K

[ ]  FLAPS 15° .... FLAPS 15°

[ ]  Positive Rate – GEAR UP .... GEAR UP

[ ]  FLAPS 0° .... FLAPS 0°

[ ]  500' AGL – 100K .... 100K

[ ]  500' AGL - 25" / 2500 RPM .... 25" / 2500 RPM

[ ]  1000' AGL - Takeoff/Cruise Checklist

[ ]  HOLDING SPEED 90K .... 90K

STEEP TURNS

[ ]  SPEED 120K - 20" / 2300 RPM .... 120K - 20" / 2300 RPM

[ ]  50° BANK – POWER 23" - TRIM .... POWER 23" - TRIM

STALLS and SLOW FLIGHT

SLOW FLIGHT

[ ]  Clearing Turns – 105K (15") .... 105K (15")

[ ]  GEAR DOWN / FLAPS 30° .... GEAR DOWN / FLAPS 30°

[ ]  GEAR DOWN Checklist

[ ]  LEVEL FLIGHT – 65K .... 65K

[ ]  CRUISE POWER .... SET

[ ]  FLAPS 0° / SPEED 85K .... FLAPS 0° / 85K

[ ]  Positive Rate – GEAR UP .... GEAR UP

[ ]  SPEED 85K .... 85K

STALL – POWER OFF

[ ]  Clearing Turns – 105K (15") .... 105K (15")

[ ]  GEAR DOWN / FLAPS 30° .... GEAR DOWN / FLAPS 30°

[ ]  GEAR DOWN Checklist

[ ]  DESCENT – 80K .... 80K

[ ]  POWER to IDLE – Pitch to BUFFET .... POWER IDLE – BUFFET

[ ]  PITCH DOWN – FULL POWER .... PITCH DOWN – FULL POWER

[ ]  Positive Rate – GEAR UP – SPEED 85K .... GEAR UP – 85K

[ ]  FLAPS 0° / SPEED 85K .... FLAPS 0° / 85K

[ ]  SPEED 85K .... 85K

STALL – POWER ON

[ ]  Clearing Turns – 105K (15") .... 105K (15")

[ ]  POWER 13" / SPEED 70K .... POWER 13" / 70K

[ ]  POWER 21" / Pitch to BUFFET .... POWER 21" / BUFFET

[ ]  PITCH DOWN – FULL POWER .... PITCH DOWN – FULL POWER

[ ]  SPEED 85K .... 85K

[ ]  SPEED 85K .... 85K

CONFIGURATION CHECKLIST

[ ]  Fuel Selectors – ON .... ON

[ ]  Cowl Flaps – OPEN .... OPEN

[ ]  Mixtures – RICH .... RICH

[ ]  Prop – HIGH .... HIGH

[ ]  Fuel Pumps - ON .... ON

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20J_MSE/mooney_m20j_procedures_mwaugh.txt

Rights: licensed

Checklist hash: 131ece7ab3a319a5c8a94956689bed5e530a3b07e807bef67a935fa9604eaa4a

Verify against the aircraft's own approved documentation before use.