Mooney M20J_MSE — Paul_Coleman_Mooney_M20J_MSE_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Cockpit Preflight

[ ]  Keys .... Dash

[ ]  Documents .... AROW

[ ]  Fire Extinguisher .... Check Secured

[ ]  Gear Selector .... Down

[ ]  Circuit Breakers .... In

[ ]  Rocker Switches / Radio Master / Alternator .... Off

[ ]  Magneto / Start Switch .... Off

[ ]  Master Switch .... On

[ ]  Fuel Gauges (L/R) .... Check

[ ]  Check Voltage .... (push to test)22-24 Volts

[ ]  Annunciator .... Press to test

[ ]  Pitot Heat .... On

[ ]  Pitot Heat Annunciator .... Illuminated

[ ]  Pitot Heat .... Off

[ ]  Exterior Lights .... Check

[ ]  High/Low Vac Annunciator .... Flashing

[ ]  Standby Vacuum .... On

[ ]  Low Vac Annunciator .... Off

[ ]  Standby Vacuum Annunciator .... On

[ ]  Vacuum Gauge .... Check

[ ]  Standby Vacuum .... Off

[ ]  Alt. Static Source .... Check/Off

[ ]  Cowl Flaps .... Open

[ ]  Flaps .... Takeoff Position

[ ]  Master Switch .... Off

[ ]  R Fuel Selector .... Pull Gascolator Ring (5 sec)

[ ]  L Fuel Selector .... Pull Gascolator Ring (5 sec)

Right Fuselage / Tail Cone / Empennage

[ ]   .... Clear

[ ]  Skin .... Inspect

[ ]  Access Panels (4) .... Secured

[ ]  Tiedown .... Removed

[ ]  Elevator, / Rudder / Skin .... Inspect

Left Fuselage

[ ]  Fresh Air Vent .... Clear

[ ]   .... Clear

[ ]  Skin (8 access panels / Data plate) .... Inspect

[ ]  Static System Drain Valve .... Push Up (3-5 sec)

Left Wing

[ ]  Skin .... Inspect

[ ]  Flap .... Inspect

[ ]  Aileron .... Inspect

[ ]  Control Linkages .... Inspect

[ ]  Tip, Lights and Lens .... Inspect

[ ]  Pitot .... Clear

[ ]  Landing / Taxi Lights Lens .... Inspect

[ ]  Stall Warning Vane .... Unobstructed

[ ]  Fuel Tank .... Check Quantity – Secure Cap

NOTE: Reduced fuel indicator located in filler neck indicates usable fuel capacity of 25 Gallons.

NOTE: Visual fuel quantity gauge is for partial fueling purposes only.

[ ]  Tiedown .... Remove

[ ]  Tank Vent .... Clear

[ ]  Main Gear .... Inspect

[ ]  Fuel Drain .... Drained / Clear

[ ]  Pitot System Drain Valve .... Push Up (3-5 Sec)

[ ]  Gascolator Drain Valve .... Closed / Check for Leaks

Nose

[ ]  Windshield .... Clean

[ ]  Left Air Inlet .... Clear

[ ]  Left Cowl and Fasteners .... Secure

[ ]  Cowl Flap .... Inspect

[ ]  Propeller and Spinner .... Inspect

[ ]  Air Intakes .... Clear

[ ]  Nose Gear .... Inspect

[ ]  Right Cowl and Fasteners .... Secured

[ ]  Engine Oil .... 6-8 Qts

[ ]  Exhaust Pipe .... Inspect

[ ]  Right Cowl Flap .... Inspect

[ ]  Right Air Inlet .... Clear

Right Wing

[ ]  Fuel Drain .... Drained / Clear

[ ]  Main Gear .... Inspect

[ ]  Tank Vent .... Clear

[ ]  Tiedown .... Remove

[ ]  Landing / Taxi Lights Lens .... Inspect

[ ]  Fuel Tank .... Check Quantity – Secure Cap

NOTE: Reduced fuel indicator located in filler neck indicates usable fuel capacity of 25 Gallons.

NOTE: Visual fuel quantity gauge is for partial fueling purposes only.

[ ]  Tip, Lights, & Lens .... Inspect

[ ]  Aileron .... Inspect

[ ]  Flap .... Inspect

[ ]  Control Linkages .... Inspect

[ ]  Skin .... Inspect

[ ]  Baggage Door .... Locked

[ ]  Final Walkaround .... Complete

Before Starting

[ ]  HOBBS .... Record

[ ]  Wheel Chocks .... Remove

[ ]  Seats / Belts / Shoulder Harness .... Adjust / Secure

[ ]  Passengers .... Brief

[ ]  Magneto / Start Switch .... Off

[ ]  Master Switch .... Off

[ ]  Alternator .... Off

[ ]  Radio Master .... Off

[ ]  Alternate Static .... Push Off

[ ]  Rocker Switches .... Off

[ ]  Gear .... Down

[ ]  ELT .... Armed

[ ]  HSI .... Slaved

[ ]  Circuit Breakers .... Check

[ ]  Throttle .... Closed

[ ]  Propeller .... High RPM

[ ]  Mixture .... Idle-Cutoff

[ ]  Parking Brake .... Set

[ ]  Cabin Vent .... As Desired

[ ]  Cabin Heat .... Push Off

[ ]  Defrost .... Push Off

[ ]  Fuel Selector .... Reduced Tank

[ ]  Emergency Gear Handle .... Down and Latched

Engine Starting - Normal Start

[ ]  Master Switch .... On

[ ]  Alternator .... On

[ ]  Nav / Recognition Lights .... On

[ ]  Annunciator Panel .... Test

[ ]  Throttle .... 1/4 Open

[ ]  Propeller .... Full Forward

[ ]  Mixture .... Full Rich

[ ]  Boost Pump (Cold Engine) .... On 3-5 sec / Off

[ ]  Mixture .... Idle-Cutoff

[ ]  Area .... Clear

[ ]  Starter .... Push / Engage (30 sec max)

NOTE: “START POWER” illuminates with Magneto / Start switch in start position and must extinguish when released

[ ]  Mixture .... Rich when engine starts

[ ]  Throttle .... 1000-1200 RPM

[ ]  Oil Pressure .... Green Arc

[ ]  Engine Instruments .... Check

[ ]  Ammeter / Volts .... Check

[ ]  Taxi / Nav Lights .... As Required

[ ]  Fuel Flow Computer .... Check / Set

FLOODED ENGINE

[ ]  Boost Pump .... Off

[ ]  Throttle .... Full Forward

[ ]  Prop .... Full Forward

[ ]  Mixture .... Idle-Cutoff

[ ]  Starter .... Engage

NOTE: Reduce throttle / enrich mixture when engine starts

WARM ENGINE

[ ]  Boost Pump .... Off

[ ]  Throttle .... 1/4 Open

[ ]  Prop .... Full Forward

[ ]  Mixture .... Idle-Cutoff

[ ]  Starter .... Engage

NOTE: Reduce throttle / enrich mixture when engine starts

Before Taxiing

[ ]  Radio Master .... On

[ ]  Elevator Trim Switch .... On

[ ]  Autopilot .... Test as required

[ ]  Flaps .... Takeoff

[ ]  Radios .... Set

[ ]  Transponder .... Set / Standby

[ ]  Altimeter .... Set

[ ]  GPS .... Programmed

During Taxi

[ ]  Brakes and Steering .... Check

[ ]  Directional Gyro .... Proper Indication

[ ]  Magnetic Compass .... Free and Correct

[ ]  Turn Coordinator .... Proper Indication

[ ]  Attitude Indicators .... Caged / Upright

Before Takeoff

[ ]  Brakes .... Set

[ ]  Fuel Selector (Timer) .... Fullest Tank

[ ]  Propeller .... Full Forward

[ ]  Mixture .... Full Forward

[ ]  Cowl Flaps .... As Required

[ ]  Throttle .... 1900-2000 RPM

[ ]  Magnetos .... Check (<175 RPM Drop, 50 RPM Δ)

[ ]  Propeller .... Cycle 3X

[ ]  Ammeter .... Charge

[ ]  Annunciators .... Check

[ ]  Throttle .... Check Idle / 1000 RPM

[ ]  Trim .... Set for Takeoff

[ ]  Takeoff Flaps (15°) .... Visually Verified

[ ]  Speedbrakes .... Tested and Retracted

[ ]  Flight Controls .... Box Check

[ ]  Autopilot .... Standby (Not On)

[ ]  Cabin Door .... Check Secure

[ ]  Window .... Closed

Takeoff

NOTE: Takeoff Performance – Max Weight, Zero Wind

[ ]  Lights .... On

[ ]  Transponder .... Set / ALT

[ ]  Boost Pump .... On

[ ]  Mixture .... Full Rich

[ ]  Throttle .... Full (2700 RPM)

[ ]  Engine Gauges and Annunciators .... Check

[ ]  Rotate at Vr .... 59 KIAS

[ ]  Gear .... Up

[ ]  Flaps .... Up

Climb

[ ]  Cruise Climb .... MP 26”/ RPM 2600

[ ]  Cowl Flaps .... As Required

[ ]  Airspeed .... 90-100 KIAS

[ ]  Boost Pump .... Off

NOTE: Maintain these power settings and attitude to at least 3000AGL or cruise altitude.

Cruise

NOTE: Cruise Performance – Press Alt / Standard Temp 25°F EGT ROP

[ ]  Power .... Set

[ ]  Economy Cruise .... 25°F rich of peak EGT

[ ]  Best Power .... 100° F rich of peak EGT

[ ]  Cowl Flaps .... Closed

Fuel Tank Selection

[ ]  Boost Pump .... On

[ ]  Fuel Selector .... Opposite Tank

[ ]  Boost Pump .... Off

Descent

[ ]  Seat Belts / Shoulder Harness .... Fastened

[ ]  Mixture / Power .... As Required

CAUTION: Avoid continuous operation between 1500 and 1950RPM with less than 15” Manifold Pressure

[ ]  CHT .... 300° F Minimum

[ ]  Speedbrakes .... As needed 22” / 2200 RPM

Approach and Landing

NOTE: Increase approach speed by approximately 10 KIAS for crosswind landing. Below 10 knots crosswind, use 15° flaps. Flaps UP for crosswind greater than 10 knots.

[ ]  Boost Pump .... On

[ ]  Fuel Selector .... Fullest Tank

[ ]  Gear .... Down & Locked below 132 KIAS

[ ]  Mixture .... Full Rich

[ ]  Propeller .... Full Forward

[ ]  Lights .... On

[ ]  Flaps .... Down below 112 KIAS

[ ]  Airspeed .... 80KIAS

[ ]  Short Final .... 70-78 KIAS

NOTE: Landing Performance – Max Weight

Missed Approach / Go Around

[ ]  Power .... Full Throttle / 2700 RPM

[ ]  Mixture .... Full Rich

[ ]  Airspeed .... 65 KIAS

[ ]  Takeoff Flaps (15°) .... After Climb Established

[ ]  Speedbrakes .... Retract

[ ]  Airspeed .... 76 KIAS

[ ]  Gear .... Up

[ ]  Flaps .... Up

[ ]  Cowl Flaps .... Open

[ ]  Airspeed .... 86 KIAS

After Landing

[ ]  Transponder .... Standby

[ ]  Boost Pump .... Off

[ ]  Lights .... As Required

[ ]  Flaps .... Up

[ ]  Cowl Flaps .... Open

[ ]  Speedbrakes .... Retracted

Shut Down

[ ]  Radio Master .... Off

[ ]  Rocker Switches .... Off

[ ]  Throttle .... Low Idle (until CHT starts to drop)

[ ]  Mixture .... Idle-cutoff

[ ]  Magnetos .... Off

[ ]  Alternator .... Off

[ ]  Master Switch .... Off

Emergency Procedure Flows  [EMERGENCY]

ENGINE FAILURES

Engine Failure During Takeoff  [EMERGENCY]

[ ]  Throttle .... Idle

[ ]  Brakes .... Apply

[ ]  Fuel Selector .... Off

[ ]  Magnetos .... Off

[ ]  Master Switch .... Off

In Flight Restart Procedure  [EMERGENCY]

[ ]  Airspeed .... 85 KIAS

[ ]  Fuel Selector .... Switch Tanks

[ ]  Fuel Pressure .... Verify

[ ]  Boost Pump .... On

[ ]  Throttle / Propeller / Mixture .... Full Forward

[ ]  Magnetos .... Both

[ ]  Starter .... Engage if propeller stopped

NOTE: If no start, mixture to idle cutoff, then gradually enrichen as engine starts

[ ]  Land .... As Soon As Practicable

Engine Roughness  [ABNORMAL]

[ ]  Engine Instruments .... Check

[ ]  Fuel Selector .... Switch Tanks

[ ]  Mixture .... Adjust for Smooth Operation

[ ]  Magneto / Starter .... R then L then Both

[ ]  Throttle .... Reduce

Cowl Flaps Fail Closed  [ABNORMAL]

[ ]  Power .... As Required

[ ]  Mixture .... Rich

[ ]  Airspeed .... 120 KIAS

[ ]  Temperatures .... Monitor

High CHT  [ABNORMAL]

[ ]  Mixture .... Enrich as Required

[ ]  Cowl Flaps .... Open as Required

[ ]  Airspeed .... Increase as Required

NOTE: Reduce Power if temperature cannot be maintained within limits

High Oil Temperature  [ABNORMAL]

NOTE: Prolonged high oil temperatures will usually be accompanied by a drop in oil pressure. If oil pressure remains normal, then a high temperature indication may be caused by a faulty gauge or temperature probe.

[ ]  Cowl Flaps .... Open as Required

[ ]  Airspeed .... Increase as Required

[ ]  Power .... Reduce

WARNING: PREPARE FOR POSSIBLE ENGINE FAILURE IF TEMPERATURE CONTINUES HIGH

Low Oil Pressure  [ABNORMAL]

[ ]  Oil Temperature and Pressure .... Monitor

[ ]  Land .... as soon as practicable

Engine Driven Boost Pump Failure  [ABNORMAL]

[ ]  Mixture .... Idle-Cutoff

[ ]  Throttle .... Cruise Position

[ ]  Boost Pump .... On

[ ]  Mixture .... Gradually Increase

[ ]  Land .... as soon as practicable

Engine Fire During Start  [EMERGENCY]

[ ]  Starter .... Continue Starting

[ ]  Throttle .... Full Throttle

IF ENGINE STARTS:

[ ]  Power .... 1500 RPM until fire extinguished

[ ]  Engine .... SHUTDOWN, Inspect for Damage

IF ENGINE DOES NOT START:

[ ]  Fuel Selector Valve .... Off

[ ]  Mixture .... Idle-Cutoff

[ ]  Throttle .... Full Forward

[ ]  Magnetos / Starter .... Off

[ ]  Master Switch .... Off

[ ]  Aircraft .... Evacuate

[ ]  Fire .... Extinguish

Engine Fire In Flight  [EMERGENCY]

[ ]  Fuel Selector .... Off

[ ]  Throttle .... Closed

[ ]  Mixture .... Idle-cutoff

[ ]  Magnetos /Starter .... Off

[ ]  Cabin Vents and Heat .... Off

[ ]  Cowl Flaps .... Closed

[ ]  Gear .... Down / Up, depending on terrain

[ ]  Flaps .... As Necessary

[ ]  Execute Forced Landing

WARNING: DO NOT ATTEMPT RESTART

Electrical Fire In Flight  [EMERGENCY]

[ ]  Master Switch .... Off

[ ]  Alternator Switch .... Off

[ ]  All electrics EXCEPT magnetos .... Off

[ ]  Cabin Air .... Open

[ ]  Fire .... Extinguished

IF FIRE IS OUT:

[ ]  Master Switch .... On

[ ]  Circuit Breakers .... In, Do NOT Reset

[ ]  Radios .... On, one at a time

[ ]  Cabin Air .... Open

Emergency Descent  [EMERGENCY]

[ ]  Power .... Retard, Initially

[ ]  Airspeed .... 132 KIAS

[ ]  Gear .... Extend

[ ]  Wing Flaps .... Up

[ ]  Cowl Flaps .... Closed

[ ]  Power .... As Required

Forced Landing Without Engine Power  [EMERGENCY]

[ ]  Glide Speed .... 93 KIAS

[ ]  Seats / Belts .... Upright / Locked

[ ]  Gear .... As required

[ ]  Mixture .... Idle-cutoff

[ ]  Fuel Selector .... Off

[ ]  Magnetos .... Off

[ ]  Flaps .... Full Down

[ ]  Master Switch .... Off

Forced Landing With Engine Power  [EMERGENCY]

[ ]  Seat Belts / Shoulder Harness .... Secure

[ ]  Cabin Door .... Unlatched

WHEN LANDING IS ASSURED:

[ ]  Fuel Selector .... Off

[ ]  Throttle .... As Required

[ ]  Mixture .... Idle Cutoff

[ ]  Magneto / Starter .... Off

[ ]  Wing Flaps .... Full Down

[ ]  Landing Gear .... Up or Down

[ ]  Master Switch .... Off

[ ]  Approach Speed .... As Slow As Possible

[ ]  Landing .... Level, Tail Low Attitude

Propeller Overspeed  [ABNORMAL]

[ ]  Throttle .... Reduce

[ ]  Oil Pressure .... Check

[ ]  Propeller .... Decrease if control available

[ ]  Airspeed .... Reduce

[ ]  Throttle .... Set to maintain RPM below 2700

Low Fuel Flow  [ABNORMAL]

[ ]  Mixture .... Enrich

[ ]  Fuel Selector .... Opposite Tank

Alternator Overvoltage  [ABNORMAL]

[ ]  Radio Master .... Off

[ ]  Master .... Off, then On

IF WARNING LIGHT IS STILL ILLUMINATED:

[ ]  Alternator Field Circuit Breaker .... Reset

IF CIRCUIT BREAKER WILL NOT RESET:

[ ]  Non-Essential Electrical Equipment .... Off

[ ]  Land .... When Practical

Voltage Warning Light Flashing  [ABNORMAL]

[ ]  Radio Master .... Off

[ ]  Non-Essential Equipment .... Off

[ ]  Alternator .... Off, then On

IF WARNING LIGHT IS STILL ILLUMINATED:

[ ]  Master Switch .... Off, then On

IF WARNING LIGHT IS STILL ILLUMINATED:

[ ]  Alternator Field Circuit Breaker .... Reset

[ ]  Ammeter .... Check

Landing Gear Fails To Extend  [ABNORMAL]

[ ]  Airspeed .... Below 132 KIAS

[ ]  Gear Circuit Breaker .... Pull

[ ]  Gear Lever .... Down

[ ]  Manual Extension .... Latch Forward / Lever Back

[ ]  T-Handle .... Pull (7-20 times) until Gear Down

[ ]   .... Stop when Resistance is felt

[ ]  Visual indicator .... Check

Landing Gear Fails To Retract  [ABNORMAL]

[ ]  Airspeed .... Below 107 KIAS

[ ]  Gear Handle .... Up

IF GEAR STILL DOWN, AND GEAR HORN ON AND GEAR SAFETY BYPASS LIGHT ILLUMINATED:

[ ]  Gear Safety Bypass Switch .... Depress and Hold until Gear Retracted

[ ]  Gear Down and Unsafe Lights .... Extinguished

[ ]  Gear Circuit Breaker .... Pull

IF GEAR STILL DOWN GEAR HORN OFF AND SAFETY BYPASS LIGHT OFF:

[ ]  Emergency Extension .... Verify Latched

[ ]  Gear Relays Circuit Breaker .... Reset

[ ]  Flight .... Continue If Desired

Cabin Door Open In Flight  [ABNORMAL]

[ ]  Airspeed .... Below 95 KIAS

[ ]  Pilots Window .... Open

[ ]  Aircraft .... Right Side Slip

[ ]  Door .... Pull Shut and Latch

Baggage Door Open In Flight  [ABNORMAL]

[ ]  Fly Normally .... Land, As Soon As Possible

Inadvertent Icing Encounter  [ABNORMAL]

[ ]  Pitot Heat .... On

[ ]  Alternate Static Source .... On, If Required

[ ]  Cabin Heat, Defrost .... On

NOTE: Any ice accumulation, limit 15° flaps for approach and landing. Increase approach speeds by at least 10 Knots.

NOTE: Reference POH for further instruction.

Emergency Exit Of Aircraft  [EMERGENCY]

[ ]  Cabin Door .... Pull Latch Handle Aft

[ ]  Open Door .... Exit Aircraft

BAGGAGE COMPARTMENT EXIT:

[ ]  Release Rear Seat Back Latches

[ ]  Fold Rear Seat Backs Forward .... Climb Over

[ ]  Pull Off Plastic Cover

[ ]  Pull Latch Pin

[ ]  Lift Red Handle Up

[ ]  Open Door and Exit Aircraft

Spins  [EMERGENCY]

[ ]  Throttle .... Idle

[ ]  Ailerons .... Neutral

[ ]  Rudder .... Full Opposite Spin

[ ]  Control Wheel .... Briskly Forward of Neutral

[ ]  Rotation .... Stop

[ ]  Wing Flaps .... Retract

[ ]  Rudder .... Neutralize, When Spin Stops

[ ]  Control Wheel .... Smoothly Move Aft

NOTE: Avoid flight conditions conducive to spin entry.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20J_MSE/Paul_Coleman_Mooney_M20J_MSE_Checklist.txt

Rights: licensed

Checklist hash: dbd62cae43366ef29886aee0f57d3a7f7acb7179960b8b6124251fdee673b4f2

Verify against the aircraft's own approved documentation before use.