Mooney M20J_MSE — phil_vergese_m20j

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

INITIAL PREFLIGHT

[ ]  Mags .... OFF

[ ]  Gear Switch .... DOWN

[ ]  Master Switch .... ON

[ ]  Fuel Gauges .... NOTE QUANTITY

[ ]  Annc. Lights .... PRESS TO TEST

[ ]  Nav, Strobe, Landing Light .... ON

OUTSIDE

[ ]  Wingtip & Tail Lights .... CHECK

[ ]  Stall Horn .... TEST

INSIDE

[ ]  Nav, Strobe, Landing Light .... OFF

[ ]  Electric Gyros .... SPOOLING

[ ]  Autopilot .... TEST

[ ]  Master .... OFF

[ ]  Alternate Static .... IN

[ ]  Controls .... FREE & CORRECT

[ ]  Fuel Drain .... LEFT & RIGHT (5 sec)

[ ]  Check Drained Fuel .... OUTSIDE

RIGHT REAR

[ ]  Grab Fuel Tester & Rag

[ ]  Antennas .... CHECK

[ ]  Right Side Static Port .... CLEAR

EMPENNAGE

[ ]  Tail Tiedown .... REMOVE

[ ]  Elevator & Rudder Attach .... CHECK

[ ]  Tail Light .... SECURE

[ ]  Tail Area Frost .... REMOVE

LEFT REAR

[ ]  Left Side Static Port .... CLEAR

[ ]  Tail Cone Access Door .... SECURE

[ ]  Static System Drain .... CHECK

LEFT WING

[ ]  Wing Skins .... CHECK

[ ]  Frost .... REMOVE

[ ]  Flap & Aileron Attach Points .... CHECK

[ ]  Wing Tip .... CHECK

[ ]  Leading Edge & Stall Strip .... CHECK

[ ]  Pitot Tube .... CHECK (remove cover)

[ ]  Stall Switch .... CHECK

[ ]  Fuel Tank .... CHECK QUANTITY

[ ]  Tiedown .... REMOVE

[ ]  Left Gear & Discs (30 psi) .... CHECK

[ ]  Fuel Drain .... SAMPLE

[ ]  Tank Vent .... UNOBSTRUCTED

[ ]  Pitot System Drain .... CHECK

LEFT COWLING

[ ]  Central Drain Valve .... CLOSED

[ ]  Windshield .... CLEAN

[ ]  Cowl Fastners .... SECURE

N201UT V2.3

PROP AREA

[ ]  Propeller .... CHECK

[ ]  Alternator Belt .... CHECK

[ ]  Air Filter .... CHECK

[ ]  Ram Air Door .... CLOSED

[ ]  Landing Light .... CHECK

[ ]  Nose Gear & Discs (49 psi) .... CHECK

RIGHT COWLING

[ ]  Cowl Fastners .... SECURE

[ ]  Oil Level .... CHECK (>7 gal)

[ ]  Windshield .... CLEAN

RIGHT WING

[ ]  Fuel Drain .... SAMPLE

[ ]  Tank Vent .... UNOBSTRUCTED

[ ]  Right Gear & Discs (30 psi) .... CHECK

[ ]  Tiedown .... REMOVE

[ ]  Fuel Tank .... CHECK QUANTITY

[ ]  Leading Edge & Stall Strip .... CHECK

[ ]  Wing Skins .... CHECK

[ ]  Frost .... REMOVE

[ ]  Wing Tip .... CHECK

[ ]  Flap & Aileron Attach Points .... CHECK

PASSENGER BRIEFING

[ ]  Bathroom Check

[ ]  Cell phones OFF

[ ]  "No Step" areas

[ ]  Treat all props as live. "Clear"

NOTE: Help Spotting Planes. Best way is to look at one spot for 2-3 seconds.

NOTE: Turbulence. Uncomfortable for us, but no danger to the aircraft.

NOTE: Ears. Let me know if you feel pain. Swallow or yawn as we climb & descend

NOTE: Questions (now or in-flight). No talking during takeoff and landing

NOTE: Seatbelts at all times. No smoking.

NOTE: If there's anything I do with the airplane that makes you uncomfortable, let me know right away.

NOTE: If you feel queasy tell me right away, look outside, nearest vent. Location of airsickness bags.

INSIDE PLANE

NOTE: Don't touch the anything without asking! If seat slips on takeoff, don't grab the yoke. Rudders.

[ ]  Vents (location & operation)

NOTE: Intercom is voice activated. I do checklists aloud.

NOTE: Radio callsign. Stop talking during incoming radio traffic. Hand up.

NOTE: Door – you control it & don't slam it

NOTE: Emergency Exit procedure (through door). Other exits

BEFORE STARTING ENGINE

[ ]  Tiedowns .... REMOVED

[ ]  Cell Phones .... OFF

[ ]  Hobbs Time .... RECORD

[ ]  Passenger Brief .... COMPLETE

[ ]  Baggage Door .... CLOSED

[ ]  Seats, Seat Belts .... SECURE

[ ]  Parking Brake .... SET

[ ]  Fuel .... LOWER TANK

[ ]  Circuit Breakers .... IN

[ ]  Master .... ON

[ ]  Avionics Master .... OFF

[ ]  Gear Light / Lever .... ON / DOWN

[ ]  Autopilot .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Ram Air .... CLOSED

[ ]  Mixture .... IDLE

[ ]  Prop .... FORWARD

[ ]  Throttle .... 1/4 (Full if flooded)

[ ]  Boost Pump .... OFF

[ ]  Key .... IN SWITCH

PRIME (IF >4 HOURS)

[ ]  Boost Pump .... ON UNTIL 15

[ ]  Mixture .... RICH (5 sec)

[ ]  Mixture .... IDLE

START

[ ]  Prop Area .... CLEAR

[ ]  Mags .... START

[ ]  Mixture .... RICH

[ ]  Throttle .... 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Fuel Pressure .... CHECK

[ ]  Mixture .... 3/4

[ ]  Avionics .... ON

[ ]  Nav Lights .... AS REQ

[ ]  ATIS/AWOS .... CHECK

[ ]  Altimiter .... SET

[ ]  Heading Indicator .... SET

TAXI

[ ]  ATC .... CLEARANCE

[ ]  Brakes .... TEST

[ ]  Turn Indicator / Ball .... CHECK

[ ]  Attitude Indicator .... CHECK

[ ]  Heading Indicator .... TRACKING

[ ]  Compass .... KNOWN HEADINGS

[ ]  Trim .... TAKEOFF

[ ]  Flaps .... TAKEOFF

[ ]  Autopilot .... OFF & HDG MODE

[ ]  Heading Bug .... TOP OF DG

[ ]  Autpilot .... ON & HDG HOLD ON

[ ]  Rotate Bug to either side

[ ]  Override Autopilot each direction

[ ]  Autopilot .... OFF

RUNUP

[ ]  Parking Brakes .... SET

[ ]  Heading Indicator .... SET

[ ]  Cowl Flaps .... OPEN

[ ]  Controls .... FREE & CORRECT

[ ]  Instruments .... CHECK

[ ]  Fuel .... FULLEST TANK

[ ]  Ram Air .... CLOSED

[ ]  Throttle .... 2000 RPM

[ ]  Mixture .... BEST POWER

[ ]  Prop .... CYCLE (3x)

[ ]  Mags .... (drop < 175, diff < 50)

[ ]  Temps / Alt / Vac / Fuel .... GREEN

[ ]  Fuel Pressure .... GREEN

[ ]  Throttle .... 1000 RPM

HOLD SHORT LINE

[ ]  Flaps .... TAKEOFF

[ ]  Trim .... ON & TAKEOFF

[ ]  Charts / Plates / Log .... READY

[ ]  COM & NAV .... FIRST FREQS

[ ]  OBS, ADF, DME .... SET

[ ]  Headset .... ON

[ ]  Flight Plan .... OPEN?

[ ]  Window & Door .... LATCH

[ ]  Time Off .... LOG

[ ]  ATC .... TOWER

CLEARED TO ACTIVE

[ ]  Boost Pump .... ON

[ ]  Landing Light .... ON

[ ]  Strobes .... ON

[ ]  Transponder .... ALT

[ ]  Pitot Heat .... ON (as req)

>1000' AGL

[ ]  Throttle .... FULL

[ ]  Prop .... 2600

[ ]  Boost Pump .... OFF

[ ]  Temps / Alt / Vac / Fuel .... GREEN

[ ]  Landing Light .... OFF

[ ]  Fuel Caps .... LOOK FOR LEAKS

CRUISE

[ ]  Throttle .... FULL

[ ]  Prop .... 2400

[ ]  Cowl Flaps .... CLOSED

[ ]  Mixture .... LEAN

[ ]  Temps / Alt / Vac / Fuel .... GREEN

[ ]  Heading Indicator .... SET

[ ]  Landing Light .... OFF

[ ]  Fuel Tank .... SWITCH?

DESCENT

[ ]  Fuel .... FULLEST TANK

[ ]  Mixture .... ENRICH

[ ]  Cowl Flaps .... CLOSED

[ ]  Ram Air .... CLOSED

[ ]  Temps / Alt / Vac / Fuel .... GREEN

[ ]  Heading Indicator .... SET

[ ]  Autopilot .... OFF

[ ]  Landing Light .... ON

CLEAR ACTIVE

[ ]  Flaps .... UP

[ ]  Boost Pump .... OFF

[ ]  Cowl Flaps .... OPEN

[ ]  Transponder .... STANDBY

[ ]  Pitot Heat .... OFF

[ ]  Strobes .... OFF

SHUT DOWN

[ ]  All Switches .... OFF

[ ]  Mixture .... IDLE

[ ]  Mags .... OFF

[ ]  Master .... OFF

[ ]  Overhead Light .... OFF

[ ]  Timer .... STOPPED

[ ]  Headset .... OFF

[ ]  GPS .... OFF

[ ]  Hobbs Time .... RECORD

[ ]  Flight Plan .... CLOSED

PRE-APPROACH

[ ]  ATIS, Approach in use

[ ]  Heading Indicator .... SET

[ ]  Altimiter .... SET

[ ]  Radios .... SET & IDENT

[ ]  DME .... SET

[ ]  Marker Beacons .... TEST & AUDIO ON

APPROACH BRIEFING

[ ]  Airport, Approach & Runway

[ ]  IAF & Procedure Turn

[ ]  FAF & Final Approach Course

[ ]  Minimums .... MDA/DA & VIS

[ ]  Circling .... ALTITUDE & NOTES

[ ]  TDZE

[ ]  MAP .... How to ID?

[ ]  Missed Approach Proc .... INITIAL

IFR REQUIRED REPORTS

[ ]  Vacating assigned altitude

[ ]  Altitude change when VFR on TOP

[ ]  Unable to climb / desc 500 FPM

[ ]  Change in TAS 5% or 10 knots

[ ]  Missed Approach

[ ]  Arrival at Clearance Limit

[ ]  Arrival / leaving Holding Fix

[ ]  Loss of any Nav or Com

[ ]  Safety, including hazardous WX

WHEN NOT IN RADAR CONTACT

[ ]  Compulsory points or fixes in clearance

[ ]  FAF inbound

[ ]  ETA change greater than 3 minutes

ICING CONDITIONS  [EMERGENCY]

[ ]  Ram Air .... OFF

[ ]  Cabin Heat .... OFF

NOTE: Leave off if engine is running rough

[ ]  Pitot Heat .... ON

[ ]  Static Air .... ALTERNATE

[ ]  Change .... HDG and/or ALTITUDE

ENGINE FAILURE or ROUGH  [EMERGENCY]

[ ]  Airspeed .... 105 MPH

[ ]  Landing Field .... SELECT

[ ]  Mixture .... CHECK

[ ]  Throttle .... ADJUST

[ ]  Boost Pump .... ON

NOTE: If no restart, then turn Boost OFF

[ ]  Fuel .... OTHER TANK

[ ]  Mags .... BOTH

AIR START PROCEDURE  [EMERGENCY]

[ ]  Propeller .... FORWARD

[ ]  Mixture .... IDLE

[ ]  Fuel Pressure .... CHECK

NOTE: Boost ON if no pressure

[ ]  Throttle .... 25% FORWARD

[ ]  Mags .... BOTH

[ ]  Mixture .... SLOWLY TO FULL RICH

NO START  [EMERGENCY]

[ ]  ELT .... ARM

[ ]  Transponder .... 7700

[ ]  Radio .... 121.5

[ ]  Seat Belts .... SECURE

[ ]  Mixture .... IDLE

[ ]  Fuel .... OFF

[ ]  Mags .... OFF

[ ]  Flaps .... AS REQ

[ ]  Gear .... UP or DOWN

[ ]  Door .... AJAR

[ ]  Master .... OFF (prior to landing)

ALTERNATOR  [EMERGENCY]

STEADY ALTERNATOR WARNING LIGHT

NOTE: Indicates high voltage due to voltage regulator exceeding limits

[ ]  Avionics Master .... OFF

[ ]  Master .... OFF, then ON

[ ]  Light on again? .... Alt Field Brkr OFF

[ ]  All non-essential electrical .... OFF

[ ]  Land .... ASAP

BLINKING ALTERNATOR WARNING LIGHT

NOTE: Indicates low voltage due to alternator malfunction, belt or breaker

[ ]  ALT FIELD BREAKER .... RESET

[ ]  Light on again?

[ ]  All non-essential electrical .... OFF

[ ]  Land .... ASAP

ENGINE FIRE – IN FLIGHT  [EMERGENCY]

[ ]  Fuel .... OFF

[ ]  Throttle .... CLOSED

[ ]  Mixture .... IDLE

[ ]  Cabin Vent & Heat .... CLOSED (fwd)

[ ]  Landing Gear .... DOWN or UP

[ ]  Wing Flaps .... EXTEND

ENGINE FIRE – GROUND  [EMERGENCY]

[ ]  Mixture .... IDLE

[ ]  Fuel .... OFF

[ ]  Master .... OFF

ELECTRICAL FIRE  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Cabin Vent .... OPEN

[ ]  Circuit Breakers .... CHECK

GEAR RETRACTION FAIL  [EMERGENCY]

NOTE: Airspeed or squat switch failed. 3 lights on: GR SAFETY BYPASS switch, GEAR DN, IN TRANSIT. Gear horn will sound.

[ ]  Gear Bypass Btn .... HOLD

NOTE: Hold until gear retracted

[ ]  Gear horn on? .... THEN

[ ]  GEAR CONT Breaker .... OFF

NOTE: Push GEAR CONT breaker back in to extend gear.

MANUAL GEAR EXTENSION  [EMERGENCY]

[ ]  Airspeed .... 125 MPH

[ ]  Gear Circuit Breaker .... OFF

[ ]  Gear Lever .... DOWN

[ ]  Handcrank Lever .... FORWARD

[ ]  Gear Crank .... CLOCKWISE

NOTE: Crank until light / floor indicator

WARNING: Do not retract gear manually

IFR LOST COMMS  [EMERGENCY]

[ ]  Squawk .... 7600

[ ]  VFR? .... Land ASAP

ROUTE

[ ]  Last assigned by ATC

[ ]  Route ATC advised to expect

ALTITUDE – HIGHEST OF

[ ]  Last assigned by ATC

[ ]  Minimum IFR altitude

[ ]  Expected altitude in clearance

LEAVE CLEARANCE LIMIT (IAF)

[ ]  At EFC time

[ ]  If no EFC given, at ETA time

LEAVE CLEARANCE LIMIT (NOT IAF)

[ ]  At EFC time

[ ]  If no EFC given proceed to IAF and begin approach at ETA time

LIGHT GUN SIGNALS

NOTE: Color Green: Ground Takeoff / Flight Land

NOTE: Flashing Green: Ground Taxi / Flight Return to land

NOTE: Red: Ground Stop / Flight Yield and circle

NOTE: Flashing Red: Ground Taxi clear of runway / Flight Airport Unsafe

NOTE: Flashing White: Ground Return to start pt. / Flight N/A

NOTE: Red / Green: Exercise extreme caution

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20J_MSE/phil_vergese_m20j.txt

Rights: licensed

Checklist hash: b8f15b6ffd8db20419430fdce5c9225a5fe475b14ce31082dd93edcdde088d22

Verify against the aircraft's own approved documentation before use.