Mooney M20K 305 Rocket — David Mazer_Mooney_305H_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Descent" item count differs between passes (5 vs 6); verify nothing was dropped.

content_gap: Section "Fuel Settings" item count differs between passes (16 vs 6); verify nothing was dropped.

content_gap: Section "Starting" item count differs between passes (36 vs 21); verify nothing was dropped.

Preflight

COCKPIT

[ ]  Landing gear .... Down

[ ]  Magnetos .... Off

[ ]  Mixture .... Idle

[ ]  Avionics master .... Off

[ ]  Master .... On

[ ]  Fuel gauges .... Check

[ ]  Lights/pitot heat .... Check

[ ]  Master .... Off

[ ]  Gascolator (R&L) .... 5 Sec each

[ ]  Brakes .... Set

RIGHT WING

[ ]  Flaps/ailerons .... Check

[ ]  Wingtip/lights .... Check

[ ]  Leading edge .... Check

[ ]  Fuel tanks .... Check

[ ]  Fuel drains (2) .... Check

[ ]  Landing gear .... Check

[ ]  Tie down/chock .... Remove

COWL

[ ]  Air inlets .... Check

[ ]  Prop .... Check

[ ]  Landing light .... Check

[ ]  Oil level (9-12 Qt) .... Check

[ ]  Landing gear .... Check

[ ]  Chock .... Remove

LEFT WING

[ ]  Leading edge .... Check

[ ]  Stall warning .... Check

[ ]  Fuel tanks .... Check

[ ]  Fuel drains (2) .... Check

[ ]  Landing gear .... Check

[ ]  Tie down/chock .... Remove

[ ]  Wingtip/lights .... Check

[ ]  Flaps/ailerons .... Check

FUSELAGE/TAIL

[ ]  Static ports .... Check

[ ]  Tie down/chock .... Remove

[ ]  Elevator/rudder .... Check

[ ]  Baggage door .... Locked

Before Starting

[ ]  Seats/Seat belts .... Secure

[ ]  Door .... Closed/latched/sealed

[ ]  Circuit breakers .... Check

[ ]  ELT .... Arm

[ ]  Avionics master .... Off

[ ]  Landing gear .... Down

[ ]  Lights/pitot heat .... Off

[ ]  Fuel pump .... Off

[ ]  Cowl flaps .... Open

[ ]  Fuel selector .... Emptiest tank

[ ]  Oxygen .... Check

Starting

HOT

[ ]  Master/Alternators .... On

[ ]  Mixture .... Lean

[ ]  Prop .... High

[ ]  Throttle .... Full open

[ ]  Prime .... 3-6 sec.

[ ]  Mixture .... Lean

[ ]  Prop area .... Clear

[ ]  Magnetos .... Start

[ ]  Mixture .... Rich

[ ]  Throttle .... Idle

COLD

[ ]  Brakes .... On

[ ]  Master/Alternators .... On

[ ]  Mixture .... Rich

[ ]  Prop .... High

[ ]  Throttle .... Full open

[ ]  Prime .... 6-12 sec.

[ ]  Throttle .... ¼ inch

[ ]  Prop area .... Clear

[ ]  Magnetos .... Start

[ ]  Throttle .... 1000 RPM

[ ]  Oil Pressure .... Check

[ ]  Engine instruments .... Normal

[ ]  Mixture .... Lean to 2.5 GPH

[ ]  Avionics master .... On

[ ]  Transponder .... Standby

[ ]  Nav lights .... On

[ ]  Other lights .... As required

[ ]  Fuel selector .... Switch tanks

[ ]  Radios .... Set

[ ]  GPS   ( expiration

[ ]  GPS .... Enter x 3, set

[ ]  Autopilot .... Test

[ ]  Altimeter .... Set

[ ]  Brakes .... Release

Run-up

[ ]  Brakes .... Set

[ ]  Door .... Inflate

[ ]  Fuel selector .... Fullest tank

[ ]  Controls .... Free and correct

[ ]  Speed brakes .... Check

[ ]  Oil Temp .... > 75 degrees

[ ]  Mixture .... Rich

[ ]  Throttle .... 1800 RPM

[ ]  Engine gauges .... Check

[ ]  Battery .... Check

[ ]  Vacuum .... 4.0-5.5

[ ]  Prop .... Cycle (3), 300 RPM

[ ]  Magnetos .... 150 max, 50 diff

[ ]  Throttle .... Idle

[ ]  Annunciator panel .... Test

[ ]  Throttle .... 1000 RPM

[ ]  Trim .... Take off

[ ]  Flaps .... Take off

[ ]  Circuit breakers .... Check

Cleared for Take-off

[ ]  Oil Temp .... > 100 degrees

[ ]  Door/window .... Closed/locked

[ ]  Seats/Seatbelts .... Secure

[ ]  Lights .... As required

[ ]  Transponder .... ALT

[ ]  Power .... 38” MP/2650 RPM

[ ]  Vr (rotate) .... 62 kts

Fuel Settings

%     SETTING          GPH

[ ]  55%

[ ]  26/2200

[ ]  13.3

[ ]  65%

[ ]  30/2200

[ ]  16.7

[ ]  72%

[ ]  31/2300

[ ]  17.5

[ ]  75%

[ ]  31/2400

[ ]  18.6

[ ]  88%

[ ]  35/2500

[ ]  21.2

Climb

[ ]  Landing gear .... Up

[ ]  VLO (retraction) .... 107 kts

[ ]  VFE (flap extension) .... 112 kts

[ ]  Flaps .... Up

[ ]  Power .... 35” MP/2500 RPM

[ ]  Mixture .... TIT 1500

[ ]  Vx (best angle) .... 91 kts

[ ]  Vy (best rate) .... 108 kts

[ ]  Cruise climb .... 135 kts

[ ]  Engine instruments .... Check

[ ]  Landing/taxi lights .... Off

Cruise

[ ]  Cowl flaps .... Closed

[ ]  Engine instruments .... Check

[ ]  Battery .... Check

[ ]  Vacuum .... 4.0-5.5

[ ]  Power .... 26-32, 2200-2400

[ ]  TIT .... 1500

[ ]  Fuel selector .... Check

[ ]  Door .... Seal

[ ]  VNE (never exceed) .... 196 kts

[ ]  VNO (max cruise) .... 160-174 kts

[ ]  VA (maneuvering) .... 101-118 kts

[ ]  VLE (gear extended) .... 165 kts

Descent

[ ]  Cowl flaps .... Closed

[ ]  Power (1 MP/min max .... 25 MP, 2200 RPM

[ ]  Mixture .... 1500 TIT

[ ]  Fuel selector .... Fullest tank

[ ]  ATIS .... Check

Landing

[ ]  Landing/taxi lights .... Check

[ ]  Power, level .... 20 MP, 2200

[ ]  Power, descent .... 15 MP, 2200

[ ]  VLO (extension) .... 140 kts

[ ]  VFE (extension) .... 112 kts

[ ]  Speed brakes .... As required

[ ]  Gas .... Fullest tank

[ ]  Undercarriage .... Down

[ ]  Mixture .... Rich

[ ]  Prop .... High

[ ]  Seatbelts .... Check

Off Active

[ ]  Cowl flaps .... Open

[ ]  Flaps .... Up

[ ]  Pitot heat .... Off

[ ]  Landing/taxi lights .... Off

[ ]  Transponder .... Standby

[ ]  Landing time .... Note

[ ]  Door seal .... Deflate

Shutdown

[ ]  Throttle .... 1000 RPM

[ ]  Idle before shutdown .... 5 min

[ ]  Lights .... Off

[ ]  Avionics master .... Off

[ ]  Mixture .... Cut-off

[ ]  Magnetos .... Off

[ ]  Alternator .... Off

[ ]  Master .... Off

Engine Failure  [EMERGENCY]

[ ]  Airspeed .... 87 kts

[ ]  Best landing option .... Select

[ ]  Checklist .... Review

[ ]  Prop .... Feather

[ ]  Throttle .... Check

[ ]  Low fuel boost .... On

[ ]  Fuel selector .... Switch tanks

[ ]  Mixture .... Check

[ ]  Magnetos .... Check

[ ]  ELT .... ARM

[ ]  Transponder .... 7700

[ ]  Radios .... 121.5

[ ]  Seatbelts .... Check

Power Off Landing  [EMERGENCY]

[ ]  Mixture .... Idle

[ ]  Fuel .... Off

[ ]  Magnetos .... Off

[ ]  Flaps .... Full when certain

[ ]  Gear .... Check

[ ]  Door .... open

[ ]  Master .... Off

Icing  [EMERGENCY]

[ ]  Alternate Air .... On

[ ]  Cabin heat .... On

[ ]  Defrost .... On

[ ]  Pitot heat .... On

[ ]  Mixture .... Check

[ ]  Cowl flaps .... Open

Fire in Flight  [EMERGENCY]

[ ]  Fuel .... Off

[ ]  Throttle .... Off

[ ]  Mixture .... Cut off

[ ]  Magnetos .... Off

[ ]  Cabin vent/heat .... Off

[ ]  Cowl flaps .... Closed

Fire on Ground  [EMERGENCY]

[ ]  Start .... 1500 RPM

[ ]  Shut down .... 3 minutes

[ ]  Mixture .... Cut off

[ ]  Throttle .... Open

[ ]  Fuel/mags/master .... Off

Electrical Fire  [EMERGENCY]

[ ]  Master .... Off

[ ]  Alternator .... Off

[ ]  Cabin vent .... Open

[ ]  Cabin heat .... Closed

[ ]  Circuit breakers .... Check

Gear Extension Failure  [EMERGENCY]

[ ]  Airspeed .... < 140 kts

[ ]  Gear circuit breakers .... Pull

[ ]  Gear selector .... Down

[ ]  Red handle .... Forward

[ ]  T-handle .... 12-20

[ ]  Gear circuit breakers .... Reset

Alternator Failure  [EMERGENCY]

[ ]  Avionics master .... Off

[ ]  Master .... Off then on

[ ]  Circuit breakers .... Check

[ ]  Gear selector .... Down

Door Open  [EMERGENCY]

[ ]  Airspeed .... 95 kts

[ ]  Right bank/Left rudder .... Check

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20K 305 Rocket/David Mazer_Mooney_305H_Checklist.txt

Rights: licensed

Checklist hash: f21ad55616e85198e9b05929277da29f20aeb5ca0a11c70c242d8c7a4c05d8e4

Verify against the aircraft's own approved documentation before use.