Mooney M20R Ovation 2 — Johnathan_Celso_MooneyM20R_Ovation2_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before Takeoff" item count differs between passes (18 vs 19); verify nothing was dropped.

content_gap: Section "Descent" item count differs between passes (4 vs 5); verify nothing was dropped.

content_gap: Section "Landing" item count differs between passes (19 vs 17); verify nothing was dropped.

content_gap: Section "Securing Aircraft" item count differs between passes (10 vs 11); verify nothing was dropped.

content_gap: Section "Starting Engines" item count differs between passes (20 vs 21); verify nothing was dropped.

Preflight Inspection

CABIN

[ ]  A.R.R.O.W. .... Check

[ ]  Control Lock .... Remove

[ ]  Alternate Static Air .... Off

[ ]  Gear Selector .... Down

[ ]  Ignition Switch .... Off

[ ]  Master Switch .... On

[ ]  Fuel Quantity .... Check

[ ]  Battery Voltage .... Check

[ ]  Standby Vacuum .... Check

[ ]  Pitot Heat .... Check

[ ]  Trim .... Neutral

[ ]  Flaps .... Down/Check

[ ]  Master Switch .... Off

[ ]  Fire Extinguisher .... Charged/Secure

FUSELAGE/TAIL

[ ]  Skin/Windows .... Check

[ ]  Antennas .... Check

[ ]  Beacon/Strobe Lights .... Check

[ ]  Access Panels .... Secure

[ ]  Static Ports .... Check

[ ]  Static Drain Valve .... Push

[ ]  Fresh Air Vent .... Check

[ ]  Elevator/Rudder .... Inspect

[ ]  Tiedown .... Remove

LEFT WING

[ ]  Aileron/Flap .... Inspect

[ ]  Nav/Strobe/Recog Lights .... Check

[ ]  Pitot Tube Cover .... Check

[ ]  Pitot Tube .... Check

[ ]  Pitot Drain Valve .... Push

[ ]  Landing/Taxi Lights .... Check

[ ]  Stall Warning Vane .... Check

[ ]  Main Gear/Wheel .... Check

[ ]  Fuel Sumps .... Test

[ ]  Fuel Tank/Cap/Vent .... Check

[ ]  Wheel Chocks/Tiedown .... Remove

NOSE

[ ]  Air Inlets .... Check

[ ]  Engine Oil/Cap .... Check

[ ]  Engine Cowl .... Secure

[ ]  Propeller/Spinner .... Check

[ ]  Air Intakes .... Check

[ ]  Exhaust Pipes .... Check

[ ]  Nose Gear/Wheel .... Check

RIGHT WING

[ ]  Wheel Chocks/Tiedown .... Remove

[ ]  Fuel Sumps .... Test

[ ]  Fuel Tank/Cap/Vent .... Check

[ ]  Main Gear/Wheel .... Check

[ ]  Landing/Taxi Lights .... Check

[ ]  Nav/Strobe/Recog Lights .... Check

[ ]  Aileron/Flap .... Inspect

[ ]  Baggage Door .... Close/Lock

Before Starting Engine

[ ]  Preflight Inspection Completed

[ ]  Passengers .... Briefed

[ ]  Seatbelts .... Fastened

[ ]  Cabin Door .... Closed/Locked

[ ]  Parking Brake .... Set

[ ]  Fuel Selector .... Right Tank

[ ]  Gascolator .... Drain (5 Sec)

[ ]  Fuel Selector .... Left Tank

[ ]  Gascolator .... Drain (5 Sec)

[ ]  Gear Selector .... Down

[ ]  Circuit Breakers .... In

[ ]  Electrical Equipment .... Off

[ ]  Radio Master Switch .... Off

[ ]  Ignition Switch .... Off

[ ]  Alternate Air .... Off

[ ]  Propeller .... Full High

[ ]  Mixture .... Idle Cutoff

[ ]  Master Switch .... On

[ ]  Alternator .... On

[ ]  Rotating Beacon .... On

Starting Engines

NORMAL

[ ]  Throttle .... Open 1/4”

[ ]  Prime (Omit If Hot)

[ ]  Mixture .... Full Rich

[ ]  Boost Pump .... On (5 Sec)

[ ]  Mixture .... Idle Cutoff

[ ]  Propeller Area .... Clear

[ ]  Ignition Switch .... Both

[ ]  Starter .... Engage

[ ]  Mixture .... Enrich

[ ]  Throttle .... 1000 RPM

FLOODED

[ ]  Throttle .... Full Open

[ ]  Mixture .... Idle Cutoff

[ ]  Propeller Area .... Clear

[ ]  Ignition Switch .... Both

[ ]  Starter .... Engage

[ ]  Throttle .... Reduce

[ ]  Mixture .... Enrich

[ ]  Throttle .... 1000 RPM

Before Taxi

[ ]  Oil Pressure .... Check

[ ]  Mixture .... Lean

[ ]  Flaps .... Up

[ ]  Annunciator Panel .... Test

[ ]  ELT .... Arm

[ ]  Radio Master Switch .... On

[ ]  Transponder .... Standby

[ ]  Avionics .... Test/Set

[ ]  Clearance .... Obtain

Taxi

[ ]  Parking Brake .... Release

[ ]  Nav/Taxi Lights .... As Required

[ ]  Brakes/Steering .... Check

[ ]  Flight Instruments .... Check

Before Takeoff

[ ]  Parking Brake .... Set

[ ]  Fuel Selector .... Fullest Tank

[ ]  Mixture .... Full Rich

[ ]  Throttle .... 2000 RPM

[ ]  Magnetos .... Check

[ ]  Propeller .... Cycle

[ ]  Vacuum/Ammeter .... Check

[ ]  Engine Gauges .... Check

[ ]  Annunciator Panel .... Check

[ ]  Throttle .... 1000 RPM

[ ]  Flight Instruments .... Set

[ ]  Flight Controls .... Free/Correct

[ ]  Trim/Flaps .... Set Takeoff

[ ]  Boost Pump .... On

[ ]  Strobe/Recog Lights .... On

[ ]  Landing Lights .... As Required

[ ]  Transponder .... On/Altitude

[ ]  Parking Brake .... Release

Takeoff

NORMAL

[ ]  Throttle .... Full Open

[ ]  Rotate .... 67 KIAS

[ ]  Gear Selector .... Up

[ ]  Climb Speed .... 105 KIAS

[ ]  Flaps .... Up

[ ]  Boost Pump/Excess Lights .... Off

SHORT FIELD

[ ]  Brakes .... Apply

[ ]  Throttle .... Full Open

[ ]  Rotate .... 67 KIAS

[ ]  Climb Speed .... 85 KIAS

[ ]  Obstacles .... Clear

[ ]  Gear Selector .... Up

[ ]  Climb Speed .... 105 KIAS

[ ]  Flaps .... Up

[ ]  Boost Pump/Excess Lights .... Off

Climb

[ ]  Power .... Max MP/2500 RPM

[ ]  Mixture .... 100° Rich EGT

[ ]  Climb Speed .... 120 KIAS

[ ]  Trim .... Set

Cruise

[ ]  Power .... As Desired

[ ]  Mixture .... 50° Rich EGT

[ ]  Trim .... Set

[ ]  Fuel Selector .... Switch Tanks

Descent

[ ]  Power .... 20” MP/2000 RPM

[ ]  Mixture .... 25° Rich EGT

[ ]  Speed Brakes .... As Required

[ ]  Trim .... Set

Before Landing

[ ]  Seatbelts .... Fastened

[ ]  Fuel Selector .... Fullest Tank

[ ]  Parking Brake .... Released

[ ]  Propeller .... Full High

[ ]  Mixture .... Full Rich

[ ]  Flaps .... As Desired

[ ]  Approach Speed .... 90-100 KIAS

[ ]  Trim .... Set

[ ]  Boost Pump .... On

[ ]  Landing/Taxi Lights .... As Required

Landing

NORMAL

[ ]  Flaps .... Down

[ ]  Gear Selector .... Down/Check

[ ]  Approach Speed .... 80 KIAS

[ ]  Trim .... Set

[ ]  Touchdown .... Main Wheels First

[ ]  Throttle .... Idle

[ ]  Brakes .... Apply

[ ]  Flaps .... Up

SHORT FIELD

[ ]  Flaps .... Down

[ ]  Gear Selector .... Down/Check

[ ]  Speed Brakes .... Extended

[ ]  Approach Speed .... 75 KIAS

[ ]  Trim .... Set

[ ]  Touchdown .... Main Wheels First

[ ]  Throttle .... Idle

[ ]  Brakes .... Apply

[ ]  Flaps .... Up

Balked

[ ]  Throttle .... Full Open

[ ]  Positive Climb Rate .... Check

[ ]  Speed Brakes .... Retracted

[ ]  Flaps .... Set Takeoff

[ ]  Climb Speed .... 85 KIAS

[ ]  Obstacles .... Clear

[ ]  Gear Selector .... Up

[ ]  Climb Speed .... 105 KIAS

[ ]  Flaps .... Up

After Landing

[ ]  Landing/Strobe/Recog Lights .... Off

[ ]  Speed Brakes .... Retract

[ ]  Boost Pump .... Off

[ ]  Flaps .... Up

[ ]  Transponder .... Standby

Securing Aircraft

[ ]  Brakes .... Apply

[ ]  ELT .... Disarm

[ ]  Electrical Equipment .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... Idle Cutoff

[ ]  Engine .... Shutdown

[ ]  Ignition Switch .... Off

[ ]  Master Switch .... Off

[ ]  Alternator .... Off

[ ]  Locks/Chocks/Tiedowns .... Install

Close Flight Plan

[ ]  Close Flight Plan

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Mooney/M20R Ovation 2/Johnathan_Celso_MooneyM20R_Ovation2_Checklist.txt

Rights: licensed

Checklist hash: 7aea5def0ff7743a35ff32095737ca069360ccdc88867c86787d3cfe89e4cc74

Verify against the aircraft's own approved documentation before use.