Piper PA-28-140 — Cherokee140BChecklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

PREFLIGHT

CABIN

[ ]  Papers .... A.R.O.W.

[ ]  Parking Brake .... SET

[ ]  Seat Belt Securing Control Wheel .... REMOVE

[ ]  Mags .... OFF

[ ]  Avionics .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Trim .... TAKEOFF

[ ]  Prop .... CLEAR

[ ]  Master .... ON

[ ]  Fuel Gauges .... TRUE

[ ]  Flaps .... EXTEND

[ ]  Lights .... INTERIOR / EXTERIOR

[ ]  Stall Indicator .... TEST

[ ]  Master .... OFF

[ ]  Fuel Selector .... CHECK EACH POSITION, SET TO EMPTIEST TANK

RIGHT WING

[ ]  Flap .... CHECK

[ ]  Aileron .... CHECK

[ ]  Navigation Light .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Fuel Vent .... CHECK

[ ]  Fuel Quantity .... CHECK VISUALLY

[ ]  Fuel Filler Cap .... SECURE

[ ]  Right Fuel Tank Sump .... DRAIN

[ ]  Chock .... REMOVE

[ ]  Tire .... CHECK (24 PSI)

[ ]  Brake Assembly .... CHECK

[ ]  Main Strut .... APPROX. 4.5" EXTENSION

NOSE

[ ]  Right Engine Compartment .... CHECK

[ ]  Oil Level .... 7 QTS (under 3 HR flight), 8 QTS (extended flight)

[ ]  Oil Cap .... SECURE

[ ]  Right Engine Cowling .... SECURE

[ ]  Exhaust System .... CHECK

[ ]  Chock .... REMOVE

[ ]  Nose Strut .... APPROX. 3.25" EXTENSION

[ ]  Nose Tire .... CHECK (24 PSI)

[ ]  Landing Light .... CHECK

[ ]  Engine Air Inlets .... CLEAR

[ ]  Alternator Belt Tension .... CHECK

[ ]  Prop/Spinner .... CHECK

[ ]  Left Engine Compartment .... CHECK

[ ]  Left Engine Cowling .... SECURE

[ ]  Fuel Strainer .... DRAIN

[ ]  Windshield .... CHECK

LEFT WING

[ ]  Chock .... REMOVE

[ ]  Tire .... CHECK (24 PSI)

[ ]  Brake Assembly .... CHECK

[ ]  Main Strut .... APPROX. 4.5" EXTENSION

[ ]  Left Fuel Tank Sump .... DRAIN

[ ]  Fuel Vent .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Pitot/Static Tube .... CLEAR

[ ]  Fuel Quantity .... CHECK VISUALLY

[ ]  Fuel Filler Cap .... SECURE

[ ]  Navigation Light .... CHECK

[ ]  Aileron .... CHECK

[ ]  Flap .... CHECK

EMPENNAGE

[ ]  Left Stabilator .... CHECK

[ ]  Tiedown .... DISCONNECT

[ ]  Trim Tab .... CHECK

[ ]  Rudder .... CHECK

[ ]  Navigation Light .... SECURE

[ ]  Right Stabilator .... CHECK

[ ]  Antennas .... CHECK

START

[ ]  Hobbs/Tach Time .... RECORDED

[ ]  Passenger Brief .... COMPLETE

[ ]  Seat Track/Back .... LOCK

[ ]  Seat Belt /Harness .... FASTEN

[ ]  Brakes .... SET

[ ]  Beacon .... ON

[ ]  Navigation Lights .... AS REQUIRED

[ ]  Mixture .... RICH

[ ]  Throttle .... OPEN 1/4"

[ ]  Carb Heat .... OFF

[ ]  Prime .... AS REQUIRED

[ ]  Prop .... CLEAR

[ ]  Master .... ON

[ ]  Fuel Pump .... ON

[ ]  Mags .... START

[ ]  Throttle .... 1,000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Fuel Pump .... OFF

[ ]  Mixture .... LEAN

PRE-TAXI

[ ]  Flaps .... UP

[ ]  Cabin Heat & Air .... AS DESIRED

[ ]  Avionics .... ON / SET

[ ]  Transponder .... CODE / STANDBY

[ ]  Clock .... SET

[ ]  Airspeed Indicator .... CHECK

[ ]  Attitude Indicator .... SET

[ ]  Heading Indicator .... SET

[ ]  VSI .... CHECK

[ ]  ATIS/AWOS .... COPY

[ ]  Altimeter .... SET / CHECK

[ ]  Landing Light .... AS REQUIRED

[ ]  Fuel Selector .... SWITCH TO FULLEST TANK

TAXI

[ ]  Brakes .... TEST

[ ]  Attitude Indicator .... CHECK

[ ]  Turn-and-Bank Indicator .... CHECK

[ ]  Heading Indicator .... CHECK

[ ]  Magnetic Compass .... CHECK

RUN-UP

[ ]  Brakes .... SET

[ ]  Mixture .... BEST POWER

[ ]  Primer .... IN & LOCKED

[ ]  Throttle .... 1,800 RPM

[ ]  Mags (R & L) .... CHECK (175 RPM max drop, 50 RPM max differential)

[ ]  Carb Heat .... TEST

[ ]  Vacuum .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Oil Pressure .... CHECK

[ ]  Oil Temperature .... CHECK

[ ]  Throttle .... 1,000 RPM

TAKEOFF

[ ]  Heading Indicator .... SET

[ ]  Avionics .... SET

[ ]  Engine Gauges .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Fuel Pump .... ON

[ ]  Carb Heat .... OFF

[ ]  Mixture .... BEST POWER

[ ]  Trim .... TAKEOFF

[ ]  Flaps .... SET (0-25°)

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Door .... LATCHED

CLEARED FOR TAKEOFF

[ ]  Landing Light .... ON

[ ]  Transponder .... ALTITUDE

[ ]  Throttle .... FULL

AFTER TAKEOFF

[ ]  Flaps .... UP

[ ]  Fuel Pump .... OFF

[ ]  Engine Gauges .... CHECK

[ ]  Landing Light .... AS DESIRED

CRUISE

[ ]  Landing Light .... OFF

[ ]  Throttle .... SET

[ ]  Mixture .... LEAN

[ ]  Engine Gauges .... CHECK

[ ]  Heading Indicator .... SET

[ ]  Fuel Selector .... AS REQUIRED

APPROACH

[ ]  Mixture .... RICHEN AS REQUIRED

[ ]  Fuel Selector .... PROPER TANK

[ ]  Carb Heat .... AS REQUIRED

[ ]  ATIS/AWOS .... COPY

[ ]  Altimeter .... SET

[ ]  Avionics .... SET

[ ]  Heading Indicator .... SET

[ ]  Engine Gauges .... CHECK

[ ]  Brakes .... PEDAL TEST

[ ]  Seatbelt/Harness .... FASTENED

[ ]  Landing Light .... ON

LANDING

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector .... PROPER TANK

[ ]  Carb Heat .... AS REQUIRED

[ ]  Mixture .... BEST POWER

[ ]  Flaps .... AS DESIRED

AFTER LANDING

[ ]  Carb Heat .... OFF

[ ]  Mixture .... LEAN

[ ]  Flaps .... UP

[ ]  Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Transponder .... STANDBY

SHUTDOWN

[ ]  Avionics .... OFF

[ ]  Throttle .... 1,000 RPM

[ ]  Mags .... GROUNDING CHECK

[ ]  Mixture .... FULL LEAN

[ ]  Prop .... CHECK STOPPED

[ ]  Mags .... OFF

[ ]  Master .... OFF

[ ]  Lights .... OFF

SECURING

[ ]  Hobbs/Tach Time .... RECORDED

[ ]  Seat Belt Securing Control Wheel .... INSTALL

[ ]  Chocks .... INSTALL

[ ]  Tiedowns .... SECURE

[ ]  Door/Window .... CLOSED

FLOODED ENGINE  [ABNORMAL]

FLOODED ENGINE

[ ]  Mixture .... FULL LEAN

[ ]  Throttle .... FULL OPEN

[ ]  Mags .... START

WHEN ENGINE FIRES:

[ ]  Mixture .... RICH

[ ]  Throttle .... 1,000 RPM

GROUND BURNOUT  [ABNORMAL]

GROUND BURNOUT

[ ]  Mixture .... RICH

[ ]  Throttle .... 2,000 RPM

[ ]  Mixture .... LEAN TO 50 RPM DROP BELOW BEST POWER

AFTER 1 MINUTE:

[ ]  Mixture .... RICH

[ ]  Mags .... RECHECK

RADIO FAILURE  [ABNORMAL]

RADIO FAILURE

[ ]  Volume .... CHECK

[ ]  Headset Connection .... CHECK

[ ]  Audio Controls .... CHECK

[ ]  Circuit Breakers .... CHECK

[ ]  Speaker .... ON

[ ]  Hand-Held Microphone .... CHECK

[ ]  Frequency .... SWITCH

IF IFR (OR VFR IN CLASS B, C, D AIRSPACE):

[ ]  Transponder .... 7600

NOTE: If failure occurs in VFR conditions, or VFR conditions are encountered after failure, continue flight under VFR and land as soon as practicable.

VFR LANDING AT A CONTROLLED FIELD:

[ ]  Remain outside or above Class D airspace until traffic flow determined. .... CHECK

[ ]  Transmit intentions "in the blind" advising tower of aircraft type, position, altitude, and intention to land. .... CHECK

[ ]  Enter pattern, report position "in the blind", and watch for light signals from tower. .... CHECK

NOTE: If failure occurs in IFR conditions, continue flight according to the following:

ROUTE TO FLY (IN ORDER): (AVE. F)

[ ]  A – Assigned .... CHECK

[ ]  V – Vectored .... CHECK

[ ]  E – Expected .... CHECK

[ ]  F – Filed .... CHECK

ALTITUDE (HIGHEST OF FOR THE ROUTE SEGMENT BEING FLOWN): (MEA)

[ ]  M – Minimum IFR Altitude .... CHECK

[ ]  E – Expected as Advised by ATC .... CHECK

[ ]  A – Assigned by ATC .... CHECK

DESCENT FOR APPROACH:

NOTE: From enroute altitude upon reaching the IAF but not before:

[ ]  Expect-further-clearance time (if given) .... CHECK

[ ]  ETA as calculated from filed ETE .... CHECK

ATC LIGHT SIGNALS

[ ]  Steady Green - On Deck .... Cleared for Takeoff

[ ]  Steady Green - Airborne .... Cleared to Land

[ ]  Flashing Green - On Deck .... Cleared for Taxi

[ ]  Flashing Green - Airborne .... Return for Landing (to be followed by steady green)

[ ]  Steady Red - On Deck .... STOP

[ ]  Steady Red - Airborne .... Give way to other aircraft and continue circling

[ ]  Flashing Red - On Deck .... Taxi clear of runway in use

[ ]  Flashing Red - Airborne .... Airport Unsafe – DO NOT LAND

[ ]  Flashing White - On Deck .... Return to starting point on airport

[ ]  Flashing White - Airborne .... Not Applicable

[ ]  Alternating Red and Green - On Deck .... Exercise extreme caution

[ ]  Alternating Red and Green - Airborne .... Exercise extreme caution

NOTE: Acknowledge all light signals by flashing landing light and/or rocking wings.

ABORT  [EMERGENCY]

ABORT

[ ]  Throttle .... IDLE

[ ]  Brakes .... AS REQUIRED

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF  [EMERGENCY]

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

[ ]  Airspeed .... 83 MPH (flaps UP)

[ ]  Mixture .... FULL LEAN

[ ]  Fuel Selector .... OFF

[ ]  Flaps .... DOWN (70 MPH glide)

[ ]  Master & Mags .... OFF

COMPLETE ENGINE FAILURE DURING FLIGHT  [EMERGENCY]

COMPLETE ENGINE FAILURE DURING FLIGHT

[ ]  Airspeed .... 83 MPH (flaps UP)   (Glide Ratio = 1.9 SM/1,000’ AGL)

IF AIRSTART WARRANTED:

[ ]  Carb Heat .... ON

[ ]  Mixture .... RICH

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector .... CHECK / SWITCH

[ ]  Primer .... IN & LOCKED

[ ]  Mags .... CHECK ALL

[ ]  Master .... ON

FORCED LANDING  [EMERGENCY]

FORCED LANDING

[ ]  Airspeed .... 83 MPH (flaps UP)

[ ]  Squawk .... 7700

[ ]  Emergency .... DECLARE

[ ]  ELT .... ON

[ ]  Mixture .... FULL LEAN

[ ]  Fuel Selector .... OFF

[ ]  Seatbelt/Harness .... FASTENED

[ ]  Flaps .... AS REQUIRED (70 MPH with flaps full)

[ ]  Master & Mags .... OFF

[ ]  Door .... UNLATCH

ENGINE ROUGHNESS  [EMERGENCY]

ENGINE ROUGHNESS

[ ]  Carb Heat .... ON

IF ROUGHNESS CONTINUES PAST 1 MIN:

[ ]  Carb Heat .... OFF

[ ]  Mixture .... ADJUST FOR MAX SMOOTHNESS

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector .... SWITCH

[ ]  Primer .... IN & LOCKED

[ ]  Engine Gauges .... CHECK

[ ]  Mags .... L then R then BOTH

[ ]  Mags .... SMOOTHEST SETTING

[ ]  Power .... REDUCE

[ ]  Mixture .... FULL RICH

[ ]  LAND AT NEAREST AIRPORT .... EXECUTE

HIGH OIL TEMPERATURE  [EMERGENCY]

HIGH OIL TEMPERATURE

[ ]  Power .... REDUCE

[ ]  Mixture .... RICH

[ ]  Airspeed .... INCREASE

IF HIGH TEMP PERSISTS:

[ ]  LAND AT NEAREST AIRPORT .... EXECUTE

LOSS OF OIL PRESSURE  [EMERGENCY]

LOSS OF OIL PRESSURE

[ ]  Oil Temp .... CHECK

[ ]  Power .... REDUCE

IF OIL TEMP SHOWS RISE, ENGINE FAILURE IMMINENT:

[ ]  Forced Landing .... EXECUTE

IF OIL TEMP DOES NOT SHOW RISE:

[ ]  LAND AT NEAREST AIRPORT .... EXECUTE

LOSS OF FUEL PRESSURE  [EMERGENCY]

LOSS OF FUEL PRESSURE

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector .... SWITCH

ENGINE FIRE ON GROUND  [EMERGENCY]

ENGINE FIRE ON GROUND

[ ]  Starter .... CRANK

[ ]  Mixture .... FULL LEAN

[ ]  Throttle .... OPEN

[ ]  Fuel Pump .... OFF

[ ]  Fuel Selector .... OFF

ENGINE FIRE IN FLIGHT  [EMERGENCY]

ENGINE FIRE IN FLIGHT

[ ]  Mixture .... FULL LEAN

[ ]  Fuel Selector .... OFF

[ ]  Fuel Pump .... OFF

[ ]  Master & Mags .... OFF

[ ]  Heat & Defrost .... OFF

[ ]  Airspeed .... 105 MPH

[ ]  Forced Landing .... EXECUTE

ELECTRICAL FIRE  [EMERGENCY]

ELECTRICAL FIRE

[ ]  Master .... OFF

WING FIRE  [EMERGENCY]

WING FIRE

[ ]  Navigation Lights .... OFF

[ ]  Sideslip .... AS REQUIRED

[ ]  Forced Landing .... EXECUTE

SMOKE AND FUMES IN COCKPIT  [EMERGENCY]

SMOKE AND FUMES IN COCKPIT

[ ]  Vents .... OPEN

[ ]  Heater .... OFF

HIGH AMMETER READING  [EMERGENCY]

HIGH AMMETER READING

[ ]  Master .... OFF

ZERO AMMETER READING  [EMERGENCY]

ZERO AMMETER READING

[ ]  Electrical Load .... REDUCE

[ ]  Alternator Circuit Breakers .... CHECK / RESET

[ ]  Alternator Switch .... OFF (1 min) then ON

IF AMMETER READING STILL ZERO:

[ ]  Land as soon as practicable .... EXECUTE

VACUUM SYSTEM FAILURE  [EMERGENCY]

VACUUM SYSTEM FAILURE

IF VMC:

[ ]  Attitude Indicator & Heading Indicator .... COVER

[ ]  Maintain VMC .... EXECUTE

IF IMC:

[ ]  Attitude Indicator & Heading Indicator .... COVER

[ ]  Fly using Turn-and-Bank, Altimeter, and Airspeed Indicator .... EXECUTE

[ ]  Declare EMERGENCY .... EXECUTE

INOPERATIVE TRIM  [EMERGENCY]

INOPERATIVE TRIM

[ ]  Flaps .... AS REQUIRED

[ ]  Power .... AS REQUIRED

FLAP MALFUNCTION  [EMERGENCY]

FLAP MALFUNCTION

[ ]  Flaps .... LEAVE WHERE SET

STRUCTURAL ICING  [EMERGENCY]

STRUCTURAL ICING

[ ]  Carb Heat .... ON

[ ]  Throttle .... MAX RPM

[ ]  Mixture .... BEST POWER

[ ]  Heat & Defrost .... MAX

[ ]  EXIT ICING CONDITIONS .... EXECUTE

IF LANDING WITH STRUCTURAL ICING:

[ ]  Flaps .... LEAVE UP

DOOR OPENING IN FLIGHT  [EMERGENCY]

DOOR OPENING IN FLIGHT

[ ]  FLY THE AIRPLANE [MEMORY] .... EXECUTE

[ ]  Airspeed .... 89 KIAS

[ ]  Vents .... CLOSE

[ ]  Storm Window .... OPEN

IF ONLY UPPER LATCH OPEN:

[ ]  Upper Latch .... LATCH

IF ONLY SIDE LATCH IS OPEN:

[ ]  Arm Rest .... PULL WHILE LATCHING

IF BOTH LATCHES ARE OPEN:

[ ]  Side Latch .... LATCH

[ ]  Top Latch .... LATCH

LANDING WITH A FLAT MAIN TIRE  [EMERGENCY]

LANDING WITH A FLAT MAIN TIRE

[ ]  Approach .... NORMAL

[ ]  Flaps .... 40°

[ ]  Touchdown .... GOOD TIRE FIRST

[ ]  Hold airplane off flat tire as long as possible .... EXECUTE

[ ]  Maintain directional control using good-tire brake .... EXECUTE

LANDING WITH A FLAT NOSE TIRE  [EMERGENCY]

LANDING WITH A FLAT NOSE TIRE

[ ]  Approach .... NORMAL

[ ]  Flaps .... AS REQUIRED

[ ]  Touchdown .... ON MAINS

[ ]  Hold nosewheel off ground as long as possible .... EXECUTE

WHEN NOSEWHEEL TOUCHES DOWN:

[ ]  FULL UP ELEVATOR .... EXECUTE

SPIN RECOVERY  [EMERGENCY]

SPIN RECOVERY

NOTE: Memory Aid: PARE

[ ]  Power [MEMORY] .... Idle

NOTE: (Power)

[ ]  Ailerons [MEMORY] .... Neutral

NOTE: (Ailerons)

[ ]  Rudder [MEMORY] .... Full opposite rotation direction

NOTE: (Rudder)

[ ]  Yoke [MEMORY] .... Full forward with neutral ailerons

NOTE: (Elevator)

[ ]  Rudder .... Neutral when rotation stops

[ ]  Yoke .... Pull smoothly from dive

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-140/Cherokee140BChecklist.txt

Rights: licensed

Checklist hash: 4442a8d8faae7d1603941cf903247bb017b06c100df8a2b3d3cbb2433b63c00a

Verify against the aircraft's own approved documentation before use.