Piper PA-28-140 — Jacob_Esparza_N6693W_Operating_Checklist_PA_28

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Albuquerque Sunport Frequencies" item count differs between passes (5 vs 3); verify nothing was dropped.

Preflight Inspection

INTERIOR

[ ]  Control Lock .... REMOVE

[ ]  A.R.R.O.W Documents .... ON BOARD

[ ]  Maintenance Records .... CHECK

[ ]  Magnetos .... OFF

[ ]  Mixture .... LEAN

[ ]  Avionics .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Master Switch .... ON

[ ]  Beacon .... ON

[ ]  Nav. Lights .... ON

[ ]  Landing Lights .... ON

[ ]  Fuel Gauges .... CHECK

EXTERIOR

[ ]  Go Outside

[ ]  Beacon .... CHECK

[ ]  Nav Light (Right) .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Pitot Cover .... REMOVE

[ ]  Nav Light (Left) .... CHECK

[ ]  Come Back Inside

[ ]  Beacon .... OFF

[ ]  Nav Lights .... OFF

[ ]  Landing Light .... OFF

[ ]  Master Switch .... OFF

[ ]  Flaps .... DOWN

RIGHT WING

[ ]  Flap .... CHECK

[ ]  Aileron .... CHECK

[ ]  Wing Tip .... CHECK

[ ]  Leading Edge .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Fuel Sample .... AQUIRE

[ ]  Fuel Level .... VISUALLY CHECK

[ ]  Fuel Vent .... CHECK

[ ]  Right Main Gear .... CHECK

NOSE

[ ]  Windshield .... CLEAN

[ ]  Oil Level .... CHECK

[ ]  Belt .... CHECK

[ ]  Exhaust .... CHECK

[ ]  Right Cowling .... SECURE

[ ]  Spinner .... CHECK

[ ]  Propeller .... CHECK

[ ]  Air Intake .... CHECK

[ ]  Brake Fluid .... CHECK

[ ]  Left Cowling .... SECURE

[ ]  Nose Gear .... CHECK

[ ]  Chocks .... REMOVE

LEFT WING

[ ]  Left Main Gear .... CHECK

[ ]  Fuel Sample .... AQUIRE

[ ]  Fuel Vent .... CHECK

[ ]  Fuel Level .... VISUALLY CHECK

[ ]  Leading Edge .... CHECK

[ ]  Pitot Tube .... CHECK

[ ]  Tie Downs .... REMOVED

[ ]  Wing Tip .... CHECK

[ ]  Aileron .... CHECK

[ ]  Flap .... CHECK

[ ]  Left Fuselage .... CHECK

[ ]  Nav/Comm Antennas .... SECURE

EMPENNAGE

[ ]  Stabilizer .... CHECK

[ ]  Trim Tab .... CHECK

[ ]  Rudder .... CHECK

[ ]  Tie Down .... REMOVED

[ ]  Right Fuselage .... CHECK

Before Engine Start

[ ]  Preflight Inspection .... COMPLETE

[ ]  Passenger Briefing .... COMPLETE

[ ]  Seats .... ADJUST AND LOCK

[ ]  Electrical Equipment .... OFF

[ ]  Circuit Breakers .... CHECK IN

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Parking Brake .... SET

Engine Start

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... OPEN ¼”

[ ]  Carburetor Heat .... OFF (Pushed In)

[ ]  Primer .... CYCLE 3-4 TIMES

[ ]  Primer .... LOCKED

[ ]  Master Switch .... ON

[ ]  Fuel Pump .... ON

[ ]  Verify fuel pressure then turn off fuel pump

[ ]  Anti Collision Light .... ON

[ ]  Key .... VERIFY START POSITION

[ ]  Propeller Area .... CLEAR

[ ]  Brakes .... HOLD

[ ]  Starter .... ENGAGE

[ ]  Throttle .... 800 R.P.M

[ ]  Oil Pressure .... CHECK

[ ]  Fuel Pressure .... CHECK

[ ]  Ammeter .... CHECK

After Engine Start

[ ]  Avionics .... ON

[ ]  Flaps .... CHECK TRAVEL (Down then Up)

[ ]  Weather (ATIS, AWOS, ASOS) .... CHECK

[ ]  Flight Instruments .... CHECK & SET

[ ]  ATC Clearance (If Required) .... REQUEST

[ ]  Transponder .... SET CODE, VERIFY STBY

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Brakes .... RELEASE & CHECK

[ ]  Lights .... AS REQUIRED

Run Up

[ ]  Cabin Door .... CLOSED & LATCHED

[ ]  Seats and Belts .... SECURE

[ ]  Elevator Trim .... SET FOR TAKEOFF

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Mixture .... FULL RICH

[ ]  Parking Brake/Brakes .... SET & HOLD

[ ]  Throttle .... 1800 R.P.M

[ ]  Magnetos .... CHECK

NOTE: Max drop 175 R.P.M, max difference 50 R.P.M

[ ]  Carburetor Heat .... ON

NOTE: Check for R.P.M drop of about 100 R.P.M and stays there

[ ]  Carburetor Heat .... OFF

NOTE: Verify R.P.M returns to 1800 R.P.M

[ ]  Mixture .... SET FOR TAKEOFF

Run Up (Cont’d)

[ ]  Fuel Pressure .... CHECK

[ ]  Oil Pressure .... CHECK

[ ]  Oil Temperature .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Suction/Vacuum .... 4-5”Hg

[ ]  Throttle .... 1000 R.P.M

[ ]  Flight Instruments .... CHECK & SET

[ ]  Avionics .... SET

Before Takeoff

[ ]  Fuel Pump .... VERIFY ON

[ ]  Fuel Selector Valve .... VERIFY FULLEST TANK

[ ]  Nav. Lights .... AS REQUIRED

[ ]  Landing Light .... ON

[ ]  Beacon .... ON

[ ]  Window .... CLOSED

[ ]  Flaps .... AS DESIRED

[ ]  Takeoff Clearance (if required) .... REQUEST

[ ]  Transponder .... SET ALT

[ ]  Parking Brake/Brakes .... RELEASE

Albuquerque Sunport Frequencies

[ ]  ATIS .... 118.00

[ ]  Tower .... 120.30

[ ]  Clearance .... 119.20

[ ]  Departure .... 123.90 or 127.40

[ ]  Ground .... 121.90

Normal Takeoff

[ ]  Throttle .... SMOOTHLY ADVANCE TO FULL POWER

[ ]  Airspeed Indicator .... ALIVE & WELL

[ ]  Engine Instruments .... MONITOR

[ ]  Elevator Control .... ROTATE AT Vr (55 KTS)

[ ]  Normal Climb Speed .... 80 KTS

[ ]  Best Angle Vx .... 65 KTS

[ ]  Best Rate of Climb Vy .... 75 KTS

[ ]  Flaps .... RETRACT

Short Field Takeoff

[ ]  Flaps .... 25° (2nd Notch)

[ ]  Brakes .... HOLD

[ ]  Throttle .... SMOOTHLY ADVANCE TO FULL POWER

[ ]  Brakes .... RELEASE

[ ]  Airspeed Indicator .... ALIVE & WELL

[ ]  Engine Instruments .... MONITOR

[ ]  Elevator Control .... ROTATE AT Vr (55 KTS)

[ ]  Vx Climb Speed .... 65 KTS

[ ]  Flaps .... RETRACT (after obstacle clearance)

Flap Settings

[ ]  1st Notch .... 10°

[ ]  2nd Notch .... 25°

[ ]  3rd Notch .... 40°

Cruise

[ ]  Fuel Selector Valve .... FULLEST TANK

NOTE: Verify fuel pump is on before switching

[ ]  Fuel Pump .... OFF

[ ]  Lights .... AS REQUIRED

[ ]  Power .... AS NEEDED

[ ]  Mixture .... SET

[ ]  Trim .... ADJUST

[ ]  Carburetor Heat .... WHEN REQUIRED

Maneuvers

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Lights .... ON

[ ]  Mixture .... FULL RICH

[ ]  Clearing Turns .... COMPLETE

[ ]  Seat Belts .... SECURE

Before Landing

[ ]  Seats and Belts .... SECURE

[ ]  Fuel Pump .... ON

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Landing Light and Beacon .... ON

[ ]  Nav. Lights .... AS REQUIRED

[ ]  Mixture .... FULL RICH

[ ]  Carburetor Heat .... WHEN REQUIRED

Normal Landing

[ ]  Flaps .... 20° (extend below 100 KTS)

[ ]  Approach Speed .... 75 KTS

[ ]  Engine Instruments .... MONITOR

[ ]  Landing Clearance (if required) .... OBTAIN

Go Around/Missed Approach

[ ]  Throttle .... FULL POWER

[ ]  Carburetor Heat .... OFF (pushed in)

[ ]  Mixture .... VERIFY RICH

[ ]  Airspeed .... MAINTAIN 80 KTS

[ ]  Flaps .... RETRACT (after positive rate of climb)

[ ]  Engine Instruments .... MONITOR

After Landing/Clear of Runway

[ ]  Contact Ground .... IF REQUIRED

[ ]  Flaps .... RETRACT

[ ]  Fuel Pump .... OFF

[ ]  Transponder .... VFR STANDBY

[ ]  Lights .... AS REQUIRED

[ ]  Carburetor Heat .... OFF

[ ]  Mixture .... FULL RICH

Shutdown

[ ]  Electrical Equipment (except anti collision light) .... OFF

[ ]  Carburetor Heat .... OFF

[ ]  Throttle .... IDLE

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Key .... REMOVE

[ ]  Avionics .... OFF

[ ]  Anti Collision Light .... OFF

[ ]  Master Switch .... OFF

[ ]  Parking Brake .... SET

[ ]  Vents .... CLOSED

[ ]  Sunshade .... INSTALL

[ ]  Trash .... REMOVED

[ ]  Yoke .... SECURE w/SEAT BELT

[ ]  Tie Downs .... SECURE

[ ]  Pitot Cover .... SECURE

[ ]  Door .... LOCKED

[ ]  Flight Plan .... CLOSED

[ ]  Discrepancies .... LOG & REPORT

Engine Failure During Takeoff Roll  [EMERGENCY]

[ ]  Throttle .... FULL IDLE

NOTE: Try to clear runway before applying full brakes

[ ]  Brakes .... APPLY FULL PRESSURE

[ ]  Flaps .... RETRACT

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Pump .... OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

Engine Failure During Takeoff Climb  [EMERGENCY]

[ ]  Airspeed .... 70 KTS

[ ]  Best Place to Land .... AQUIRE

[ ]  Fuel Tanks .... SWITCH TANKS

[ ]  Mixture .... FULL RICH

[ ]  Carburetor Heat .... ON (if engine not yet started)

[ ]  Primer .... IN & LOCKED

IF ENGINE FAILS TO RESTART

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Flaps .... AS REQUIRED

NOTE: Prepare for an emergency landing!!

In-Flight Engine Failure  [EMERGENCY]

[ ]  Airspeed .... 70 KTS

[ ]  Best Place to Land .... AQUIRE

[ ]  Fuel Pump .... ON

[ ]  Fuel Tanks .... SWITCH TANKS

[ ]  Mixture .... FULL RICH

[ ]  Carburetor Heat .... ON (if engine not yet restarted)

[ ]  Magnetos .... BOTH

[ ]  Master Switch .... ON

[ ]  Primer .... IN & LOCKED

IF ENGINE FAILS TO RESTART

[ ]  Radios .... SET to 121.5 or Remain on ATC Freq.

[ ]  Transponder .... SET TO 7700

[ ]  Radios .... DECLARE AN EMERGENCY

NOTE: Using MAYDAY! MAYDAY! MAYDAY!

NOTE: Announce tail # - type of emergency position, color of aircraft & number of persons on board

NOTE: EXECUTE AN EMERGENCY LANDING!!

Transponder Codes

[ ]  7700 .... EMERGENCY

[ ]  7600 .... RADIO FAILURE

[ ]  7500 .... HI-JACKING

Emergency Landing  [EMERGENCY]

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Fuel Pump .... OFF

[ ]  Throttle .... IDLE

[ ]  Electrical Equipment .... OFF

[ ]  Cabin Door .... UNLATCH PRIOR TO TOUCHDOWN

[ ]  Flaps .... AS REQUIRED

[ ]  Seats and Belts .... SECURE

NOTE: Without Stalling the Airplane, Touchdown at Slowest Possible Airspeed!!

Emergency Decent  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Carburetor Heat .... ON

[ ]  Bank .... 45 DEGREES

[ ]  Flaps .... FULL

[ ]  Airspeed .... 95 KTS

[ ]  Fuel Pump .... ON

Engine Fire  [EMERGENCY]

[ ]  Airspeed .... 105 KTS

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Cabin Heat/Air .... CLOSE

[ ]  Master Switch .... OFF

NOTE: Prepare for an Emergency Landing!!

Electrical Fire  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Electrical Equipment .... OFF

[ ]  Cabin Heat/Air .... CLOSE

NOTE: Prepare for an Emergency Landing!!

Cabin Fire  [EMERGENCY]

[ ]  Cabin Heat/Air .... OFF

[ ]  Fire .... EXTINGUISH

NOTE: Prepare for an Emergency Landing!!

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-140/Jacob_Esparza_N6693W_Operating_Checklist_PA_28.txt

Rights: licensed

Checklist hash: f001ec90e72d187b5fe2b3ff15f4ee648216eab966573e1e1f05085a55360aec

Verify against the aircraft's own approved documentation before use.