Piper PA-28-140 — jochman-clpa28140

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Engine Failure During Takeoff Run  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... APPLY

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition Switch .... OFF

[ ]  Master .... OFF

Engine Failure Immediately After Takeoff  [EMERGENCY]

[ ]  Airspeed .... MAINTAIN SAFE AIRSPEED

[ ]  Fuel Selector Valve .... SWITCH TANKS

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Carburetor Heat .... ON

Engine Failure During Flight  [EMERGENCY]

[ ]  Airspeed .... 83 MPH

[ ]  Carburetor Heat .... ON

[ ]  Fuel Selector Valve .... SWITCH TANKS

[ ]  Mixture .... RICH

[ ]  Master Switch .... ON

[ ]  Auxiliary Fuel Pump .... ON

[ ]  Ignition Switch .... BOTH

[ ]  Primer .... IN & LOCKED

NOTE: If engine failure was caused by fuel exhaustion, power will not be regained after tanks are switched until empty fuel lines are filled, which may require up to ten seconds.

Landing w/o Engine Power  [EMERGENCY]

[ ]  Airspeed .... 83 MPH

[ ]  Radio .... MAYDAY on 121.5 MHz

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

[ ]  Door .... OPEN

NOTE: Touchdown should be made at lowest possible airspeed with full flaps.

Spins  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Rudder .... FULL OPPOSITE TO ROTATION

[ ]  Control Wheel .... FULL FORWARD

[ ]  Rudder .... NEUTRAL (WHEN ROTATION STOPS)

[ ]  Control Wheel .... REGAIN LEVEL FLIGHT

Electrical Fire in Flight  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Vents .... OPEN

[ ]  Cabin Heat and Air .... OFF

[ ]  Land .... AS SOON AS POSSIBLE

Engine Fire in Flight  [EMERGENCY]

[ ]  Fuel Selector Valve .... OFF

[ ]  Throttle .... CLOSED

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Cabin Heat and Air .... OFF

[ ]  Defroster .... OFF

[ ]  Forced Landing .... EXECUTE

Engine Fire During Start On Ground  [EMERGENCY]

[ ]  Cranking .... CONTINUE

IF ENGINE STARTS:

[ ]  Power .... 1800 RPM for a few minutes

[ ]  Engine .... SHUTDOWN & INSPECT

[ ]  Fuel Selector .... OFF

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

IF ENGINE FAILS TO START:

[ ]  Passengers .... EVACUATE

[ ]  Engine .... SECURE

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Fire .... EXTINGUISH

Cockpit

[ ]  Required Documents .... AVAILABLE

[ ]  Control Wheel .... RELEASE

[ ]  Master Switch .... ON

[ ]  Fuel Quantity Gauges .... CHECK

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Flaps .... DOWN

Right Wing

[ ]  Flap .... SECURE & UNDAMAGED

[ ]  Aileron .... FREEDOM OF MOVEMENT

[ ]  Wing Tip & Light .... UNDAMAGED

[ ]  Aileron Counterweight .... UNOBSTRUCTED

[ ]  Tiedown .... REMOVED

[ ]  Fuel Tank .... FULL, CAP SECURE

[ ]  Tank Drain .... DRAIN, SECURE

[ ]  Fuel .... PROPER COLOR

[ ]  Landing Gear, Fairing .... UNDAMAGED

[ ]  Tire .... PROPERLY INFLATED

[ ]  Chocks .... REMOVED

Nose

[ ]  Windshield .... CLEAN & UNDAMAGED

[ ]  Engine Oil Level .... 2 Qts. MIN, 8 Qts MAX

[ ]  Engine Oil Dipstick .... SECURED

[ ]  Propeller & Spinner .... SECURE, UNDAMAGED

[ ]  Cowling .... SECURE, UNDAMAGED

[ ]  Landing Light .... SECURE, UNDAMAGED

[ ]  Nose Gear, Fairing .... UNDAMAGED

[ ]  Tire .... PROPERLY INFLATED

[ ]  Tow Bar .... REMOVED & STOWED

[ ]  Chocks .... REMOVED

[ ]  Alternator Belt .... PROPER TENSION

[ ]  Fuel Sump Drain .... DRAIN, SECURE

Left Wing

[ ]  Fuel Tank .... FULL, CAP SECURE

[ ]  Tank Drain .... DRAIN, SECURE

[ ]  Fuel .... PROPER COLOR

[ ]  Landing Gear, Fairing .... UNDAMAGED

[ ]  Tire .... PROPERLY INFLATED

[ ]  Chocks .... REMOVED

[ ]  Wing Tip & Light .... UNDAMAGED

[ ]  Aileron Counterweight .... UNOBSTRUCTED

[ ]  Tiedown .... REMOVED

[ ]  Pitot Mast .... UNDAMAGED

[ ]  Aileron .... FREEDOM OF MOVEMENT

[ ]  Flap .... SECURE & UNDAMAGED

Empennage

[ ]  Elevators .... FREEDOM OF MOVEMENT

[ ]  Rudder .... FREEDOM OF MOVEMENT

[ ]  Trim Tabs .... SECURE, UNDAMAGED

[ ]  Tail Cone & Light .... SECURE, UNDAMAGED

[ ]  Tie Down .... REMOVED

Before Starting Engine

[ ]  Preflight Inspection .... COMPLETE

[ ]  Flaps .... UP

[ ]  Seats, Belts, Shoulder Harness .... ADJUSTED, LOCKED

[ ]  Door .... LATCHED

[ ]  Radio, Autopilot, Electrical Equipment .... OFF

[ ]  Parking Brake .... SET

[ ]  Fuel Selector Valve .... FULLEST TANK

[ ]  Controls .... CHECK

[ ]  Circuit Breakers .... CHECK

Engine Start (COLD)

[ ]  Master Switch .... ON

[ ]  Mixture .... FULL RICH

[ ]  Fuel Pump .... ON

[ ]  Fuel Pressure .... CHECK

[ ]  Carburetor Heat .... OFF

[ ]  Prime .... AS REQUIRED

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Propeller .... CLEAR

[ ]  Ignition Switch .... START

[ ]  Oil Pressure .... CHECK

[ ]  Engine .... WARM UP 1000 RPM

[ ]  Radios .... ON

[ ]  Transponder .... STANDBY

Before Taxi

[ ]  Parking Brake .... SET

[ ]  Brakes .... TEST

[ ]  Lights .... AS REQUIRED

[ ]  Radios .... ON, CHECKED

Before Takeoff

[ ]  Parking Brake .... SET

[ ]  Lights .... AS REQUIRED

[ ]  Flight Instruments .... SET

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Throttle .... 2000 RPM

[ ]  Magnetos .... CHECK (175 RPM Max drop, 50 RPM diff.)

[ ]  Carburetor Heat .... ON, CHECK RPM, OFF

[ ]  Engine Instruments .... IN GREEN ARC

[ ]  Vacuum Gauge .... 4.9-5.1

[ ]  Throttle .... 1000 RPM

[ ]  Trim Tab .... SET

[ ]  Mixture .... FULL RICH

[ ]  Throttle Friction Lock .... ADJUSTED

[ ]  Auxiliary Fuel Pump .... OFF, CHECK, ON

[ ]  Parking Brake .... OFF

[ ]  Transponder .... ON-ALT

Normal Takeoff

[ ]  Flaps .... UP

[ ]  Carburetor Heat .... OFF

[ ]  Auxiliary Fuel Pump .... ON

[ ]  Throttle .... FULL OPEN

[ ]  Rotate .... 50-60 MPH

[ ]  Climb Speed .... 85 MPH

Short Field Takeoff, Obstacle Clearance

[ ]  Flaps .... 25° (Second Notch)

[ ]  Carburetor Heat .... OFF

[ ]  Auxiliary Fuel Pump .... ON

[ ]  Throttle .... FULL OPEN

[ ]  Rotate .... 55-60 MPH

[ ]  Initial Climb Speed .... 74 MPH

Climb

[ ]  Normal Climb .... 85-100 MPH

[ ]  VY – Best Rate of Climb Speed .... 85 MPH

[ ]  VX – Best Angle of Climb Speed .... 74 MPH

Cruise

[ ]  Electric Fuel Pump .... OFF

[ ]  Power .... 2200-2600 RPM

[ ]  Trim Tab .... SET

[ ]  Mixture .... ADJUST

[ ]  Fuel Selector .... SWITCH AS NEEDED

Before Landing

[ ]  Seats, Belts, Shoulder Harness .... ADJUST, LOCK

[ ]  Fuel Selector .... ON, FULLEST TANK

[ ]  Mixture .... FULL RICH

[ ]  Auxiliary Fuel Pump .... ON

[ ]  Parking Brake .... OFF

[ ]  Flaps .... AS REQUIRED, BELOW 115 MPH

[ ]  Airspeed .... 85 MPH

[ ]  Landing Light .... ON AS REQUIRED

After Landing

[ ]  Flaps .... UP

[ ]  Auxiliary Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Carburetor Heat .... OFF

[ ]  Transponder .... STANDBY

Shut-Down, Securing Airplane

[ ]  Electrical Equipment, Radios, Lights .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Ignition .... OFF

[ ]  Master Switch .... OFF

[ ]  Parking Brake .... SET (IF REQUIRED)

[ ]  Chocks/Tiedowns .... INSTALLED

[ ]  Parking Brake .... OFF

Speeds for Normal Operation

NOTE: Vx 74 MPH = Best Angle of Climb Speed

NOTE: Vy 85 MPH = Best Rate of Climb Speed

NOTE: Vbest l/d 83 MPH = Best Glide (lift/drag), flaps up/down

NOTE: Vs 64 MPH = Stall Speed, normal configuration

NOTE: Vso 55 MPH = Stall Speed, landing configuration

NOTE: Vfo 115 MPH = Maximum Flap Extension Speed

NOTE: Va 129 MPH = Maneuvering Speed (at gross weight)

NOTE: Vno 140 MPH = Maximum Structural Cruising Speed

NOTE: Vne 171 MPH = Never Exceed Speed

NOTE: 100 MPH = Normal Climb Out

NOTE: 74 MPH = Short Field T/O, Flaps 25°

NOTE: 85 MPH = Normal Landing Approach, Flaps Up

NOTE: 76 MPH = Normal Landing Approach, Flaps 40°

NOTE: 76 MPH = Short Field Approach, Flaps 40°

Specifications

NOTE: 2150 lbs = Maximum Gross Weight

NOTE: 1340 lbs = Basic Empty Weight

NOTE: 594 lbs = Payload With Standard Fuel

NOTE: 8.4 gph = Fuel Flow @ 75% Power

NOTE: 50 gal = Total Fuel Capacity

NOTE: 36 gal = Standard Fuel Capacity

NOTE: 100LL = Fuel Octane Rating (Blue)

NOTE: 8 qts = Oil Sump Capacity

NOTE: 2 qts = Minimum Oil Quantity

NOTE: 150 hp = Lycoming O-320

NOTE: 12 V = Battery

NOTE: 3.8g = Positive Limit Load Factor

NOTE: 0g = Negative Limit Load Factor

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-140/jochman-clpa28140.txt

Rights: licensed

Checklist hash: f53a62a4af2a2fb966d6cf963bb81a079f93d11695a10b3dc6919710e5a4359c

Verify against the aircraft's own approved documentation before use.