Piper PA-28-161 Warrior II — David_Kohl_PiperWarriorIIChecklists5x8

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Danbury Radio Frequencies" item count differs between passes (12 vs 13); verify nothing was dropped.

content_gap: Section "Pre-flight Inspection – Exterior" item count differs between passes (11 vs 54); verify nothing was dropped.

content_gap: Section "Takeoff Reference Speeds" item count differs between passes (6 vs 9); verify nothing was dropped.

Pre-flight Planning

[ ]  Weather .... Checked

[ ]  Performance data .... Checked

[ ]  Weight & balance

[ ]  Aircraft documents .... Verified

NOTE: Airworthiness certificate, Registration, Operations manual (ARrOw)

Pre-flight Inspection – Interior

[ ]  Parking brake .... Set as desired

[ ]  Controls .... Free and clear

[ ]  Ignition .... Off

[ ]  Avionics .... Off

[ ]  Master switch .... On

[ ]  Lights .... On and verified

[ ]  Fuel pump .... On

[ ]  Fuel pressure / quantity .... Checked

[ ]  Fuel pump .... Off

[ ]  Master switch .... Off

[ ]  Flaps .... Extended

Pre-flight Inspection – Exterior

RIGHT WING

[ ]  Flap condition .... Checked

[ ]  Aileron .... Checked

[ ]  Wing tip .... Checked

[ ]  Wing surface condition .... Checked

[ ]  Fuel level .... Checked

[ ]  Fuel vent .... Checked

[ ]  Fuel sump .... Drained

[ ]  Main gear strut .... Checked

[ ]  Tire, brakes and fluid .... Checked

[ ]  Tie down .... Removed

Engine Cowling

[ ]  Windshield .... Checked

[ ]  Surface condition .... Checked

[ ]  Right cowling fasteners .... Open

[ ]  Interior wiring, pipes, etc. .... Checked

[ ]  Oil level .... Between 6 and 8 quarts

[ ]  Right cowling fasteners .... Closed

[ ]  Propeller .... Checked

[ ]  Spinner .... Checked

[ ]  Alternator belt .... Checked

[ ]  Air inlets .... Free and clear

[ ]  Exhaust system .... Tight

[ ]  Nose gear strut .... Checked

[ ]  Tire .... Checked

[ ]  Left cowling fasteners .... Open

[ ]  Interior wiring, pipes, etc. .... Checked

[ ]  Brake fluid .... Checked

[ ]  Left Cowling fasteners .... Closed

[ ]  Fuel sump .... Checked

[ ]  Surface condition .... Checked

[ ]  Windshield .... Checked

Left Wing

[ ]  Main gear strut .... Checked

[ ]  Tire, brakes and fluid .... Checked

[ ]  Fuel sump .... Checked

[ ]  Fuel vent .... Checked

[ ]  Tie down .... Removed

[ ]  Pitot and static port .... Checked

[ ]  Fuel level .... Checked

[ ]  Stall warning .... Checked

[ ]  Surface condition .... Checked

[ ]  Wing tip .... Checked

[ ]  Aileron .... Checked

[ ]  Flap condition .... Checked

Aft Fuselage and Empennage

[ ]  Left surface condition .... Checked

[ ]  Antennas .... Checked

[ ]  Stabilator, trim tab .... Checked

[ ]  Rudder .... Checked

[ ]  Tie down .... Removed

[ ]  Tail cone, skid .... Checked

[ ]  Right surface condition .... Checked

[ ]  Baggage door .... Checked

Pre-flight Inspection (continued)

INTERIOR

[ ]  Flaps .... Retracted

[ ]  Seats .... Adjusted and locked

[ ]  Seatbelts .... On

[ ]  Shoulder harness .... On

[ ]  Passenger briefing .... Complete

Before Engine Start

[ ]  Trim (elevator & rudder) .... Set

[ ]  Fuel selector .... Lowest tank

[ ]  Heat / fresh air controls .... Set

[ ]  Circuit breakers .... Checked

[ ]  Carb heat .... Cold

[ ]  Friction lock .... Set

[ ]  Mixture .... Full rich

[ ]  Throttle .... Set

[ ]  Primer .... As required, then locked

[ ]  Mags .... Off

[ ]  Flight instruments .... Checked

[ ]  Avionics .... Off

[ ]  Lights .... Beacon on

[ ]  Fuel pump .... On

[ ]  Master switch .... On

[ ]  Fuel pressure .... Checked

[ ]  Propeller area .... Cleared

[ ]  Toe brakes .... Feet on pedals

[ ]  Parking brake .... Released

[ ]  Ignition switch .... Start engine

After Engine Start

[ ]  Throttle .... 1,000 RPM

[ ]  Oil pressure .... Checked

[ ]  Fuel pump .... Off

[ ]  Fuel pressure .... Checked

[ ]  Alternator .... Checked

[ ]  Vacuum .... Checked

[ ]  Avionics .... On

[ ]  Radios .... Set

[ ]  ATIS .... Obtain

[ ]  Altimeter .... Set

[ ]  Directional gyro .... Magnetic Compass

Taxi

[ ]  Ground control for taxi .... Call

[ ]  Parking brake .... Released

[ ]  Toe brakes .... Checked

[ ]  Flight controls .... As required for wind

[ ]  Taxi .... Monitor traffic

[ ]  Flight instruments .... Checked

[ ]  Hold in run-up area .... Hold

Before Takeoff

[ ]  Parking brake or toe brakes .... Set

[ ]  Throttle .... 1000 RPM

[ ]  Trim .... Set

[ ]  Fuel pump .... On

[ ]  Fuel pressure .... Checked

[ ]  Fuel selector .... Fullest tank

[ ]  Fuel pump .... Off

[ ]  Fuel pressure .... Checked

Engine Run-up

[ ]  Throttle .... 2000 RPM

[ ]  Magnetos .... Checked

NOTE: Max drop 175 RPM, Max difference 50 RPM

[ ]  Engine Instruments .... Green band

[ ]  Enunciator lights .... V A O Checked

[ ]  Vacuum .... 4.8 to 5.2 PSI

[ ]  Ammeter .... Charging

[ ]  Carb heat .... Some decrease, no rise

[ ]  Mixture .... Full rich

NOTE: Lean mixture above 3000 feet

[ ]  Throttle .... Idle, then 1000 RPM

[ ]  Parking brake .... Verify off

Pre-flight Completion

[ ]  Altimeter .... Indicated alt = Ground alt

[ ]  Directional gyro .... Magnetic compass

[ ]  Flight instruments .... Checked

[ ]  Radios .... COM / NAV set

[ ]  Flight controls .... Free and clear

[ ]  Flaps .... Set as needed

[ ]  Safety briefing .... Complete

[ ]  Doors and windows .... Locked

[ ]  Tower .... Call

Position and Hold

[ ]  Strobe lights .... On

[ ]  Nav lights .... On for night flight

[ ]  Auto-pilot .... Verify Off

[ ]  Flight controls .... Free and clear

[ ]  Transponder .... On ALT

Cleared for Takeoff

[ ]  Fuel pump .... On

[ ]  Fuel pressure .... Checked

[ ]  Landing lights .... On

[ ]  Time .... Check

After Takeoff

[ ]  Flaps .... Up

[ ]  Landing lights .... Off

[ ]  Fuel pump .... Off

[ ]  Fuel pressure .... Checked

Cruise

[ ]  Cruise power .... Set

[ ]  Mixture .... Set

[ ]  Engine instruments .... Checked

[ ]  Directional Gyro .... Magnetic Compass

[ ]  Fuel selector .... As required

NOTE: Switch tanks after first ½ hour and then every hour

Descent

[ ]  ATIS .... Obtain

[ ]  Tower .... Call

[ ]  Safety briefing .... Complete

[ ]  Seat belts and harness .... On

[ ]  Landing lights .... On

[ ]  Fuel pump .... On

[ ]  Fuel pressure .... Checked

[ ]  Fuel selector .... Fullest tank

[ ]  Mags .... Checked on both

[ ]  Engine instruments .... Checked

[ ]  Directional Gyro .... Magnetic Compass

[ ]  Primer .... Locked

[ ]  Mixture .... Full rich

[ ]  Carb heat .... Checked

Approach for Landing

[ ]  VFE Clean to 10˚ .... 103 KIAS

[ ]  VMin Approach .... 70 KIAS

[ ]  VLanding with flaps at 40˚ .... 63 KIAS

After Landing

[ ]  Flaps .... Retract

[ ]  Trim .... Reset

[ ]  Fuel pump .... Off

[ ]  Fuel pressure .... Checked

[ ]  Landing and strobe lights .... Off

[ ]  Transponder .... Standby

[ ]  Ground control .... Call

Engine Shutdown

[ ]  Parking brake .... As required

[ ]  Throttle .... 1000 RPM

[ ]  Avionics .... Off

[ ]  Mixture .... Idle cutoff

[ ]  Magnetos .... Off after prop stops

[ ]  Lights .... Off

[ ]  Master switch .... Off

Secure Aircraft

[ ]  Controls .... Seat belt secured

[ ]  All switches .... Verify off

[ ]  Hobbs / Tach times .... Record

[ ]  Doors and windows .... Closed

[ ]  Aircraft .... Tie down

Engine Fire During Start  [EMERGENCY]

Engine Fire During Start

[ ]  Starter .... Continue cranking

[ ]  Mixture .... Idle cutoff

[ ]  Throttle .... Wide open

[ ]  Fuel pump .... Off

[ ]  Fuel selector .... Off

[ ]  If fire persists .... Evacuate

Engine Fire During Flight  [EMERGENCY]

Engine Fire During Flight

[ ]  Airspeed .... Increase

[ ]  Fuel pump .... Off

[ ]  Fuel selector .... Off

[ ]  Throttle .... Closed

[ ]  Mixture .... Idle cutoff

[ ]  Magnetos .... Off

[ ]  Cabin heat / defrost / air .... Off

[ ]  Emergency landing check .... Perform

Electrical Fire (Smoke)  [EMERGENCY]

Electrical Fire (Smoke)

[ ]  Master switch .... Off

[ ]  Electrical equipment .... Off

[ ]  Floor air vents .... Open

[ ]  Cabin heat / air .... Off

[ ]  If smoke stops .... Troubleshoot

[ ]  Land .... ASAP

Loss of Oil Pressure  [EMERGENCY]

Loss of Oil Pressure

[ ]  Land .... ASAP

[ ]  Prepare for Engine failure

Loss of Fuel Pressure  [EMERGENCY]

Loss of Fuel Pressure

[ ]  Fuel pump .... On

[ ]  Fuel selector .... Switch tanks

[ ]  Fuel pressure .... Checked

[ ]  Prepare for Engine failure

High Oil Temperature  [EMERGENCY]

High Oil Temperature

[ ]  Mixture .... Full rich

[ ]  Land .... ASAP

[ ]  Prepare for Engine failure

Alternator / Generator Failure  [EMERGENCY]

Alternator / Generator Failure

[ ]  Ammeter .... Check

[ ]  Alternator switch .... Off

[ ]  Circuit breakers .... Check

[ ]  Alternator switch .... On

NOTE: If power cannot be restored, reduce electrical load

[ ]  Land .... ASAP

Engine Failure  [EMERGENCY]

Engine Failure

[ ]  Safe airspeed .... 73 KIAS

[ ]  Best place to land .... Choose

[ ]  Carb heat .... On

[ ]  Begin maneuvering toward safe landing area

[ ]  Fuel pump .... On

[ ]  Mixture .... Different setting

[ ]  Throttle .... Different setting

[ ]  Primer .... Verify locked

[ ]  Fuel pressure .... Checked

[ ]  Magnetos .... Checked

[ ]  Fuel selector .... Switch tanks

Emergency Landing  [EMERGENCY]

Emergency Landing

[ ]  Best glide speed .... 73 KIAS

[ ]  Seat belt / harness .... On

[ ]  Fuel selector .... Off

[ ]  Magnetos .... Off

[ ]  Mixture .... Idle cutoff

[ ]  Fuel pump .... Off

[ ]  Door .... Consider opening

[ ]  Master switch .... Off

Transponder Codes

[ ]  7500 .... Hijacking

[ ]  7600 .... Lost radio communications

[ ]  7700 .... General emergency

[ ]  7777 .... Military interceptor

Pilot-Controlled Lighting

[ ]  Seven clicks .... Turns on, high intensity

[ ]  Five clicks .... Medium intensity

[ ]  Three clicks .... Low intensity

ATC Light Signals

ON THE GROUND

[ ]  Flashing Green .... Cleared to taxi

[ ]  Steady Green .... Cleared for takeoff

[ ]  Flashing Red .... Taxi clear of runway

[ ]  Steady Red .... Stop

[ ]  Flashing White .... Return to FBO

[ ]  Alt Red and Green .... Use caution

IN THE AIR

[ ]  Flashing Green .... Return for landing

[ ]  Steady Green .... Cleared to land

[ ]  Flashing Red .... Airport unsafe

[ ]  Steady Red .... Give way, circle

[ ]  Alt Red and Green .... Use caution

Takeoff Reference Speeds

[ ]  VRotate (clean) .... 55 KIAS

[ ]  VRotate (25˚ flaps) .... 52 KIAS

[ ]  VY .... 79 KIAS

NOTE: Best Rate: Greatest alt gain in shortest time

[ ]  VX .... 63 KIAS

NOTE: Best Angle: Greatest alt gain in shortest distance

Danbury Runways

[ ]  Rwy 08-26 .... 4422X150

[ ]  Rwy 17-35 .... 3135X100

Danbury Radio Frequencies

[ ]  ATIS .... 127.75

[ ]  Ground Control .... 121.60

[ ]  Tower / CTAF .... 119.40

[ ]  UNICOM .... 122.95

[ ]  NY Dep / Appr .... 126.40

[ ]  Clearance Delivery .... 128.60

NOTE: when DXR tower closed

[ ]  Carmel VOR/DME .... 116.60

[ ]  ILS/DME 08 .... 111.55

Danbury Radio Frequencies

[ ]  ATIS .... 127.75

[ ]  Ground Control .... 121.60

[ ]  Tower / CTAF .... 119.40

[ ]  UNICOM .... 122.95

[ ]  NY Dep / Appr .... 126.40

[ ]  Bridgeport FSS .... 122.20

[ ]  Carmel VOR/DME .... 116.60

[ ]  ILS/DME 08 .... 111.55

NOTE: Mayday: Call 121.5 and Squawk 7700

[ ]  VA at 2325 Lbs .... 111 KIAS

[ ]  VA at 1531 Lbs .... 88 KIAS

[ ]  VEn-Route Climb .... 87 KIAS

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-161 Warrior II/David_Kohl_PiperWarriorIIChecklists5x8.txt

Rights: licensed

Checklist hash: fc0d4841b415eba5f4c9e9535063af39f654a2c701f0744ce9b6d8b5f1668738

Verify against the aircraft's own approved documentation before use.