Piper PA-28-161 Warrior II — paul_fawcett_warriorchecklist18

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "After Landing" item count differs between passes (3 vs 8); verify nothing was dropped.

content_gap: Section "Before Takeoff" item count differs between passes (23 vs 14); verify nothing was dropped.

Preflight Inspection

COCKPIT

[ ]  Electrical Equip. .... Off

[ ]  Avionics Master .... Off

[ ]  Circuit Breakers .... In

[ ]  Master Switch .... On

[ ]  Annunciators .... Chk on

[ ]  Turn Coordinator .... Chk flag

[ ]  Fuel Gauges .... Chk qty.

[ ]  Fuel Selector .... Low tank

[ ]  Static/Pitot .... Drain

[ ]  Fuel Pump - On .... Chk press.

[ ]  Lights .... Chk

[ ]  Master Switch .... Off

[ ]  Trims .... Chk

[ ]  Flaps .... Down

[ ]  Aircraft Docs .... AROW

RIGHT WING

[ ]  Flap .... Chk

[ ]  Aileron .... Chk

[ ]  Wingtip / lights .... Chk

[ ]  Leading edge .... Chk clean

[ ]  Fuel quantity .... Chk

[ ]  Fuel vent .... Clear

[ ]  Fuel strainer .... Sample

[ ]  Landing gear/tire .... Chk

[ ]  Air inlet .... Clear

NOSE

[ ]  Engine condition R .... Chk

[ ]  Oil quantity .... Chk

[ ]  Prop and spinner .... Chk

[ ]  Alternator belt .... Chk

[ ]  Air inlets .... Clear

[ ]  Exhaust stacks .... Chk

[ ]  Landing light .... Chk

[ ]  Nose gear, tire .... Chk

[ ]  Engine condition L .... Chk

[ ]  Brake reservoir .... Chk

LEFT WING

[ ]  As per right wing plus:

[ ]  Pitot / static mast .... Clear

[ ]  Stall horn .... Chk audio

[ ]  Nose fuel drain .... Chk (2x)

FUSELAGE & EMPENNAGE

[ ]  Antennas .... Chk

[ ]  Air inlet .... Chk

[ ]  Stabilator & trim .... Chk

[ ]  Jackscrew .... Chk

[ ]  Rudder / stab stops .... Chk

[ ]  Roll & "Big picture" .... Chk

Before Start

[ ]  Hobbs Time .... Record

[ ]  Charts .... Available

[ ]  Seats .... Adjust & lock

[ ]  Restraints .... Fastened

[ ]  Radio and elect. .... Off

Engine Start

[ ]  Brakes .... Set

[ ]  Carb heat .... Off

[ ]  Mixture .... Full rich

[ ]  Master switch .... On

[ ]  Fuel pump .... On

[ ]  Prime .... 3 times

[ ]  Tail beacon .... On

[ ]  Propellor .... Clear!

[ ]  Starter .... Engage

[ ]  Throttle .... Pump twice

[ ]  Throttle .... 1000 RPM

[ ]  Oil temperature .... Chk

[ ]  Oil pressure .... Chk

[ ]  Ammeter .... Chk

[ ]  Vacuum .... Chk

[ ]  Annunciators .... Out

[ ]  Mixture .... Lean for taxi

[ ]  Fuel pump off .... Chk press.

Before Taxi

[ ]  Flaps .... Up

[ ]  Avionics .... On

[ ]  Transponder .... 1200, stdby

[ ]  ATIS .... Copy

[ ]  Altimeter .... Set & chk

[ ]  Hdg indicator .... Set

[ ]  Parking brake .... Release

[ ]  Ground control .... Clearance

Taxi

[ ]  Brakes .... Chk R & L

[ ]  Turn coord. .... Chk in turn

[ ]  Directional gyro .... Chk in turn

[ ]  Compass .... Chk in turn

[ ]  Attitude indic. .... No D in turn

Run-Up

[ ]  Brakes .... Set

[ ]  Mixture .... Full rich

[ ]  Fuel pump .... On

[ ]  Fuel selector .... Switch tank

[ ]  Fuel pump - Off .... Chk press.

[ ]  Throttle .... 2000 RPM

[ ]  Magnetos .... <175, <50 dif

[ ]  Vacuum .... Chk (4.9-5.1)

[ ]  Annunciators .... Test

[ ]  Engine instr. .... In green

[ ]  Carb heat on .... Chk drop

[ ]  Throttle .... idle

[ ]  Carb heat .... Off

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... As required

Before Takeoff

[ ]  Circuit breakers .... Chk in

[ ]  Flight instr. .... Chk & set

[ ]  Comm. radio .... Set tower

[ ]  Pitot heat .... Load test

[ ]  Primer .... In & locked

[ ]  Flaps .... Set

[ ]  Trims .... Set both

[ ]  Controls .... Free & crct

[ ]  Doors / window .... Secured

[ ]  Seats belts .... Secured

[ ]  Briefing .... Complete

[ ]  Air conditioner .... Off

[ ]  Autopilot .... Off

[ ]  Tower .... Clearance

[ ]  Cleared for Takeoff

[ ]  Brakes .... Off

[ ]  Lights .... On

[ ]  Camera .... Xpdr ALT

[ ]  Action

[ ]  Mixture .... Full rich

[ ]  Fuel pump .... On

[ ]  Compass & DG .... Chk rwy hdg.

[ ]  Time off .... Noted

After Takeoff

[ ]  Flaps .... Up

[ ]  Fuel pump .... Off - chk pres.

Cruise

[ ]  Landing lights .... Off

[ ]  Throttle .... As req.

[ ]  Mixture .... Leaned

Approach

[ ]  Comm. radios .... Set

[ ]  Flight instrmts. .... Chk & set

[ ]  Fuel selector .... Best tank

[ ]  Fuel pump .... On

[ ]  Mixture .... Full rich

[ ]  Landing light .... On

[ ]  Brakes .... Chk

[ ]  Seats belts .... Secured

After Landing

[ ]  Throttle .... 1000 RPM

[ ]  Transponder .... Standby

[ ]  Carb heat .... Off

Shutdown

[ ]  Comm. radio .... Monitor 121.5

[ ]  Avionics .... Off

[ ]  Electr. equip. .... Off

[ ]  Throttle .... Idle

[ ]  Magnetos .... Chk GND

[ ]  Throttle .... 1200 RPM

[ ]  Mixture .... Idle Cutoff

[ ]  Magnetos/keys .... Off & out

[ ]  Master switch .... Off!!!!

[ ]  Hobbs/Tach .... Record

Speeds

[ ]  Vr .... 55

[ ]  Vx .... 63

[ ]  Vy .... 79

[ ]  Climb .... 87

[ ]  Vs .... 50

[ ]  Vso .... 44

[ ]  Va .... 88-111

[ ]  Vfe .... 103

[ ]  Vno .... 126

[ ]  Vne .... 160

[ ]  Glide .... 73

[ ]  Max dem.XW .... 17

[ ]  Vapp (flaps) .... 63

Airports and Frequencies

PAO

[ ]  ATIS .... 120.6

[ ]  GND .... 125.0

[ ]  TWR .... 118.6

SQL

[ ]  ATIS .... 125.9

[ ]  GND .... 121.6

[ ]  TWR .... 119.0

HAF

[ ]  UNICOM .... 122.8

LVK

[ ]  ATIS .... 119.65

[ ]  GND .... 121.6

[ ]  TWR .... 118.1

Engine Failure  [EMERGENCY]

[ ]  Altitude for restart??

NO

[ ]  Glide .... 73 Kts

[ ]  Landing Site .... Select

[ ]  Mixture .... Cut-off

[ ]  Master Switch .... Off

[ ]  Magnetos .... Off

[ ]  Fuel Selector .... Off

YES

[ ]  Fuel Selector .... Opp. tank

[ ]  Fuel Pump .... On

[ ]  Carb Heat .... On

[ ]  Mixture .... Full rich

[ ]  Throttle .... Adjust

[ ]  Primer .... Locked

[ ]  Magnetos .... Check

[ ]  Glide .... 73 Kts

[ ]  Engine restarted?

NO

[ ]  Mayday .... Transmit

[ ]  Transponder .... 7700

[ ]  Mixture .... Cut-off

[ ]  Fuel Selector .... Off

[ ]  Magnetos .... Off

[ ]  Master Switch .... Off

Engine Fire During Start  [EMERGENCY]

[ ]  Starter .... Engage

[ ]  Mixture .... Cut-off

[ ]  Throttle .... Open

[ ]  Fuel Pump .... Off

[ ]  Fuel Selector .... Off

WARNING: Evacuate if fire continues!

Engine Fire During Taxi  [EMERGENCY]

[ ]  Airplane .... Stop

[ ]  Mixture .... Cut-off

[ ]  Fuel Selector .... Off

[ ]  Master switch .... Off

WARNING: Evacuate Aircraft

Engine Fire In Flight  [EMERGENCY]

[ ]  Mixture .... Cut-off

[ ]  Fuel selector .... Off

[ ]  Glide .... 73 Kts

[ ]  Mayday .... Transmit

[ ]  Transponder .... 7700

[ ]  Magnetos .... Off

[ ]  Master switch .... Off

Cabin Fire in Flight  [EMERGENCY]

[ ]  Master switch .... Off

[ ]  Electrical equip. .... Off

[ ]  Cabin heater .... Off

[ ]  Vents .... Open

Low Fuel Pressure  [EMERGENCY]

[ ]  Fuel pump .... On

[ ]  Fuel selector .... Opp. tank

Engine Roughness  [EMERGENCY]

[ ]  Carb heat .... On

[ ]  Still rough after 1 min?

[ ]  Carb heat .... Off

[ ]  Mixture .... Adjust

[ ]  Fuel pump .... On

[ ]  Fuel selector .... Opp. tank

[ ]  Engine instr. .... Chk

[ ]  Magnetos .... Chk

Loss of Oil Pressure or High Oil Temperature  [EMERGENCY]

WARNING: Land as soon as possible

WARNING: Prepare for engine failure

Spin Recovery  [EMERGENCY]

[ ]  Throttle .... Idle

[ ]  Ailerons .... Neutral

[ ]  Rudder .... Against spin

[ ]  Yoke .... Full forward

[ ]  Rotation stopped?

[ ]  Rudder .... Neutral

[ ]  Yoke .... Neutral

[ ]  Pitch .... Level

[ ]  Throttle .... As required

Electrical Failure  [EMERGENCY]

[ ]  Ammeter .... Verify inop.

[ ]  Load is zero?

[ ]  Alternator .... Off

[ ]  Electrical load .... Minimize

[ ]  Alt. circ. brkr. .... Chk & reset

[ ]  Alternator .... On

[ ]  Power not restored?

[ ]  Alternator .... Off

[ ]  Electrical load .... Minimize

WARNING: Land as soon as possible

Electrical Overload  [EMERGENCY]

[ ]  Alternator .... Off

[ ]  Battery .... On

[ ]  Loads reduced?

[ ]  Electrical load .... Minimize

WARNING: Land as soon as possible

[ ]  Not reduced?

[ ]  Alternator .... Off

[ ]  Battery .... As required

WARNING: Essential equipment only!

Carburetor Icing  [EMERGENCY]

[ ]  Carb heat .... On

[ ]  Mixture .... Adjust

Emergency Frequencies  [EMERGENCY]

[ ]  Frequency .... 121.5

[ ]  Transponder .... 7700

Xwind Component

[ ]  0.5 .... 30 deg.

[ ]  0.7 .... 45 deg.

[ ]  0.9 .... 60 deg.

Source Footer

NOTE: PA-28-161 Checklist V1.8

WARNING: May 1, 2003 - Use at own Risk!

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-161 Warrior II/paul_fawcett_warriorchecklist18.txt

Rights: licensed

Checklist hash: 14676a961c447c1129f042d2bfa8f5b23d5e0d3677196a7ad5f58e21fb91da52

Verify against the aircraft's own approved documentation before use.