Piper PA-28-180 Cherokee Archer — pa28180_abbreviated_checklist_millstone_valley

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Piper Cherokee Airspeeds (MCAS)" item count differs between passes (11 vs 1); verify nothing was dropped.

BEFORE STARTING ENGINE

NOTE: Refer to Aircraft Flight Manual for N3471R or N4677R as Appropriate.

[ ]  Exterior Preflight .... COMPLETE

[ ]  Tach/Hobbs Meter .... RECORD

[ ]  Seats, Belts, Harnesses .... ADJUST & LOCKED

[ ]  Radios & Electrical Equipment .... OFF

[ ]  Brakes .... TEST & SET

STARTING ENGINE

[ ]  Parking Brake .... SET

[ ]  Carburetor Heat .... COLD

[ ]  Fuel Selector .... SET TO DESIRED TANK

COLD START

[ ]  Throttle .... OPEN ¼ INCH

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Magnetos .... ROTATE SWITCH CLOCKWISE WHILE PRESSING IN

N4677R: With Magnetos on Both, Press the Starter Push-button.

[ ]  Throttle .... ADVANCE WHEN ENGINE FIRES

HOT START

[ ]  Throttle .... OPEN ½ INCH

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... IDLE CUTOFF

[ ]  Starter .... ROTATE SWITCH CLOCKWISE WHILE PRESSING IN

N4677R: With Magnetos on Both, Press the Starter Push-button.

[ ]  Mixture .... ADVANCE WHEN ENGINE FIRES

[ ]  Throttle .... ADJUST

FLOODED START

[ ]  Throttle .... OPEN FULLY

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Magnetos .... ROTATE SWITCH CLOCKWISE WHILE PRESSING IN

N4677R: With Magnetos on Both, Press the Starter Push-button.

[ ]  Mixture .... ADVANCE WHEN ENGINE FIRES

[ ]  Throttle .... ADJUST

GROUND CHECK

[ ]  Brakes .... SET

[ ]  Throttle .... 1800 RPM

[ ]  Magnetos .... CHECK

125 MAX DROP/50 DIFF.

[ ]  Carburetor Heat .... CHECK FOR RPM DROP

[ ]  Engine Gauges .... CHECK

[ ]  Fuel Pump .... CHECK OPERATION

TAKEOFF

[ ]  Fuel .... ON PROPER TANK

[ ]  Electric Fuel Pump .... ON

[ ]  Engine Gauges .... CHECKED

[ ]  Carburetor Heat .... OFF

[ ]  Mixture .... SET

[ ]  Quadrant Friction Lock .... SET

[ ]  Trim Tabs .... SET

[ ]  Flaps .... SET

[ ]  Controls .... FREE & CORRECT

[ ]  Door .... LATCHED

ENROUTE CLIMB

[ ]  Airspeed .... 85 - 100 MPH

[ ]  Throttle .... FULL OPEN

[ ]  Mixture .... RICH

Unless engine is rough.

CRUISE

[ ]  Power .... 2300 to 2700

No more than 75%.

[ ]  Fuel Selector .... ON PROPER TANK

[ ]  Mixture .... FULL RICH BELOW 5000 FT

APPROACH & LANDING

[ ]  Fuel Selector .... ON PROPER TANK

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... SET

[ ]  Flaps .... SET

[ ]  Airspeed .... 85 MPH (NO FLAPS) OR 80 MPH (FLAPS)

BALKED LANDING

[ ]  Throttle .... FULL OPEN

[ ]  Carburetor Heat .... COLD

[ ]  Wing Flaps .... RETRACT TO 2nd NOTCH

[ ]  Airspeed .... 85 MPH

[ ]  Wing Flaps .... RETRACT (slowly)

NORMAL LANDING

[ ]  Touchdown .... MAIN WHEELS FIRST

[ ]  Landing Roll .... LOWER NOSE WHEEL GENTLY

[ ]  Braking .... MINIMUM REQUIRED

AFTER LANDING

[ ]  Wing Flaps .... UP

[ ]  Fuel Pump/Unnecessary Equipment .... OFF

SECURING AIRCRAFT

[ ]  Parking Brake .... SET

[ ]  Radios, Electrical Equipment .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Magnetos .... OFF

[ ]  Master Switch .... OFF

[ ]  Control Lock .... INSTALL

[ ]  Tach/Hobbs Meter .... RECORD

Piper Cherokee Airspeeds (MCAS)

NOTE: Vr 65

NOTE: Vx 74

NOTE: Vy 85

NOTE: Va 129

NOTE: Vs1 63

NOTE: Vso 54

NOTE: Vfe 115

NOTE: Vno 140

NOTE: Vne 171

NOTE: Glide 85

NOTE: XW demo 15

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-180 Cherokee Archer/pa28180_abbreviated_checklist_millstone_valley.txt

Rights: licensed

Checklist hash: cc2bf011b8bb7e710bc9f1f0803a9e0f946216b933c645946430d43054829fef

Verify against the aircraft's own approved documentation before use.