Piper PA-28-180 Cherokee Archer — pa28180_expanded_checklist_millstone_valley

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Airspeed Limitations" item count differs between passes (8 vs 6); verify nothing was dropped.

content_gap: Section "Before Takeoff Checklist" item count differs between passes (25 vs 22); verify nothing was dropped.

content_gap: Section "Cabin Fire" item count differs between passes (5 vs 4); verify nothing was dropped.

content_gap: Section "Ditching" item count differs between passes (16 vs 14); verify nothing was dropped.

content_gap: Section "Engine Failure In Flight" item count differs between passes (16 vs 15); verify nothing was dropped.

content_gap: Section "Maximum Performance Takeoff (Short Field)" item count differs between passes (16 vs 15); verify nothing was dropped.

content_gap: Section "Normal Approach and Landing Procedures" item count differs between passes (18 vs 16); verify nothing was dropped.

content_gap: Section "Normal Engine Starting Checklist" item count differs between passes (39 vs 38); verify nothing was dropped.

content_gap: Section "Normal Takeoff and Climb Procedures" item count differs between passes (12 vs 11); verify nothing was dropped.

content_gap: Section "Post-Landing Checklists" item count differs between passes (22 vs 23); verify nothing was dropped.

content_gap: Section "Pre-Flight Inspection Checklist" item count differs between passes (62 vs 59); verify nothing was dropped.

content_gap: Section "Short Field Landing" item count differs between passes (19 vs 17); verify nothing was dropped.

content_gap: Section "Soft Field Landing" item count differs between passes (19 vs 17); verify nothing was dropped.

content_gap: Section "Soft Field Takeoff" item count differs between passes (16 vs 13); verify nothing was dropped.

Airspeed Limitations

[ ]  VNE — Never Exceed Speed .... 148

[ ]  Do not exceed this speed in any operation .... 171

[ ]  VNO — Max structural cruising speed .... 121

[ ]  Do not exceed this speed except in smooth air and then only with caution .... 140

[ ]  VA — Maneuvering speed at 2,400 Pounds .... 112

[ ]  VFE — Maximum flap extended speed .... 100

[ ]  VS — Stall speed (No Flaps) .... 58

[ ]  VS0 — Stall speed in landing configuration (40° Flaps) .... 49

Optimum / Recommended Speeds

[ ]  VX — Best angle of climb .... 64

[ ]  VY — Best rate of climb .... 74

[ ]  VR — Normal rotation .... 53

[ ]  Normal climb .... 74

[ ]  Enroute climb .... 87

[ ]  Normal landing (no flaps) .... 74

[ ]  Normal landing (full flaps) .... 66

[ ]  Powered landing (no flaps) .... 74

[ ]  Powered landing (full flaps) .... 66

[ ]  Max performance approach .... 61

[ ]  Optimum glide .... 71

Maneuvering Limits

[ ]  Chandelles .... USE VA

[ ]  Lazy Eights .... USE VA

[ ]  Steep Turns .... USE VA

[ ]  Spins .... NOT AUTHORIZED

[ ]  Stalls (except whip stalls)

Engine Failure During Takeoff Run  [EMERGENCY]

[ ]  Throttle .... IDLE

[ ]  Brakes .... APPLY

[ ]  Flaps .... RETRACT

[ ]  Mixture .... IDLE CUTOFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

Engine Failure Immediately After Takeoff  [EMERGENCY]

IF ENOUGH RUNWAY REMAINING TO LAND:

[ ]  Throttle .... IDLE

[ ]  Land airplane

[ ]  Brakes .... APPLY

[ ]  Flaps .... 40°

[ ]  Mixture .... IDLE CUTOFF

[ ]  Ignition Switch .... OFF

[ ]  Master Switch .... OFF

NOT ENOUGH RUNWAY TO LAND:

[ ]  Airspeed .... 82 MPH IAS (71 KNOTS)

[ ]  Fly runway heading to emergency landing site

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Selector .... OFF

[ ]  Ignition switch .... OFF

[ ]  Flaps .... AS REQUIRED

[ ]  Master switch .... OFF

[ ]  Door .... AJAR

Engine Failure In Flight  [EMERGENCY]

[ ]  Gain all the altitude you can! Pull back (gently) to use the aircraft’s momentum to gain altitude until airspeed falls off to the optimum glide speed (82 MPH IAS; 71 Knots).

[ ]  Airspeed .... OPTIMUM GLIDE SPEED 82 MPH IAS (71 KNOTS)

[ ]  Trim the airplane for optimum glide speed

[ ]  Find a suitable place to land and fly to it. If altitude and distance to selected site permit, try to set up a normal landing pattern. If that’s not possible, take what you can get. Make sure the approach results in a landing parallel to any furrows in the selected field.

IF TIME PERMITS, TRY TO CORRECT THE PROBLEM:

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... RICH (FORWARD)

[ ]  Carburetor Heat .... ON

[ ]  Throttle .... 1/4 INCH

[ ]  Primer .... IN AND LOCKED

[ ]  Master Switch .... ON (BOTH SIDES)

[ ]  Ignition switch .... BOTH MAGNETOS — START IF PROPELLER STOPPED

IF STILL HAVE TIME COMMUNICATE:

[ ]  Transponder .... 7700

[ ]  Comm Radio .... 121.5

Emergency Landing Without Engine Power  [EMERGENCY]

FLY THE AIRPLANE

[ ]  Airspeed .... 82 MPH IAS / 71 KNOTS (FLAPS UP); 76 MPH IAS / 66 KNOTS (FLAPS DOWN)

PREPARE AIRCRAFT FOR LANDING

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Selector .... OFF

[ ]  Electric Fuel Pump .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Flaps .... AS REQUIRED (40° RECOMMENDED)

[ ]  Master Switch .... OFF

[ ]  Door .... UNLATCH PRIOR TO TOUCHDOWN

LANDING

[ ]  Touchdown .... LOWEST POSSIBLE SPEED

[ ]  Brakes .... APPLY HEAVILY

Precautionary Landing With Engine Power  [EMERGENCY]

FLY THE AIRPLANE

[ ]  Airspeed .... AS APPROPRIATE

[ ]  Flaps .... 25°

[ ]  Selected Field .... INSPECT — FLY OVER FIELD NOTING TERRAIN AND OBSTRUCTIONS, THEN RETRACT FLAPS ON REACHING SAFE ALTITUDE AND AIRSPEED.

PREPARE AIRPLANE FOR LANDING

[ ]  Radios and Electrical .... OFF

[ ]  Flaps .... AS REQUIRED

[ ]  Airspeed .... 76 MPH IAS (66 KNOTS) ON FINAL

[ ]  Master Switch .... OFF

[ ]  Door .... UNLATCH PRIOR TO TOUCHDOWN

LANDING

[ ]  Touchdown .... AS SLOW AS POSSIBLE

[ ]  Ignition Switch .... OFF

[ ]  Brakes .... APPLY HEAVILY

Ditching  [EMERGENCY]

PREPARE FOR DITCHING

[ ]  Radio .... TRANSMIT MAYDAY ON 121.5

NOTE: Give location, situation and intentions. If already communicating with ATC, report situation to controller rather than using 121.5.

[ ]  Transponder .... 7700

[ ]  Heavy Baggage .... SECURE OR JETTISON

FLY THE AIRPLANE

[ ]  Approach — High wind / Heavy seas .... INTO THE WIND

[ ]  Approach — Light winds / Heavy swells .... PARALLEL TO THE SWELLS

[ ]  Flaps .... 25° - 40°

[ ]  Power .... 300 FT/MIN DESCENT, 76 MPH IAS (66 KNOTS)

[ ]  Cabin Doors .... UNLATCH PRIOR TO TOUCHDOWN

LANDING

[ ]  Touchdown .... LEVEL ATTITUDE AT 300 FT/MIN DESCENT

[ ]  Face .... CUSHION WITH FOLDED COAT

[ ]  Evacuate .... THROUGH DOOR — IF NECESSARY, OPEN WINDOW TO ALLOW CABIN TO FLOOD TO EQUALIZE PRESSURE SO DOOR CAN BE OPENED.

[ ]  Life Vests and Raft .... INFLATE

Engine Fire During Start Up  [EMERGENCY]

IF ENGINE HAS NOT STARTED

[ ]  Mixture .... IDLE CUTOFF

[ ]  Throttle .... FULL OPEN

[ ]  Crank engine with starter to suck fire into the engine

IF ENGINE HAS STARTED

[ ]  Continue running to try to suck fire into the engine

IN EITHER CASE IF FIRE IS NOT OUT IN A FEW SECONDS

[ ]  Fuel .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Use fire extinguisher

Engine Fire In Flight  [EMERGENCY]

[ ]  Fuel Selector .... OFF

[ ]  Throttle .... CLOSED

[ ]  Mixture .... IDLE CUTOFF

[ ]  Heater .... OFF

[ ]  Defroster .... OFF

[ ]  Landing .... FORCED LANDING WITHOUT POWER

Electrical Fire In Flight  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Vents .... OPEN

[ ]  Cabin Heat/Defroster .... OFF

[ ]  Landing .... AS SOON AS POSSIBLE

Cabin Fire  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Vents, Cabin Heat/Air .... CLOSED

[ ]  Fire Extinguisher .... ACTIVATE

NOTE: After using fire extinguisher within a closed cabin ventilate the cabin.

[ ]  Landing .... AS SOON AS POSSIBLE

Wing Fire  [EMERGENCY]

[ ]  Navigation lights .... OFF

[ ]  Strobe Lights .... OFF

[ ]  Pitot Heat .... OFF

[ ]  Attitude .... PERFORM SIDE-SLIP TO KEEP FLAMES AWAY FROM FUEL TANK AND CABIN

[ ]  Land .... ASAP — DO NOT USE FLAPS

Pre-Flight Inspection Checklist

WING TOPS/FUEL TANKS

[ ]  Fuel Level — Both Wings - Visual check

NOTE: If needed get gas (AVGAS 100) before proceeding with other fuel tank related items.

[ ]  Filler caps — Both Wings - Secure

[ ]  Wing Tops — Inspect for loose screws, rivets and damage

COCKPIT

[ ]  Control wheel lock .... REMOVE

[ ]  Ignition switch .... OFF

[ ]  Master switch .... ON (BOTH SIDES)

[ ]  Fuel gauges — Check quantity

[ ]  Flaps .... 10°

[ ]  Pitot Heat .... ON - OBSERVE AMMETER DROP - THEN OFF

[ ]  Strobe/Beacon .... ON - VISUALLY CHECK - OFF

[ ]  Master Switch .... OFF

[ ]  Fuel tank selector .... ON FULLEST TANK

PAPERWORK:

[ ]  Airworthiness certificate

[ ]  Registration

[ ]  Operating limitations (POH)

[ ]  Weight/loading data

COCKPIT - NIGHT FLIGHTS

[ ]  Nav Lights & Strobes .... ON

NOTE: Walk around plane and visually check to see that all are operating.

[ ]  Landing Light .... ON

NOTE: Visually check from outside if not dark enough to see that it’s on from inside the cockpit.

[ ]  Instrument Lights .... ON

[ ]  Light switches .... OFF (EXCEPT BEACON)

[ ]  Master switch .... OFF

RIGHT WING

[ ]  Flap — Check freedom of movement and security.

[ ]  Aileron — Check security and freedom of movement

[ ]  Wing tip — Check for cracks; Check light security.

[ ]  Leading edge — Look for dents

[ ]  Fuel sump — Check sample for color, water and dirt.

[ ]  Main landing gear — Check tire wear, brake pads, leaking brake fluid.

NOSE

[ ]  Cowling — Check secured.

[ ]  Oil .... 8 QUARTS MAX; 6 QUARTS MINIMUM.

[ ]  Nose gear — Check tire wear; Strut inflation; security

[ ]  Engine compartment — Check for bird’s nests

[ ]  Propeller — Check for nicks and security;

[ ]  Spinner — Check security

[ ]  Landing light — Check security

[ ]  Air Filter — Check clean and security

[ ]  Fuel sump — Check sample for color, water and dirt.

LEFT WING

[ ]  Leading edge — Look for dents

[ ]  Main landing gear — Check tire wear, brake pads, leaking brake fluid.

[ ]  Fuel sump — Check sample for color, water and dirt.

[ ]  Pitot/Static — Check cover off; Check holes not obstructed.

[ ]  Wing tip — Check for cracks; Check light security.

[ ]  Aileron — Check security and freedom of movement.

[ ]  Flap — Check freedom of movement and security.

[ ]  Stall Warning Vane — Check for free operation

FUSELAGE – LEFT SIDE

[ ]  Check for loose rivets, screws and damage.

EMPENNAGE

[ ]  Stabilator — Check security and freedom of movement.

[ ]  Rudder — Check security and freedom of movement.

FUSELAGE – RIGHT SIDE

[ ]  Check for loose rivets, screws and damage

[ ]  Baggage Door .... CLOSED AND LOCKED

Normal Engine Starting Checklist

BEFORE STARTING

[ ]  Preflight Inspection .... COMPLETED

[ ]  Seat position — Adjust & ensure locked

[ ]  Seat belts/harness — Adjust and lock

NOTE: Brief passengers on use of belts/harnesses and requirements for wearing them.

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Radios/electrical .... OFF

[ ]  Autopilot .... OFF

[ ]  Brakes — Test and set

[ ]  Circuit Breakers — Check all in

STARTING ENGINE

[ ]  Mixture .... RICH (FORWARD)

[ ]  Carburetor heat .... COLD (UP)

[ ]  Primer — Prime if required — Make sure locked in

[ ]  Throttle .... 1/4 INCH

[ ]  Key .... IN IGNITION

[ ]  Master Switch .... ON (BOTH SIDES)

[ ]  Fuel Pump .... ON

[ ]  Propeller Area — Call “Clear” & check prop area and behind plane

[ ]  Ignition .... START - RELEASE ON START

[ ]  Throttle .... 1,000 RPM

[ ]  Oil Pressure — Check in green

BEFORE TAXIING

[ ]  Radios — On and set to appropriate frequency. Call for radio check

[ ]  Transponder .... STANDBY

[ ]  Beacon/Strobe .... ON

[ ]  Nav. Lights/Strobes — On if required

[ ]  Flaps .... FULL UP (NORMAL TAKEOFF)

TAXIING

[ ]  Clearance — Check for things in way of wings — Check for people ahead of and behind plane

[ ]  Flight Controls — Set for existing wind conditions

[ ]  Brakes — Come to full stop immediately after starting taxi roll

IFR INSTRUMENT CHECKS

[ ]  Turn Coordinator — Should indicate turn in proper direction while taxiing.

[ ]  Attitude Indicator — Very little change due to turns; Slight pitch indications due to acceleration or deceleration.

[ ]  Heading Indicator — Should track headings.

[ ]  Altimeter — When set to current altimeter setting should indicate within 75 ft. of airport elevation.

[ ]  VSI — Should indicate zero. If not, note indication and use for level indication in flight.

[ ]  VORs — Check at local ground check point or against each other based on some receivable signal.

Before Takeoff Checklist

FINAL COCKPIT CHECK

[ ]  Cabin door .... CLOSED AND LATCHED

[ ]  Flight Controls — Free and correct

[ ]  Elevator trim .... TAKEOFF POSITION

[ ]  Rudder trim .... TAKEOFF POSITION

[ ]  Flight Instruments — Check and set

NOTE: Set attitude indicator to level flight position. Set altimeter to runway altitude or locally reported altimeter setting. Set heading indicator to magnetic compass.

[ ]  Comm Radio/VOR — Set to appropriate freqs

[ ]  Beacon/Strobe .... ON

[ ]  Nav Lights/Strobes — On if required

[ ]  Autopilot .... OFF

ENGINE RUN-UP

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Mixture .... RICH (FORWARD)

[ ]  Parking brake — Set or hold foot brakes

[ ]  Throttle .... 1,800 RPM

[ ]  Magnetos — Check

NOTE: RPM drop should not exceed 125 RPM on either magneto. RPM difference between magnetos should not exceed 50 RPM.

[ ]  Carburetor Heat .... ON — CHECK FOR RPM DROP THEN BACK TO OFF

[ ]  Engine instruments — Check

NOTE: Oil pressure/Temperature; Suction; Ammeter - Create electrical load with landing light. Make sure no more than needle width deflection.

[ ]  Fuel Pump — Momentarily Off; Check for good fuel pressure; On

[ ]  Throttle .... 1,000 RPM

[ ]  Throttle quadrant lock — Adjust

[ ]  Transponder .... SET TO MODE C/ALTITUDE

Normal Takeoff and Climb Procedures

[ ]  Flaps .... FULL UP

[ ]  Carburetor Heat .... COLD (UP)

[ ]  Fuel Pump .... ON

[ ]  Elevator Trim .... TAKEOFF POSITION

[ ]  Rudder Trim .... TAKEOFF POSITION

[ ]  Heading indicator — Calibrate against compass

[ ]  Throttle .... FULL OPEN (FORWARD)

[ ]  Engine Instruments — Check while starting roll

NOTE: RPM – In the green arc; Oil Pressure - In the green; Oil Temperature - In the green; Suction - In the green

[ ]  Airspeed — Building

[ ]  Elevator .... LIFT NOSE WHEEL AT 60 MPH (52 KIAS)

[ ]  Climb Speed .... 85 MPH (74 KIAS)

Maximum Performance Takeoff (Short Field)

[ ]  Taxi — Maximum runway usage — Takeoff procedure should be started using ALL available runway. Taxi to end of runway and align with centerline.

[ ]  Brakes — Set and hold

[ ]  Flaps .... 25°

[ ]  Carburetor Heat .... COLD (UP)

[ ]  Fuel Pump .... ON

[ ]  Elevator Trim .... TAKEOFF POSITION

[ ]  Rudder Trim .... TAKEOFF POSITION

[ ]  Heading indicator — Calibrate against compass

[ ]  Throttle .... FULL OPEN (IN)

[ ]  Engine Instruments — Check before starting roll

NOTE: RPM – In the green arc; Oil Pressure - In the green; Oil Temperature - In the green; Suction - In the green

[ ]  Brakes .... RELEASE

[ ]  Airspeed — Building

[ ]  Elevator .... SLIGHTLY TAIL LOW

[ ]  Climb Speed .... 74 MPH (65 KIAS) UNTIL OBSTACLES CLEARED, THEN 85 MPH (74 KIAS)

[ ]  Flaps — Retract at safe altitude with positive rate of climb.

Soft Field Takeoff

[ ]  Taxi — Keep rolling to avoid bogging down

[ ]  Flaps .... 25°

NOTE: If 25° flaps are used, with obstacles ahead, leave them extended until the obstacle is cleared and at a safe altitude.

[ ]  Carburetor Heat .... COLD (UP)

[ ]  Fuel Pump .... ON

[ ]  Elevator Trim .... TAKEOFF POSITION

[ ]  Rudder Trim .... TAKEOFF POSITION

[ ]  Heading indicator — Calibrate against compass or runway heading

[ ]  Throttle .... FULL OPEN (IN)

[ ]  Engine Instruments — Check as starting roll

NOTE: RPM – In the green arc; Oil Pressure - In the green; Oil Temperature - In the green; Suction - In the green

[ ]  Airspeed — Building

[ ]  Elevator .... SLIGHTLY TAIL LOW

NOTE: Allow the airplane to lift off as soon as possible (before reaching safe climb speed). Level off at a few feet above the ground and fly in ground effect until reaching normal climb speed.

[ ]  Climb Speed .... 74 MPH (65 KIAS) UNTIL OBSTACLES CLEARED, THEN 85 MPH (74 KIAS)

[ ]  Flaps — Retract at safe altitude with positive rate of climb.

After Takeoff Checklist

CLIMBOUT

[ ]  Airspeed .... 85 MPH (74 KIAS)

[ ]  Altitude .... ABOVE 300 AGL

[ ]  Flaps .... UP (ONE CLICK AT A TIME)

[ ]  Fuel Pump — Off at 1,000 AGL; Check fuel pressure

AT CRUISE ALTITUDE

[ ]  Attitude .... LEVEL

[ ]  Airspeed — Let build to desired cruise speed

[ ]  Throttle — Reduce to desired cruise setting

[ ]  Heading Indicator — Calibrate against compass

ABOVE 5,000 MSL

[ ]  Mixture .... LEAN FOR MAXIMUM RPM

Enroute Climb

[ ]  Normal Airspeed .... 100 MPH (87 KIAS)

[ ]  Max Performance .... SEE POH CLIMB TABLE IN SECTION IV

[ ]  Throttle .... FULL OPEN (FORWARD)

[ ]  Carburetor Heat .... COLD (UP)

[ ]  Mixture .... RICH (FORWARD) BELOW 5,000 FT. LEANED FOR MAXIMUM RPM ABOVE 5,000.

Normal Approach and Landing Procedures

PRE-LANDING (DOWNWIND) CHECK

[ ]  Seat belts/Harnesses — Adjust and lock

[ ]  Mixture .... RICH (FORWARD)

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Fuel Pump — On; Check fuel pressure

[ ]  Autopilot .... OFF

APPROACH AND LANDING

[ ]  Power — Reduce to 1,300 to 1,500 RPM abeam approach end of runway

[ ]  Airspeed — Let bleed off to less than 115 MPH (100 KIAS)

[ ]  Flaps — Use as desired

NOTE: Under light (less than 10 Knots) wind conditions 10° descending on the end of the downwind leg, 25° on base, and full flaps over the threshold as required. In heavier winds 25° or less is good flap setting for landing. Use minimum flap setting possible for cross wind landing.

AIRSPEEDS

[ ]  Downwind through base .... 90 MPH (78 KIAS)

[ ]  Final approach .... 80 - 85 MPH (70 – 74 KIAS)

NOTE: In gusty winds add 1/2 difference between gust and average wind speed to approach speed.

[ ]  Touchdown — Just above stalling speed - main wheels first.

[ ]  Landing Roll — Lower nose wheel gently

[ ]  Braking .... MINIMUM REQUIRED

Short Field Landing

PRE-LANDING (DOWNWIND) CHECK

[ ]  Seat belts/Harnesses — Adjust and lock

[ ]  Mixture .... RICH (FORWARD)

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Fuel Pump — On; Check fuel pressure

[ ]  Autopilot .... OFF

APPROACH AND LANDING

[ ]  Power — Reduce to 1,300 to 1,500 RPM abeam approach end of runway

[ ]  Airspeed — Let bleed off to less than 115 MPH (100 KIAS)

[ ]  Flaps

NOTE: Under light (less than 10 Knots) wind conditions 10° descending on the end of the downwind leg, 25° on base, and full flaps on final.

AIRSPEEDS

[ ]  Downwind through base .... 90 MPH (78 KIAS)

[ ]  Final approach .... 70 - 75 MPH (61 – 65 KIAS)

NOTE: In gusty winds add 1/2 difference between gust and average wind speed to approach speed.

[ ]  Touchdown — Roundout must be done much faster than usual due to low airspeed.

[ ]  Landing Roll — Hold Nose off as long as possible.

[ ]  Braking .... MAXIMUM POSSIBLE WITHOUT SLIDING TIRES; HOLD FULL UP ELEVATOR

[ ]  Flaps .... RETRACT

Soft Field Landing

PRE-LANDING (DOWNWIND) CHECK

[ ]  Seat belts/Harnesses — Adjust and lock

[ ]  Mixture .... RICH (FORWARD)

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Fuel Pump — On; Check fuel pressure

[ ]  Autopilot .... OFF

APPROACH AND LANDING

[ ]  Power — Reduce to 1,300 to 1,500 RPM abeam approach end of runway

[ ]  Airspeed — Let bleed off to less than 115 MPH (100 KIAS)

[ ]  Flaps — Use as desired

NOTE: Under light (less than 10 Knots) wind conditions 10° descending on the end of the downwind leg, 25° on base, and full flaps over the threshold as required. In heavier winds 25° or less is good flap setting for landing. Use minimum flap setting possible for cross wind landing.

AIRSPEEDS

[ ]  Downwind through base .... 90 MPH (78 KIAS)

[ ]  Final approach .... 80 - 85 MPH (70 – 74 KIAS)

NOTE: In gusty winds add 1/2 difference between gust and average wind speed to approach speed.

[ ]  Touchdown — Just above stalling speed - main wheels first.

[ ]  Landing Roll — Hold nose wheel off as long as possible

[ ]  Braking .... MINIMUM REQUIRED

[ ]  Flight Controls .... FULL UP ELEVATOR

Balked Landing (Go Around)

[ ]  Throttle .... FULL OPEN (IN)

[ ]  Carburetor Heat .... COLD (UP)

[ ]  Flaps — Raise in increments once a positive rate of climb and safe airspeed are obtained

[ ]  Airspeed .... 85 MPH (74 KIAS)

Post-Landing Checklists

AFTER LANDING - CLEAR OF RUNWAY

[ ]  Flaps .... FULL UP

[ ]  Carburetor Heat .... OFF (UP)

[ ]  Fuel Pump .... OFF

[ ]  Elevator Trim .... TAKEOFF POSITION

[ ]  Rudder Trim .... TAKEOFF POSITION

IF HARD LANDING

[ ]  ELT — Listen for on 121.5 on communications radio

ENGINE SHUTDOWN

[ ]  Radios/Electrical .... ALL OFF

[ ]  Throttle .... 1,000 RPM

[ ]  Master Switch .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Ignition .... OFF

SECURING THE AIRPLANE

[ ]  Control Lock .... INSTALL

[ ]  Tiedown .... WINGS AND TAIL

[ ]  Pitot Cover .... INSTALL

DOUBLE CHECK

[ ]  All electrical equipment - Off

[ ]  Master Switch .... OFF

CLOSE YOUR FLIGHT PLAN

Before Leaving Home

SELF CHECK

[ ]  Feeling ok .... YES

[ ]  Under any stress .... NO MORE THAN USUAL

[ ]  Taking any medication .... NO

[ ]  Alcohol in last 12 Hrs. .... NO

FLIGHT PLANNING/NAVIGATION EQUIPMENT

[ ]  Current Charts

[ ]  A/FD

[ ]  POH

[ ]  Airport Guide

[ ]  E6-B

[ ]  Plotter

[ ]  Calculator

[ ]  Timer

[ ]  Custom Checklists

[ ]  Flight Plans

[ ]  Weather Reports

[ ]  Pencils

[ ]  Clipboards

EMERGENCY ITEMS

[ ]  Hand Compass

[ ]  Knife

[ ]  Flashlights

[ ]  Batteries

[ ]  Bulbs

[ ]  Cell Phone (Charged)

[ ]  Spare Glasses

[ ]  Sun Glasses

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-180 Cherokee Archer/pa28180_expanded_checklist_millstone_valley.txt

Rights: licensed

Checklist hash: f904c46f685acddc9f8ba130212d95ac762e9a9cadd14b3991197adbc359b9f1

Verify against the aircraft's own approved documentation before use.