Piper PA-28-181 Archer II_III — carl_kintanar_archer_2_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Cockpit

[ ]  Weight/Balance .... IN LIMITS

[ ]  Line Box .... LOCKED

[ ]  Parking Brake .... SET

[ ]  Control Lock .... REMOVE

[ ]  Ignition .... OFF

[ ]  Ignition Key .... ON DASH

[ ]  Mag Compass .... CHECK

[ ]  Hobbs .... RECORD

[ ]  Tach .... VERIFY

[ ]  Transponder .... OFF

[ ]  Fan .... OFF

[ ]  Pitot Heat .... OFF

[ ]  All Lights .... OFF

[ ]  Avionics .... OFF

[ ]  Master Switch .... ON

[ ]  Annunciator Panel .... TEST

[ ]  Fuel Quantity .... CHECK

[ ]  Exterior Lights .... CHECK

[ ]  Master Switch .... OFF

[ ]  Flaps .... 10 deg

[ ]  Pitot/Static Drains .... CLOSE

[ ]  Aircraft Documents .... ON BOARD

Exterior Inspection

[ ]  Right Flap .... CHECK

[ ]  Aileron .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Fuel Sump .... SAMPLE

[ ]  Fuel Vent .... CLEAR

[ ]  Wing Leading Edge .... CHECK

[ ]  Air Inlet .... CHECK

[ ]  Wheel/Brake .... CHECK

[ ]  Chocks .... REMOVE

[ ]  Engine Oil Level .... MIN 5 qts

[ ]  Right Cowling .... SECURE

[ ]  Propeller/Spinner .... CHECK

[ ]  Engine Air Intake .... CLEAR

[ ]  Alternator Belt .... CHECK

[ ]  Landing Light .... CHECK

[ ]  Carb Air Filter .... CHECK

[ ]  Nose Gear .... CHECK

[ ]  Nose Chocks .... REMOVE

[ ]  Left Cowling .... SECURE

[ ]  Fuel Strainer (Nose) .... SAMPLE

[ ]  Wing Leading Edge .... CHECK

[ ]  Air Inlet .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Fuel Sump .... SAMPLE

[ ]  Wheel/Brake .... CHECK

[ ]  Chocks .... REMOVE

[ ]  Pitot/Static Head .... CLEAR

[ ]  Fuel Vent .... CLEAR

[ ]  Stall Opening .... CLEAR

[ ]  Aileron .... CHECK

[ ]  Flap .... CHECK

[ ]  Fuselage Left Side .... CHECK

[ ]  Antennas .... CHECK

[ ]  Elevator .... CHECK

[ ]  Elevator Trim .... CHECK

[ ]  Rudder .... CHECK

[ ]  Rudder Trim .... CHECK

[ ]  Tail Tie Down .... REMOVE

[ ]  Fuselage Right Side .... CHECK

Before Engine Start

[ ]  Flaps .... UP

[ ]  Seats .... ADJUSTED

[ ]  Seatbelts/Shoulder .... FASTENED

[ ]  Master Switch .... ON

[ ]  Anti Collision Lights .... ON

[ ]  Fuel Selector .... LOWEST

[ ]  Parking Brake .... SET

[ ]  Brake .... SET

[ ]  Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Throttle .... ¼” OPEN

[ ]  Prime .... AS REQUIRED

[ ]  Prop Area .... CLEAR!

[ ]  Starter .... ENGAGE

[ ]  Throttle .... 1000 RPM

[ ]  Oil Pressure .... CHECK

[ ]  Engine Instruments .... CHECK

[ ]  Fuel Pump .... OFF

[ ]  Primer .... LOCKED

Before Taxi

[ ]  Fan .... AS REQUIRED

[ ]  Defrost .... AS REQUIRED

[ ]  Cabin Heat .... AS REQUIRED

[ ]  Pitot Heat .... AS REQUIRED

[ ]  Annunciator .... TEST

[ ]  Avionics .... ON

[ ]  Comm Radios .... SET

[ ]  Rnav/Nav Mode .... SET

[ ]  Nav Radios .... SET

[ ]  Transponder .... STDBY 1200

[ ]  Circuit Breakers .... IN

[ ]  Fuel Selector .... FULLEST

[ ]  ATIS/Weather .... CHECK

[ ]  Taxi Clearance .... AS REQUIRED

[ ]  Parking Brake .... RELEASE

[ ]  Brakes .... CHECK

Before Takeoff

[ ]  Parking Brake .... SET

[ ]  Brakes .... SET

[ ]  Mixture .... RICH

[ ]  Throttle .... 2000 RPM

[ ]  Engine Instruments .... IN LIMITS

[ ]  Suction .... CHECK

[ ]  Ammeter .... CHECK

[ ]  Magnetos .... CHECK

[ ]  Carb Heat .... CHECK

[ ]  Throttle .... IDLE

[ ]  RPM/Oil Pressure .... IN LIMITS

[ ]  Throttle .... 1000 RPM

[ ]  Flight Controls .... FREE / CORRECT

[ ]  Flight Instruments .... CHECK / SET

[ ]  Comm Radios .... SET

[ ]  Flaps .... UP

[ ]  Trim .... SET

[ ]  Takeoff Briefing .... COMPLETE

[ ]  Door and Window .... SECURE

[ ]  Autopilot .... OFF

[ ]  Fuel Pump .... ON

[ ]  Strobes .... ON

[ ]  Landing Light .... ON

[ ]  Transponder .... ALT 1200

[ ]  Parking Brake .... RELEASE

[ ]  Takeoff Clearance .... AS REQUIRED

After Takeoff

[ ]  Flaps .... UP

[ ]  Fuel Pump .... OFF

Cruise

[ ]  Throttle .... CRUISE RPM

[ ]  Mixture .... LEAN

[ ]  Landing Light .... AS REQUIRED

[ ]  Fuel Quantity .... CHECK

[ ]  Fuel Selector .... SET

[ ]  Engine Instruments .... CHECK

[ ]  Flight Instruments .... CHECK

Approach

[ ]  ATIS/Weather .... CHECK

[ ]  Comm Radios .... SET

[ ]  Nav Radios .... SET

[ ]  Flight Instruments .... CHECK / SET

[ ]  Engine Instruments .... CHECK

[ ]  Landing Light .... ON

[ ]  Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Fuel Selector .... FULLEST

[ ]  Approach Briefing .... COMPLETE

[ ]  Seats .... ADJUSTED

[ ]  Seat Belts .... FASTENED

[ ]  Autopilot .... OFF

After Landing

[ ]  Transponder .... OFF

[ ]  Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Strobes .... OFF

[ ]  Flaps .... UP

[ ]  Taxi Clearance .... AS REQUIRED

Parking

[ ]  Parking Brake .... SET

[ ]  Avionics .... OFF

[ ]  Pitot Heat .... OFF

[ ]  All Lights .... OFF

[ ]  Fan .... OFF

[ ]  Carb Heat .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... CUT OFF

[ ]  Master Switch .... OFF

[ ]  Ignition .... OFF

Securing Aircraft

[ ]  Flight Plan .... CLOSED

[ ]  Tie Downs .... SECURE

[ ]  Chocks .... SECURE

[ ]  Door and Window .... SECURE

[ ]  Keys .... SECURE

[ ]  Fuel .... FULL (DPA)

[ ]  Hobbs .... RECORD (DPA)

[ ]  Tach .... RECORD (DPA)

[ ]  Flight Controls .... SECURE (DPA)

[ ]  Extra Oil .... REPLACE (DPA)

[ ]  Fuel Card .... IN LOGBOOK (DPA)

[ ]  Logbook .... LINEBOX? (DPA)

[ ]  Line Box .... LOCKED (DPA)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-181 Archer II_III/carl_kintanar_archer_2_checklist.txt

Rights: licensed

Checklist hash: 5b7130040b76f247e5fa27ace4b347476f231014184a5e4604cf2bd651a1183e

Verify against the aircraft's own approved documentation before use.