Piper PA-28-181 Archer II_III — peter_pa28-simulator-flight-planner

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Flight Planning Sheet" item count differs between passes (123 vs 120); verify nothing was dropped.

content_gap: Section "General Specifications" item count differs between passes (35 vs 42); verify nothing was dropped.

General Specifications

1. WEIGHT

SYMBOL | DESCRIPTION | VALUE [LBS]

[ ]  MTOW maximum takeoff .... 2550 (normal); 2130 (utility)

[ ]  MLW maximum landing .... 2550 (normal); 2130 (utility)

[ ]  Mbc maximum baggage compartment .... 200 (normal); 0 (utility)

[ ]  Wb (unusable fuel + fluids) basic empty .... 1604

[ ]  MUW (MTOW - Ws) maximum useful .... 946 (normal); 526 (utility)

NOTE: utility = no rear passengers; no (baggage + cargo)

NOTE: useful load normal = usable fuel + front passengers + rear passengers + (baggage + cargo)

NOTE: useful load utility = usable fuel + front passengers

2. SPEED

SYMBOL | DESCRIPTION | SPEED VALUES [KIAS]

[ ]  Va maneuvering .... 113 (2550 GW); 89 (1634 GW)

[ ]  Vfe max flap extended .... 102

[ ]  Vne never exceed .... 154

[ ]  Vno structural cruising (> in smooth air) .... 125

[ ]  Vs stalling speed .... fig. 5-3

[ ]  Vso Vs/Vs in landing config. .... n/a

[ ]  Vx best angle of climb .... 64

[ ]  Vy best rate of climb .... 76

[ ]  Vapp (full flaps) final approach speed .... 66

[ ]  Vcrosswind max crosswind .... 17

[ ]  Green arc operational range .... 55-125

[ ]  White arc flaps range .... 49-102

3. CENTER OF GRAVITY

NOTE: datum @ WLE + 78.4"

[ ]  NORMAL 2550 lbs .... 88.6" aft / 82" aft

[ ]  NORMAL < 2050 lbs .... 93" aft / 93" aft

[ ]  UTILITY 2130 lbs .... 82" aft / 83" aft

[ ]  UTILITY < 2050 lbs .... 93" aft / 93" aft

4. FUEL

CONDITION | VALUE [US GAL]

[ ]  max .... 50

[ ]  unusable .... 2

[ ]  usable .... 48

Flight Planning Sheet

STEP | VALUES

0. LOAD CONFIGURATION

1. CONFIG-O-MATIC

ITEM | WEIGHT [LBS]

[ ]  standard empty .... —

[ ]  operational equipment .... —

[ ]  basic empty .... —

[ ]  pilot .... —

[ ]  passenger 1 (co-pilot) .... —

[ ]  passenger 2 (rear) .... —

[ ]  passenger 3 (rear) .... —

[ ]  baggage + cargo .... —

[ ]  MAXIMUM USABLE FUEL .... —

[ ]  MAXIMUM GROSS .... —

[ ]  DRY .... —

2. CLIMB

A. CLIMB PERFORMANCE

STATE | ITEM | VALUE

[ ]  INPUT OAT [°C] .... —

[ ]  Hp [ft] .... —

[ ]  OUTPUT V/S [ft/min] .... —

B. CLIMB TIME, DISTANCE, FUEL

STATE | ITEM | VALUE

[ ]  INPUT OAT [°C] departure .... —

[ ]  Hp [ft] departure .... —

[ ]  OAT [°C] cruise .... —

[ ]  Hp [ft] cruise .... —

[ ]  OUTPUT CORRECTIONS climb time departure [min] .... —

[ ]  climb distance departure [mi] .... —

[ ]  climb fuel departure [gal] .... —

[ ]  climb time cruise [min] .... —

[ ]  climb distance cruise [mi] .... —

[ ]  climb fuel cruise [gal] .... —

[ ]  OUTPUT climb time [min] .... 0

[ ]  climb distance [mi] .... 0

[ ]  climb fuel [gal] .... 0

3. DESCENT

STATE | ITEM | VALUE

[ ]  INPUT OAT [°C] arrival .... —

[ ]  Hp [ft] arrival .... —

[ ]  OAT [°C] cruise .... 0

[ ]  Hp [ft] cruise .... 0

[ ]  OUTPUT CORRECTIONS descent time arrival [min] .... —

[ ]  descent distance arrival [mi] .... —

[ ]  descent fuel arrival [gal] .... —

[ ]  descent time cruise [min] .... —

[ ]  descent distance cruise [mi] .... —

[ ]  descent fuel cruise [gal] .... —

[ ]  OUTPUT descent time [min] .... 0

[ ]  descent distance [mi] .... 0

[ ]  descent fuel [gal] .... 0

4. CRUISE

A. DISTANCE [MI]

TRIP DISTANCE [MI] | CLIMB DISTANCE [MI] | DESCENT DISTANCE [MI] | CRUISE DISTANCE [MI]

[ ]  TRIP DISTANCE [mi] .... 0

[ ]  CLIMB DISTANCE [mi] .... 0

[ ]  DESCENT DISTANCE [mi] .... 0

[ ]  CRUISE DISTANCE [mi] .... 0

B. TAS [KTS]

STATE | ITEM | VALUE | CRUISE TYPE [P/E]

[ ]  INPUT OAT [°C] cruise .... 0

[ ]  Hp [ft] cruise .... 0

[ ]  Cruise Power [65 %, 70 %, 75 %] .... —

[ ]  OUTPUT TAS [kts] .... —

C. TIME [HR]

CRUISE DISTANCE [MI] | TAS [KTS] | TAS [MI/HR] | CRUISE TIME [HR]

[ ]  CRUISE DISTANCE [mi] .... 0

[ ]  TAS [kts] .... 0

[ ]  TAS [mi/hr] .... 0

[ ]  CRUISE TIME [hr] .... 7

D. FUEL [GAL]

CRUISE POWER [%] | CRUISE TYPE [P/E] | FUEL FLOW [GPH] | CRUISE FUEL [GAL]

[ ]  CRUISE POWER [%] .... 0

[ ]  CRUISE TYPE [P/E] .... 0

[ ]  FUEL FLOW [gph] .... —

[ ]  CRUISE FUEL [gal] .... 7

E. MIXTURE RANGE [NM]

CRUISE OAT [°C] | CRUISE HP [FT] | CRUISE POWER [%] | CRUISE P/E RANGE [NM]

[ ]  CRUISE OAT [°C] .... 0

[ ]  CRUISE Hp [ft] .... 0

[ ]  CRUISE POWER [%] .... 0

[ ]  CRUISE P/E RANGE [nm] .... —

5. TOTAL FLIGHT TIME

[ ]  TOTAL FLIGHT TIME [hr] .... 7

6. TOTAL FLIGHT FUEL

[ ]  TOTAL FLIGHT FUEL [gal] .... 7

7. FLIGHT SIM FUEL

CAPACITY [LBS] | LEFT WING FUEL [LBS] | RIGHT WING FUEL [LBS]

[ ]  CAPACITY [lbs] .... 288

[ ]  LEFT WING FUEL [lbs] .... 7

[ ]  RIGHT WING FUEL [lbs] .... 7

8. LOADING CONSTRAINTS

ITEM | WEIGHT [LBS] | ARM AFT DATUM [IN] | MOMENT [LB*IN]

[ ]  basic empty .... 0 / 87.6 / —

[ ]  pilot + passenger 1 (front) .... 0 / 80.5 / —

[ ]  passenger 2 + passenger 3 (rear) .... 0 / 118.1 / —

[ ]  baggage + cargo .... 0 / 142.8 / —

[ ]  total flight fuel .... 7 / 95 / —

[ ]  ACTUAL GROSS .... 7 / — / 0

A. WEIGHT CONSTRAINT

NOTE: The actual gross weight must be < 2550 lbs (normal) and < 2130 lbs (utility).

A. CG CONSTRAINT

[ ]  CG POSITION THEO. [in] .... —

[ ]  CG POSITION GRAPH [in] .... 7

NOTE: The CG position must adhere to the value range from the "General Specifications" data sheet with operational maxima and minima shown on the graph.

9. TAKEOFF

STATE | ITEM | VALUE

[ ]  INPUT OAT [°C] departure .... 0

[ ]  Hp [ft] departure .... 0

[ ]  Headwind (H) / Tailwind (T) [kts] .... —

[ ]  Actual Gross Weight [lbs] .... 7

[ ]  Flaps 0/25 [°] .... —

[ ]  Performance (P) / Ground Roll (G) .... —

[ ]  OUTPUT minimum takeoff distance [ft] .... —

10. LANDING

STATE | ITEM | VALUE

[ ]  INPUT OAT [°C] arrival .... 0

[ ]  Hp [ft] arrival .... 0

[ ]  Headwind (H) / Tailwind (T) [kts] .... —

[ ]  Landing Gross Weight [lbs] .... 7

[ ]  Performance (P) / Ground Roll (G) .... —

[ ]  OUTPUT minimum landing distance [ft] .... —

11. EXTRA

Credits and version number

NOTE: All information in this worbook was based on the Piper Archer II manuals by Dreamfleet.

WARNING: All information contained herein is only to be used for flight simulation entertainment purposes and NOT real world flying!

NOTE: The coding and layout of the document was made by me, Peter.

NOTE: I am in no way responsible for the manner in which you use this document and am not responsible for any damages it may incurr with your software or hardware.

NOTE: You are free to modify certain things (but wouldn't recomment it due to the programming and cell detail) but cannot distribute this document for financial gain.

NOTE: If you choose to upload the document with your modifications to a flight simulator enthusiasts website, I require credit for my work cause this took some time.

NOTE: Enjoy! Peter

NOTE: VERSION : 1.0

NOTE: 37377

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-181 Archer II_III/peter_pa28-simulator-flight-planner.txt

Rights: licensed

Checklist hash: 36421b6802f26c5f9b3b57ee27f1fce8afb597b655638c5fb4061c2627fae245

Verify against the aircraft's own approved documentation before use.