Piper PA-28-181 Archer II_III — Roland_Bialy_PA28-181_checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Before TAKE OFF" item count differs between passes (22 vs 23); verify nothing was dropped.

content_gap: Section "PRE - FLIGHT PA 28-181" item count differs between passes (12 vs 11); verify nothing was dropped.

PRE - FLIGHT PA 28-181

PREFLIGHT

[ ]  Seat Belts, Shoulder Harness .... ON

[ ]  Pax

[ ]  Radios / NAV / Avionics .... OFF

[ ]  Circuit Breakers .... IN

[ ]  Fuel selector .... FULLEST TANK

[ ]  Park Brake .... SET

[ ]  Flaps Full Range

[ ]  Trim

[ ]  Timer

[ ]  Freq XPDR ADF VOR BUGS .... SET

[ ]  Carb Heat .... OFF

Engine Start

[ ]  Primer

[ ]  Rotating Beacon .... ON

[ ]  Master Switch .... ON

[ ]  Prop Area .... CLEAR

[ ]  Electrical Fuel Pump .... ON

[ ]  Throttle .... 1/4 INCH OPEN

[ ]  Mixture .... RICH

[ ]  Starter .... ENGAGE

[ ]  Throttle .... ADJUST 800-1000 RPM

[ ]  Oil Press .... GREEN

[ ]  Mixture .... LEAN FOR TAXI

[ ]  Electrical Fuel Pump .... OFF

[ ]  Alternator

[ ]  Radio / Avionics .... ON

[ ]  Suction

[ ]  Annunciator Warn .... OFF

[ ]  A / P .... TEST

[ ]  Elec Trim .... TEST

[ ]  Gyro .... SET TO COMP

[ ]  Altimeter .... QNH

[ ]  XPDR .... STDBY

[ ]  Circuit Breakers .... ALL IN

[ ]  RPM

[ ]  Oil Press .... GREEN

[ ]  Fuel Qty / Selector .... AS CALC / FULLEST TANK

[ ]  Start UP / T H P .... CONFIRMED

TAXI

[ ]  Toe Brakes

[ ]  Bank Indicator / Gyro / Comp

RUN UP

[ ]  Park Brake .... SET

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... 2000 RPM

[ ]  Oil Press .... GREEN

[ ]  Oil Temp .... GREEN

[ ]  MAGNETOS L / R max .... DELTA 50 DROP 175 RPM CHECKED

[ ]  Carb. Heat .... MAX DROP 100 RPM CHECKED

[ ]  Gyro Suction .... DROP IN LIMITS

[ ]  Alternator

[ ]  Throttle .... BACK TO IDLE - 1000 RPM

[ ]  Oil Press .... GREEN

[ ]  Oil Temp .... GREEN

[ ]  Annunciators .... TEST

[ ]  XPDR .... ON

Before TAKE OFF

[ ]  Flight Controls .... FREE AND CORRECT

[ ]  Altimeter .... SET

[ ]  GYRO .... SET

[ ]  Radios .... SET

[ ]  AVIONICS .... SET

[ ]  NAV BUGS .... SET

[ ]  Flaps .... AS REQRD

[ ]  Trim .... SET

[ ]  A / P .... OFF

[ ]  TAKE OFF BRIEFING

[ ]  EMERGENCY BRIEFING

[ ]  Window & Door .... SECURED

[ ]  PARK BRAKE .... OFF

[ ]  LINE UP .... READ BACK

[ ]  Fuel Pump .... ON

[ ]  RUNWAY .... LINE UP CENTER

[ ]  GYRO .... COMPASS IDENTICAL

[ ]  RUNWAY DESIGNATOR .... GYRO IDENTICAL

[ ]  XPDR .... ALTITUDE

[ ]  Landing Light .... ON

[ ]  Anti-Coll.-, NAV-Light / Pitot Heat .... AS REQRD

[ ]  TIME OFF

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-181 Archer II_III/Roland_Bialy_PA28-181_checklist.txt

Rights: licensed

Checklist hash: 2c8108b72a9b5081002f3f91a9b91c3d14bb5b368757d780085dff8ebf172cb1

Verify against the aircraft's own approved documentation before use.