Piper PA-28-200R Arrow II — MikeOMalley_PA28R-200_CHECKLIST-NORMAL

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Passenger Briefing

[ ]  Seat belts, Shoulder Harnesses

[ ]  Door Operation

[ ]  Emergency Exits

[ ]  Emergency Brace Position

[ ]  Fire Extinguisher

[ ]  Eating, drinking, smoking

[ ]  Control interference

[ ]  Collision avoidance

[ ]  Physiological considerations

[ ]  Conversation

[ ]  Supplemental information

Before Engine Start

[ ]  Door .... Closed and latched

[ ]  Seats .... Adjusted

[ ]  Seat belts and harnesses .... Fasten

[ ]  Fire extinguisher .... Secure

[ ]  Pitch trim .... Takeoff

[ ]  Flaps .... Retracted

[ ]  Rudder trim .... Neutral

[ ]  Prop .... Full forward

[ ]  Mixture .... Idle Cut-Off

[ ]  Alternate Air .... Off

[ ]  Alternate Static .... Off

[ ]  Fuel Selector .... Desired Tank

[ ]  Breakers .... Check

[ ]  Vent fan .... Off

[ ]  Pitot heat .... Off

[ ]  Avionics Master .... Off

Engine Start- Cold

[ ]  Beacon .... On

[ ]  Throttle .... Open ½ Inch

[ ]  Master .... On

[ ]  Fuel Pump .... On

[ ]  Mixture .... Full Rich until positive fuel flow indicated

[ ]  Mixture .... Idle cut-off

[ ]  Ignition .... Start

[ ]  Mixture .... FULL RICH when engine fires

[ ]  Throttle .... 800 RPM

[ ]  Oil Pressure and Temp .... Check

[ ]  Fuel pump .... Off

Engine Start- Hot

[ ]  Beacon .... On

[ ]  Throttle .... Open ½ Inch

[ ]  Master .... On

[ ]  Fuel Pump .... On

[ ]  Mixture .... Idle cut-off

[ ]  Ignition .... Start

[ ]  Mixture .... FULL RICH when engine fires

[ ]  Throttle .... 800 RPM

[ ]  Oil Pressure and Temp .... Check

[ ]  Fuel pump .... Off

Engine Start- Flooded

[ ]  Beacon .... On

[ ]  Throttle .... Full open

[ ]  Master .... On

[ ]  Fuel Pump .... On

[ ]  Mixture .... Idle cut-off

[ ]  Ignition .... Start

[ ]  Mixture .... FULL RICH when engine fires

[ ]  Throttle .... 800 RPM

[ ]  Oil Pressure and Temp .... Check

[ ]  Fuel pump .... Off

Warm Up

[ ]  Throttle .... 800-1200 RPM

[ ]  Annunciator Panel .... Test

[ ]  Flight Instruments .... Set and Check

[ ]  Avionics .... On and Set

[ ]  COMM .... Freq, Vol, Opr

[ ]  NAV .... Freq, Ident, Indicator, Accuracy, Course

[ ]  DME .... Freq, Ident

[ ]  ADF .... Freq, Ident

[ ]  MB .... Test, Sensitivity, Audio

[ ]  Transponder .... Test, Code

[ ]  LORAN .... Set

Taxi

[ ]  Taxi Area .... Clear

[ ]  Brakes and Steering .... Check

[ ]  Instruments .... Taxi Inst. Check

[ ]  Airspeed .... “Zero”

[ ]  Attitude .... Erect, Stable

[ ]  Altimeter .... Set, 75’ max

[ ]  VSI .... “Zero” noted

[ ]  DG .... Turning freely

[ ]  TC .... Proper Yaw, ball opposite

[ ]  Compass .... Fluid, turning freely

[ ]  ADF .... Tracking

Run-up

[ ]  Brakes .... Set

[ ]  Throttle .... 2000

[ ]  Engine Gauges .... Green

[ ]  Mags .... Check 175 max, 50 diff

[ ]  Vacuum .... 5 InHg ±0.1”

[ ]  Ammeter .... Check, load

[ ]  Alternate air .... Check

[ ]  Prop .... Cycle, then full forward

Take-off

[ ]  Fuel .... proper tank

[ ]  Fuel Pump .... On

[ ]  Engine Gauges .... Check

[ ]  Flight instruments .... Check and Set

[ ]  Master .... On

[ ]  Alternate Air .... Closed

[ ]  Mixture .... Set

[ ]  Prop .... Full forward

[ ]  Lights .... As required

[ ]  Transponder .... Alt

[ ]  Controls .... Free and Correct

[ ]  Seat backs .... Erect

[ ]  Seat belts, Harnesses .... Set

[ ]  Empty seats .... Secured

[ ]  Flaps .... Exercised and Set

[ ]  Trim .... Set

[ ]  Controls .... Free and Correct

[ ]  Door .... Latched

[ ]  Briefing

[ ]  Runway

[ ]  Crosswind

[ ]  Heading, altitude

[ ]  Emergency procedures

[ ]  Time .... Noted

Short Field

[ ]  Flaps .... 25°

[ ]  Rotate .... 60-65

[ ]  Vx .... 85

[ ]  Gear .... Up

[ ]  Vy .... 100

[ ]  Flaps .... Retract slowly

Climb

[ ]  Fuel Pump .... Off at 1000’ AGL

[ ]  Airspeed .... As required (110 MPH)

Cruise

[ ]  Lights .... As required

[ ]  Power .... As desired

[ ]  Mixture .... Lean

Descent

[ ]  Mixture .... Enrichen as required

[ ]  Airspeed .... As required

[ ]  Lights .... As required

[ ]  Weather

[ ]  Approach Briefing

Landing

[ ]  Seat backs .... erect

[ ]  Belts, harness .... fastened

[ ]  Fuel .... Proper tank

[ ]  Fuel pump .... On

[ ]  Mixture .... Set

[ ]  Propeller .... Set

[ ]  Gear .... DOWN (150 MPH)

[ ]  Flaps .... Set (125 MPH)

After Landing

[ ]  Transponder .... “STBY”

[ ]  Pitot Heat .... Off

[ ]  Alternate Air .... Off

[ ]  Flaps .... Retract

[ ]  Fuel Pump .... Off

[ ]  Lights .... As required

Shutdown

[ ]  Throttle .... Idle

[ ]  Avionics .... Off

[ ]  Mixture .... Idle Cut Off

[ ]  Electrical Switches .... Off

[ ]  Mags .... Off

[ ]  Master .... Off

[ ]  Trim .... Neutral

[ ]  Control Wheel .... Secure

[ ]  Tie-Downs .... Secure

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-200R Arrow II/MikeOMalley_PA28R-200_CHECKLIST-NORMAL.txt

Rights: licensed

Checklist hash: 5da985ed7dc981efe12dcdd68d20c64d0d4b193af973af22e39ae4e035f3bdbd

Verify against the aircraft's own approved documentation before use.