Piper PA-28-200R Arrow II — p28r_arrow_checklist_moorabbin

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "AFTER T/O CHECKS" item count differs between passes (6 vs 5); verify nothing was dropped.

content_gap: Section "RUN-UP CHECKS" item count differs between passes (15 vs 14); verify nothing was dropped.

PRE START

[ ]  Loose items stowed

[ ]  Seat adjusted

[ ]  Hatches and harnesses secure

[ ]  Stab trim .... THROUGH RANGE SET TO TAKE OFF

[ ]  Flaps .... CHECKED, SET ZERO

[ ]  Rudder trim set

[ ]  Park Brake .... ON

[ ]  Fuel .... ON LOWEST TANK

[ ]  Alternate air .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Pitch .... FULL FINE

[ ]  Throttle .... CLOSED

[ ]  Instruments checked and set

[ ]  Avionics .... OFF

[ ]  Switches .... OFF

[ ]  Master .... ON

[ ]  Alternator .... ON

[ ]  Circuit breakers .... IN

[ ]  Controls .... FULL AND FREE, CORRECT SENSE

START CHECKS

[ ]  Throttle .... 1 CM OPEN

[ ]  Fuel pump .... ON

[ ]  Mixture .... RICH TO PRIME, THEN IDLE C/O

[ ]  Fuel flow indicator .... PEAK

[ ]  Fuel pump .... OFF

[ ]  Check “Clear Prop”

[ ]  Magnetos .... START

AFTER START

[ ]  Mixture .... RICH

[ ]  Throttle .... SET 1000 RPM

[ ]  Oil pressure .... GREEN WITHIN 30 SECS

[ ]  Ammeter .... POSITIVE CHARGE

[ ]  Suction .... WITHIN LIMITS

[ ]  Gyro instruments erection and sync

[ ]  Annunciators .... TEST & OUT

[ ]  Avionics .... ON

[ ]  Beacon .... ON

[ ]  Transponder .... STBY

TAXI CHECKS

[ ]  Brakes .... CHECKED

[ ]  Instruments turning checks

RUN-UP CHECKS

[ ]  Park brake .... ON

[ ]  Fuel selector .... CHANGE TANKS

[ ]  Oil temp & pressure .... GREEN ARC

[ ]  Lookout .... ALL CLEAR

[ ]  Throttle .... SET 2000 RPM

[ ]  Oil temp & pressure .... GREEN ARC

[ ]  Magnetos .... LEFT, BOTH, RIGHT, BOTH

[ ]  Alternate air .... ON THEN OFF

[ ]  Pitch Lever .... BRING BACK TILL DROP

[ ]  Pitch Lever drop .... MAX 500 RPM. REPEAT 3 TIMES

[ ]  Pitch Lever .... SET FULLY FINE

[ ]  Ammeter .... POSITIVE CHARGE

[ ]  Suction .... 4.8” – 5.1” Hg

[ ]  Slow idle .... SMOOTH RUNNING

[ ]  Throttle .... SET 1000 RPM

PRE T/O VITAL ACTIONS

[ ]  T .... TRIMS SET, THROTTLE FRICTION ADJUSTED

[ ]  M .... MIXTURE RICH

[ ]  P .... PROPELLER, FULL FINE

[ ]  F .... FUEL ON, CONTENTS CHECKED

[ ]  F .... 3 STAGES FLAP CHECKED

[ ]  I .... INSTRUMENTS CHECKED AND SET

[ ]  I .... AUTO PILOT CHECKED

[ ]  S .... SWITCHES, AS REQUIRED

[ ]  C .... CONTROLS FULL & FREE

[ ]  H .... HATCHES & HARNESSES SECURE

PRE LINE UP CHECKS

[ ]  F .... FUEL PUMP ON, FLAPS SET

[ ]  I .... INSTRUMENTS, DG TO RUNWAY HEADING

[ ]  S .... SWITCHES, LIGHTS AS REQUIRED

[ ]  T .... TRANSPONDER TO ALT

AFTER T/O CHECKS

[ ]  Undercarriage .... IDENTIFY, SELECT, RETRACT

NOTE: Speed below 107, gear up

[ ]  Flaps .... RETRACTED

[ ]  Power .... SET AS REQUIRED, 25/2500

[ ]  Oil temp & pressure .... GREEN ARC

[ ]  Fuel pump .... OFF, PRESSURE CHECKED

Pre manoeuvre checks

[ ]  H .... HEIGHT

[ ]  A .... AIRFRAME

[ ]  S .... SECURITY

[ ]  E .... ENGINE

[ ]  L .... LOCATION

[ ]  L .... LOOKOUT

PRE LANDING CHECKS

[ ]  S .... SPEED BELOW 129 KIAS

[ ]  U .... UNDERCARRIAGE DOWN AND LOCKED

[ ]  M .... MIXTURE RICH

[ ]  F .... FUEL PUMP ON, PRESS & CONTENTS

[ ]  O .... OIL TEMP & PRESSURE

[ ]  H .... HATCHES & HARNESSES SECURE

[ ]  B .... BRAKES, CHECK PRESSURE AND OFF

FINAL CHECKS

[ ]  P .... PITCH FULLY FINE

[ ]  U .... UNDERCARRIAGE DOWN, 3 GREENS

[ ]  F .... FLAPS SET AS REQUIRED

AFTER LANDING CHECKS

[ ]  Flaps .... RETRACT

[ ]  Strobes .... OFF

[ ]  Transponder .... STBY

[ ]  Radio .... TO SMC

SHUTDOWN

[ ]  Park brake .... ON

[ ]  Throttle .... SET 1000 RPM

[ ]  Avionics .... OFF

[ ]  Beacon .... OFF

[ ]  Magnetos .... CHECK FOR RPM DROP

[ ]  Temp and pressure .... GREEN ARC

[ ]  Fuel Pump .... OFF

[ ]  Mixture .... IDLE CUT OFF

[ ]  Throttle .... CLOSED

[ ]  Magnetos .... OFF, KEY OUT

[ ]  Master .... OFF

[ ]  Park brake .... RELEASE IF TIEING DOWN

POST FLIGHT

[ ]  Controls .... SECURE

[ ]  Hatches .... SECURE

[ ]  Tie downs .... SECURED

RADIO FREQUENCIES

MOORABBIN

NOTE: MB GROUND 119.9 MHz

NOTE: MB TOWER (EAST) 118.1 MHz

NOTE: MB TOWER (WEST) 123.0 MHz

NOTE: MB ATIS 398 or 120.9 MHz

ESSENDON

NOTE: EN GROUND 121.9 MHz

NOTE: EN TOWER 125.1 MHz

NOTE: EN ATIS 356 or 119.8 MHz

MELBOURNE

NOTE: ML RAS 135.7 MHz

NOTE: ML APP 124.7 MHz

NOTE: ML DEP 118.9 or 129.4 MHz

NOTE: ML TWR 120.5 MHz

NOTE: ML SMC 121.7 MHz

NOTE: ML ACD 127.2 MHz

NOTE: ML ATIS 114.1 or 132.7 MHz

EMERGENCY PROCEDURES  [EMERGENCY]

ENGINE FIRE DURING START

[ ]  Starter crank engine

[ ]  Mixture .... IDLE CUT OFF

[ ]  Throttle .... OPEN

[ ]  Electric fuel pump .... OFF

[ ]  Fuel selector .... OFF

[ ]  Abandon if fire continues .... ABANDON IF FIRE CONTINUES

ENGINE FAILURE IN FLIGHT

INITIAL ACTIONS:

CHECK FOR FIRE

[ ]  Throttle .... SET 1/3 POSITION

[ ]  Alternate air .... ON

[ ]  Mixture .... RICH

[ ]  Fuel pump .... ON

[ ]  Fuel selector .... CHANGE TANKS

ASSESS WIND

SELECT FIELD

MAKE PLAN

TROUBLE CHECKS:

[ ]  Fuel .... ON CONTENTS CHECKED

[ ]  Mixture .... THROUGH RANGE, SET RICH

[ ]  Oil temps and pressure

[ ]  Switches .... CHECK MAGNETOS

[ ]  Throttle .... THROUGH RANGE SET 1/3 POSITION

MAYDAY CALL

SHUTDOWN CHECKS:

[ ]  Brakes .... CHECK PRESSURE AND OFF

[ ]  Undercarriage .... DOWN

[ ]  Switches .... ALL OFF, FUEL OFF

[ ]  Hatches and harnesses .... SECURE

ENGINE FIRE IN FLIGHT

[ ]  Mixture [MEMORY] .... IDLE CUT-OFF

[ ]  Throttle [MEMORY] .... CLOSED

[ ]  Fuel Pump [MEMORY] .... OFF

[ ]  Fuel Selector [MEMORY] .... OFF

[ ]  Heater .... OFF

[ ]  Defroster .... OFF

[ ]  Proceed with .... POWER OFF LANDING

LOSS OF OIL PRESSURE / HIGH OIL TEMPERATURE

[ ]  Land as soon as possible and investigate cause.

[ ]  Prepare for power off landing

EMERGENCY GEAR EXTENSION

[ ]  Gear selector .... DOWN

[ ]  Circuit breaker .... IN

[ ]  Nav lights .... OFF

[ ]  Master switch .... ON

[ ]  Bulbs .... CHECKED

[ ]  Speed .... BELOW 87 KIAS

[ ]  Emerg gear dump .... HOLD DOWN

NOTE: If gear fails to extend, yaw aircraft abruptly.

NOTE: Reduce airspeed to below 87 KTS.

[ ]  Move landing gear selector .... TO DOWN.

[ ]  Move the emergency gear lever .... TO EMERGENCY DOWN POSITION.

NOTE: If gear fails to lock down, yaw the aircraft abruptly from side to side with rudder.

NOTE: For other emergency procedures refer to section 3 of the Pilots Operating Handbook.

PA28R (PIPER ARROW) DATA

AIRSPEEDS

NOTE: Basic Stall Speed (VS1) 60 KIAS

NOTE: Stall with Flap (VS0) 55 KIAS

NOTE: Best Climb Angle (VX) 78 KIAS

NOTE: Best Climb Rate (VY) 90 KIAS

NOTE: Cruise Climb (Normal) 104 KIAS

NOTE: Flap Limit Speed (VFE) 103 KIAS

NOTE: Gear Extending (VLE) 129 KIAS

NOTE: Gear Retracting (VLO) 107 KIAS

NOTE: Threshold 70 KIAS

NOTE: Manoeuvre Speed (VA) 96-118 KIAS

NOTE: Flight Planning TAS 135 KTAS

NOTE: Maximum Cruising (VNO) 146 KIAS

NOTE: Never Exceed (VNE) 183 KIAS

NOTE: Best Glide 79 KIAS

NOTE: TOSS 70 KIAS

NOTE: Base 90 KIAS

NOTE: Final 75 KIAS

NOTE: Maximum Crosswind 17 KIAS

WEIGHT LIMITATIONS

NOTE: Max Takeoff Weight 1247 Kg 2650 Lb

NOTE: Empty Weight: 720 Kg 1585 Lb ( for VH-LFP)

NOTE: Max Baggage Weight 90 Kg 200 Lb

FUEL

NOTE: Type: 100 Octane Avgas (green) or 100 LL (blue)

NOTE: Quantity: Total 276 lt, Useable 272 lt

NOTE: Typical Consumption: 42 lt per hour (fuel flow at 75%)

NOTE: Endurance with 45 min reserve intact: 4.2 hrs

POWER SETTINGS

NOTE: Take-off 29.5” / 2700 RPM

NOTE: Climb 25” / 2500 RPM

NOTE: Cruise (2400) 55% 65% 75%

NOTE: 2000” 20.0” 22.5” 25.0”

NOTE: 4000” 19.5” 22.0” 24.4”

NOTE: 6000” 19.1” 21.5”

NOTE: 8000” 18.7” 21.0”

NOTE: Descent 20-24” / 2400 RPM

NOTE: Circuit 21” / 2400 RPM

NOTE: Base 15” / 2400 RPM

ENGINE

NOTE: Avco Lycoming IO-360-C Series

NOTE: 4 cylinder, horizontally opposed, direct drive, air cooled, fuel injected

NOTE: Output: 200 Hp @ 2700 rpm

OIL

NOTE: Type: Aviation grade D100

NOTE: Quantity: Max 8 US quarts, Min 6 US quarts (Company reqts)

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-200R Arrow II/p28r_arrow_checklist_moorabbin.txt

Rights: licensed

Checklist hash: 35b5c2bf8c133e957646a04b207de053ba37b28cb87a13d709db2eaa51ed3316

Verify against the aircraft's own approved documentation before use.