Piper PA-28-200R Arrow II — PA28200_matt_crocker

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

CABIN CHECKLIST

[ ]  Parking Brake .... SET

[ ]  Legal Documents .... AVAILABLE

[ ]  Master Switch .... OFF

[ ]  Ignition Switch .... OFF

[ ]  Landing Gear Switch .... DOWN

[ ]  Control Wheel Lock .... OFF

[ ]  Master Switch .... ON

[ ]  Fuel Quantity .... CHECK

[ ]  Landing Gear Lights .... 3 GREEN

[ ]  Stall Warning Light .... CHECK

[ ]  Cabin Lighting .... CHECK

[ ]  Exterior Lighting .... CHECK

[ ]  Pitot Heat .... CHECK

[ ]  Master Switch .... OFF

[ ]  Fuel Selector .... PROPER TANK

[ ]  Flaps .... EXTEND

RIGHT WING CHECKLIST

[ ]  Wing Condition .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Chocks .... REMOVE

[ ]  Right Flap .... CHECK

[ ]  Right Aileron .... CHECK

[ ]  Static Wicks .... CHECK

[ ]  Fuel .... CHECK LEVEL

[ ]  Fuel Cap .... SECURE

[ ]  Fuel Sump .... CHECK

[ ]  Tire .... INFLATION / WEAR

[ ]  Gear .... CHECK CONDITION

[ ]  Wheel Well .... UNOBSTRUCTED

[ ]  Air Vent .... UNOBSTRUCTED

NOSE SECTION CHECKLIST

[ ]  Chocks .... REMOVE

[ ]  Oil Level .... MINIMUM 2 QUARTS

[ ]  Propeller & Spinner .... GOOD CONDITION / NO OIL RESIDUE

[ ]  Cowling .... GOOD CONDITION

[ ]  Cowl Scoop .... UNOBSTRUCTED

[ ]  Windshield .... CLEAN / NO CRACKS

[ ]  Nose Gear .... PROPER INFLATION

[ ]  Nose Gear Well .... UNOBSTRUCTED / NO LEAKS

[ ]  Exhaust Pipes .... UNOBSTRUCTED

LEFT WING CHECKLIST

[ ]  Chocks .... REMOVE

[ ]  Wing Condition .... CHECK

[ ]  Tie Down .... REMOVE

[ ]  Left Flap .... CHECK

[ ]  Left Aileron .... CHECK

[ ]  Static Wicks .... CHECK

[ ]  Fuel Level .... CHECK

[ ]  Fuel Cap .... SECURE

[ ]  Fuel Sump .... CHECK

[ ]  Tire .... INFLATION / WEAR

[ ]  Gear .... CHECK CONDITION

[ ]  Squat Switch .... PROPER POSITION

[ ]  Wheel Well .... UNOBSTRUCTED

[ ]  Pitot / Static .... UNOBSTRUCTED

[ ]  Air Vent .... UNOBSTRUCTED

[ ]  Auto Gear Extension Sensor .... UNOBSTRUCTED

FUSELAGE CHECKLIST

[ ]  General Condition .... CHECK

[ ]  Antennas .... CHECK

[ ]  Hydraulic Pumps .... CHECK

[ ]  Battery .... CHECK

[ ]  Baggage Door .... CLOSED & LOCKED

TAIL SECTION CHECKLIST

[ ]  Stabilator .... CHECK

[ ]  Trim Tab .... CHECK

[ ]  Rudder / Vertical Stabilizer .... CHECK

NORMAL ENGINE START CHECKLIST

[ ]  Parking Brake .... ON

[ ]  Propeller .... FULL RPM

[ ]  Fuel Tank .... FULLEST TANK

[ ]  Throttle .... OPEN 1/2 INCH

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture Control .... FULL RICH UNTIL RISE IN FUEL FLOW; THEN CUTOFF POSITION

[ ]  Starter .... ENGAGE

[ ]  Mixture .... FULL RICH

[ ]  Fuel Timer .... START

HOT ENGINE START CHECKLIST

[ ]  Parking Brake .... ON

[ ]  Propeller .... FULL RPM

[ ]  Fuel Tank .... FULLEST TANK

[ ]  Throttle .... OPEN 1/2 INCH

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Starter .... ENGAGE

[ ]  Mixture .... FULL RICH

FLOODED ENGINE START CHECKLIST

[ ]  Throttle .... FULL OPEN

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... OFF

[ ]  Mixture .... IDLE CUT-OFF

[ ]  Starter .... ENGAGE

ENGINE START WITH EXTERNAL POWER

[ ]  Master Switch .... OFF

[ ]  Piper External Power (PEP) .... CONNECT RED LEAD TO POSITIVE TERMINAL OF THE EXTERNAL 12 VOLT BATTERY AND BLACK LEAD TO NEGATIVE TERMINAL

[ ]  Jumper Cable Plug .... INSERT INTO SOCKET ON AIRCRAFT FUSELAGE

[ ]  Master Switch .... ON

[ ]  Engine Start .... PROCEED WITH NORMAL STARTING TECHNIQUE

[ ]  Master Switch .... OFF

[ ]  Jumper Cable Plug .... REMOVE

[ ]  Master Switch .... ON

[ ]  Alternator / Ammeter .... POSITIVE OUTPUT

PRE-TAKEOFF CHECKLIST

[ ]  Parking Brake .... SET

[ ]  Seatbacks .... ERECT

[ ]  Seatbelts / Shoulder Harnesses .... CHECK

[ ]  Air Conditioner .... OFF

[ ]  Cabin Doors .... CLOSED & LOCKED

[ ]  Flight Controls .... FREE & CORRECT

[ ]  Attitude Indicator .... CHECK

[ ]  Turn Coordinator / Ball .... CHECK

[ ]  Heading Indicator .... CHECK / SET

[ ]  Altimeter .... CHECK / SET

[ ]  VSI .... CHECK

[ ]  Fuel Quantity .... CHECK

[ ]  Mixture .... RICH

[ ]  Elevator Trim .... CHECK

[ ]  Flaps .... EXERCISE

[ ]  Propeller .... FULL RPM

[ ]  Throttle .... 2000 RPM

[ ]  Magnetos .... CHECK (175/50)

[ ]  Propeller .... EXERCISE—THEN FULL RPM

[ ]  Vacuum .... 5'' Hg +- 0.1''

[ ]  Oil Temperature / Pressure .... GREEN

[ ]  Fuel Pump .... ON / OFF / ON

[ ]  Alternate Air .... CLOSED

[ ]  Autopilot .... OFF

[ ]  Radios / Avionics .... SET

[ ]  Lights .... AS REQUIRED

[ ]  Transponder .... SQUAWK ALT.

NORMAL TAKEOFF CHECKLIST

[ ]  Throttle .... FULL OPEN

[ ]  Engine Instruments .... CHECK

[ ]  Airspeed .... ALIVE

[ ]  Rotate .... 65 MPH

[ ]  Best Angle .... 85 MPH DIRTY / 96 MPH CLEAN

[ ]  Best Rate .... 95 MPH DIRTY / 100 MPH CLEAN

[ ]  Gear .... UP

[ ]  Climb Power .... 25 SQUARED

[ ]  Cruise Climb .... 110 MPH

SHORT FIELD / OBSTACLE TAKEOFF CHECKLIST

[ ]  Flaps .... 25° (SECOND NOTCH)

[ ]  Throttle .... FULL OPEN

[ ]  Airspeed .... ALIVE

[ ]  Rotate .... 65 MPH

[ ]  Accelerate .... 85 MPH

[ ]  Gear .... UP

[ ]  Accelerate .... 100 MPH

[ ]  Flaps .... RETRACT

SOFT FIELD / OBSTACLE TAKEOFF CHECKLIST

[ ]  Flaps .... 25° (SECOND NOTCH)

[ ]  Throttle .... FULL OPEN

[ ]  Airspeed .... ALIVE

[ ]  Rotate .... LOWEST POSSIBLE SPEED

[ ]  Best Angle Climb .... 85 MPH

[ ]  Gear .... UP

[ ]  Accelerate .... 100 MPH

[ ]  Flaps .... RETRACT

ENROUTE CLIMB CHECKLIST

[ ]  Power .... 25 SQUARED

[ ]  Mixture .... LEAN

[ ]  Climb Speed .... 110 MPH

[ ]  Landing Light .... OFF

[ ]  Engine Instruments .... CHECK

CRUISE CHECKLIST

[ ]  Power .... 22''- 23'' MANIFOLD / 2400 RPM

[ ]  Trim .... LEVEL FLIGHT

[ ]  Mixture .... LEAN

[ ]  Engine Instruments .... CHECK

FUEL TANK CHANGE CHECKLIST

[ ]  Verify .... 30 MIN CHANGE

[ ]  Fuel Pump .... ON

[ ]  Fuel Tank Switch .... PROPER TANK

[ ]  Fuel Flow .... POSITIVE

[ ]  Fuel Pump .... OFF

[ ]  Fuel Flow .... POSITIVE

[ ]  Other Engine Instruments .... CHECK

APPROACH & LANDING CHECKLIST

[ ]  Seat backs .... ERECT

[ ]  Seatbelts/Harnesses .... FASTEN

[ ]  Fuel .... FULLEST TANK

[ ]  Fuel Pump .... ON

[ ]  Mixture .... SET

[ ]  Propeller .... SET

[ ]  Gear .... DOWN BELOW

[ ]  Flaps .... SET

[ ]  Air Conditioner .... OFF

[ ]  Airspeed .... APPROX. 85 – 90mph ON FINAL

[ ]  G.U.M.P. Checks .... CHECK

AFTER LANDING TAXI CHECKLIST

[ ]  Ground Control .... CONTACT/MONITOR

[ ]  Fuel Pump .... OFF

[ ]  Flaps .... RETRACT

[ ]  Transponder .... STANDBY

SHUT DOWN CHECKLIST

[ ]  Parking Brake .... SET

[ ]  Fuel Pump .... OFF

[ ]  Propeller .... FULL RPM

[ ]  Avionics .... OFF

[ ]  Lights .... OFF

[ ]  Throttle .... 1000 RPM

[ ]  Mixture .... IDLE

[ ]  Throttle .... OPEN FULL / THEN CUTOFF AFTER ENGINE SHUTDOWN

ENGINE POWER LOSS IN FLIGHT CHECKLIST  [EMERGENCY]

[ ]  Fuel Selector .... SWITCH TANKS

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Alternate Air .... ON

[ ]  Engine Gauges .... CHECK

[ ]  If no fuel pressure .... CHECK TANKS AGAIN

WHEN POWER IS RESTORED:

[ ]  Alternate Air .... OFF

[ ]  Electric Fuel Pump .... OFF

IF POWER IS NOT RESTORED:

[ ]  Ignition Switches .... CHECK "L", "R", & BOTH

[ ]  Throttle & Mixture .... DIFFERENT SETTINGS

GEAR DOWN LANDING CHECKLIST  [EMERGENCY]

[ ]  Gear .... DOWN

[ ]  Throttle .... CLOSE

[ ]  Master Switches .... OFF

[ ]  Ignition Switches .... OFF

[ ]  Flaps .... AS DESIRED

[ ]  Fuel Selector Valve .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Seat belts .... TIGHTEN

[ ]  Touchdown .... AT LOWEST POSSIBLE SPEED

GEAR UP LANDING CHECKLIST  [EMERGENCY]

[ ]  Emergency Gear Extender .... OVERRIDE ENGAGED

[ ]  Flaps .... AS DESIRED

[ ]  Throttle .... CLOSE

[ ]  Master Switches .... OFF

[ ]  Ignition Switches .... OFF

[ ]  Fuel Selector Valve .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Seat belts .... TIGHTEN

[ ]  Contact surface .... AT MINIMUM POSSIBLE SPEED

PROPELLER OVERSPEED CHECKLIST  [EMERGENCY]

[ ]  Throttle .... RETARD

[ ]  Oil Pressure .... CHECK

[ ]  Propeller Control .... FULL DECREASE RPM, THEN SET IF ANY CONTROL AVAILABLE

[ ]  Airspeed .... REDUCE

[ ]  Throttle .... AS REQUIRED TO REMAIN BELOW 2700 RPM

EMERGENCY LANDING GEAR EXTENSION CHECKLIST  [EMERGENCY]

[ ]  Master Switch .... CHECK ON

[ ]  Circuit Breakers .... CHECK

[ ]  Panel Lights .... OFF (in daytime)

[ ]  Gear Indicator Bulbs .... CHECK

IF LANDING GEAR DOES NOT CHECK DOWN AND LOCKED:

[ ]  Airspeed .... REDUCE BELOW 100 MPH

[ ]  Landing Gear Selector Switch .... DOWN

[ ]  IF EQUIPPED WITH BACKUP GEAR EXTENDER, RAISE EMERGENCY GEAR LEVER .... "OVERRIDE ENGAGED" POSITION

[ ]  IF GEAR IS STILL NOT DOWN AND LOCKED, MOVE EMERGENCY GEAR LEVER .... "EMERGENCY DOWN" POSITION

[ ]  IF GEAR IS STILL NOT DOWN AND LOCKED, YAW AIRPLANE ABRUPTLY FROM SIDE TO SIDE WITH THE RUDDER .... AS REQUIRED

IN-FLIGHT FIRE CHECKLISTS  [EMERGENCY]

[ ]  Fire Source .... CHECK / DETERMINE

ELECTRICAL FIRE CHECKLIST  [EMERGENCY]

[ ]  Master Switch .... OFF

[ ]  Vents .... OPEN

[ ]  Cabin Heat .... OFF

[ ]  Land as soon as practicable .... LAND AS SOON AS PRACTICABLE

ENGINE FIRE CHECKLIST  [EMERGENCY]

[ ]  Fuel Selector .... OFF

[ ]  Throttle .... CLOSE

[ ]  Mixture .... IDLE CUTOFF

[ ]  Heater .... OFF

[ ]  Defroster .... OFF

[ ]  IF TERRAIN PERMITS .... LAND IMMEDIATELY!

GROUND ENGINE FIRE CHECKLIST  [EMERGENCY]

[ ]  Mixture .... IDLE CUTOFF

[ ]  Fuel Selector .... OFF

[ ]  Throttle .... OPEN

[ ]  Starter .... ENGAGE TO PULL FIRE INTO ENGINE

PERFORMANCE

[ ]  TAKEOFF RUN (average) .... 770FT

[ ]  BEST RATE OF CLIMB (GEAR UP) .... 100MPH

[ ]  RATE OF CLIMB .... 900FPM

[ ]  SERVICE CEILING .... 15,000FT

[ ]  ABSOLUTE CEILING .... 17,000FT

[ ]  TOP SPEED .... 175MPH

[ ]  OPTIMUM CRUISING SPEED .... 165MPH

[ ]  CRUISING RANGE (55% POWER & OPTIMUM ALTITUDE) .... 900MI

[ ]  CRUISING RANGE (75% POWER & OPTIMUM ALTITUDE) .... 780MI

[ ]  STALLING SPEED (DIRTY) .... 64MPH

[ ]  STALLING SPEED (CLEAN) .... 71MPH

[ ]  LANDING ROLL (FLAPS DOWN) .... 780FT

[ ]  LANDING ROLL (50 FT) .... 1380FT

WEIGHTS

[ ]  GROSS WEIGHT .... 2650LBS

[ ]  EMPTY WEIGHT (STANDARD) .... 1499LBS

[ ]  USEFUL LOAD (STANDARD) .... 1151LBS

NOTE: *WEIGHTS ARE APPROXIMATE

POWER PLANT

[ ]  ENGINE (LYCOMING) .... IO-360-C1C

[ ]  RATED HORSEPOWER .... 200HP

[ ]  RATES SPEED .... 2700RPM

[ ]  BORE .... 5.125IN

[ ]  STROKE .... 4.375IN

[ ]  DISPLACEMENT .... 361.0 CU. IN.

[ ]  COMPRESSION RATIO .... 8.7:1

[ ]  DRY WEIGHT .... 326LBS

AIRCRAFT AIRSPEED GUIDE

[ ]  Vx .... 85mph

[ ]  Vy .... 100mph

[ ]  VS .... 71mph

[ ]  VSO .... 64mph

[ ]  VA .... 131mph

[ ]  VFE .... 125mph

[ ]  VLE .... 150mph

[ ]  VLO .... 125mph

[ ]  VNO .... 170mph

[ ]  VNE .... 214mph

NOTE: *Maximum Demonstrated Crosswind Component 20mph

FLIGHT LOAD FACTORS

[ ]  Positive Load Factor (Maximum) .... 3.8 G

[ ]  Negative Load Factor (Maximum) .... NONE

NOTE: NO INVERTED MANUVERS APPROVED

AIRSPEED INSTRUMENT MARKINGS

[ ]  RED RADIAL LINE (NEVER EXCEED) .... 214MPH

[ ]  YELLOW ARC (CAUTION RANGE) .... 170 – 214MPH

NOTE: (SMOOTH AIR ONLY)

[ ]  GREEN ARC (NORMAL RANGE) .... 71 – 170MPH

[ ]  WHITE ARC (FLAP DOWN RANGE) .... 64 – 125MPH

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-200R Arrow II/PA28200_matt_crocker.txt

Rights: licensed

Checklist hash: 58c5b5df9ec55a38e86ba15788cd5d2d17fbf87ee01f7bfbf8b57e08590379a2

Verify against the aircraft's own approved documentation before use.