Piper PA-28-200R Arrow — John_Peck_N98JG_Piper_PA-28R_Checklist

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

content_gap: Section "Airspeeds (MPH)" item count differs between passes (15 vs 16); verify nothing was dropped.

content_gap: Section "Power Setting Table" item count differs between passes (11 vs 13); verify nothing was dropped.

Before Starting Engine

[ ]  Preflight Checklist – COMPLETE

[ ]  30 Day VOR Test – CHECK

[ ]  Doors & Belts – SECURE

[ ]  Seats – POSITION and LOCK

[ ]  Circuit Breakers – CHECK

[ ]  Avionics Master .... OFF

[ ]  Electrical Switches .... ALL OFF

[ ]  Landing Gear Switch .... DOWN

[ ]  Autopilot .... OFF

[ ]  Fuel Selector .... FULLEST TANK

[ ]  Altitude Hold .... OFF

Starting Engine

[ ]  Parking Brake .... SET

[ ]  Fin Strobe .... ON

[ ]  Mixture .... IDLE CUTOFF

[ ]  Prop .... FULL FORWARD

[ ]  Throttle .... OPEN ½ INCH

[ ]  Propeller Area – CLEAR

[ ]  Master Switch .... ON

[ ]  Electric Fuel Pump .... ON

COLD START ONLY – PRIME

[ ]  Mixture .... FULL RICH

[ ]  Fuel Flow – CHECK FLOW

[ ]  Mixture .... IDLE CUTOFF

[ ]  Starter .... ENGAGE

[ ]  Mixture – FULL RICH WHEN ENGINE FIRES

[ ]  Oil Pressure – CHECK

[ ]  Throttle .... 800 RPM

[ ]  Electric Fuel Pump .... OFF

[ ]  Avionics Master .... ON

[ ]  Transponder – STANDBY/1200

[ ]  Heading Indicator – SET

[ ]  Lights – AS REQUIRED

[ ]  GPS Units – INITIALIZE

Taxi

[ ]  Brakes – TEST

[ ]  Gyro Instruments – TEST

Before Takeoff

[ ]  Parking Brake .... SET

[ ]  Flight Controls – FREE/CORRECT

[ ]  Stabilator Trim – TEST & SET

[ ]  Mixture .... FULL RICH

[ ]  Throttle .... 2100 RPM

[ ]  Magnetos – CHECK (175/50)

[ ]  Propeller – CYCLE 3 TIMES

[ ]  Engine Instruments – CHECK

[ ]  Suction Gage – CHECK

[ ]  Throttle .... 1500 RPM

[ ]  Throttle Friction – SET

[ ]  Electric Fuel Pump – ON/CHECK

[ ]  Flight Instruments – SET/CHECK

[ ]  Avionics – SET/CHECK

[ ]  Flaps – SET

[ ]  Marker Beacons – TEST

[ ]  IFR Clearance – RECEIVE

[ ]  Charts – READY

[ ]  IFR Transponder Code – SET

WHEN CLEARED FOR TAKEOFF

[ ]  Door & Window – CLOSE

[ ]  Transponder .... ALT

[ ]  Wing Strobe Lights .... ON

Takeoff

[ ]  Throttle – FULL (2700 RPM)

[ ]  Landing Gear – RETRACT @ 85

[ ]  Flaps .... UP

[ ]  Climb Power – 25”/2500RPM

Cruise

[ ]  Landing Light .... OFF

[ ]  Electric Fuel Pump .... OFF

[ ]  Power – SET AS REQUIRED

CAUTION: Avoid – 2100-2350 RPM

[ ]  Mixture – LEAN <75% POWER

[ ]  Power - 25F Rich of Peak EGT

[ ]  Economy – 25F Lean of Peak

[ ]  Trim – ADJUST

[ ]  Flight Instruments – CHECK

[ ]  Engine Instruments – CHECK

Descent

[ ]  Altitude Hold .... OFF

[ ]  Power – AS REQUIRED

[ ]  Mixture – ADJUST

Before Landing

[ ]  Seats & Belts – SECURE

[ ]  Fuel Selector – FULLEST TANK

[ ]  Landing Light .... ON

[ ]  Electric Fuel Pump .... ON

[ ]  Mixture .... RICH

[ ]  Propeller .... 2600 RPM

[ ]  Landing Gear – DOWN 150 MPH

[ ]  Flaps – AS REQUIRED 125 MPH

After Landing

Clear of Active Runway

[ ]  Flaps .... UP

[ ]  Electric Fuel Pump .... OFF

[ ]  Landing Light .... OFF

[ ]  Wing Strobes .... OFF

[ ]  Transponder .... STANDBY

Securing Airplane

[ ]  Brakes – SET AS REQUIRED

[ ]  Avionics Master .... OFF

[ ]  All Electrical Switches .... OFF

[ ]  Mixture .... IDLE CUTOFF

[ ]  Ignition Sw .... OFF/KEY OUT

[ ]  Master Switch .... OFF

[ ]  Hobbs/Tach – RECORD

[ ]  Flight Plan – CLOSED

Airspeeds (MPH)

[ ]  Rotate VR .... 60-70

[ ]  Best Angle Climb – No Flaps VX .... 81 (91 Gear Up)

[ ]  Obstacle Clearance Speed/Flaps .... 80/25º GEAR OVERIDE

[ ]  Best Rate of Climb VY .... 85 (95 Gear Up)

[ ]  Cruise Climb .... 110

[ ]  Approach (No Flaps) .... 100

[ ]  Approach (Full Flaps) .... 90

[ ]  Short Field Approach Speed/Flaps .... 85/40º

[ ]  Stall (Gear & Flaps Down) VS0 .... 64

[ ]  Stall (Gear & Flaps Up) VS1 .... 70

[ ]  Flap Extend VFE .... 125

[ ]  Landing Gear Extend VLE .... 150

[ ]  Landing Gear Retract .... 125

[ ]  Maneuvering Speed (Gross) VA .... 134

[ ]  Best Glide (Gear & Flaps Up) .... 105

Airplane Weights

[ ]  Max Gross .... 2600

[ ]  Basic Empty Weight .... 1536

[ ]  Useful Load .... 1064

[ ]  Payload - Fuel Full .... 764

[ ]  Payload - Fuel Tabs .... 860

Power Setting Table

NOTE: Power setting table: 55% = 8.0 gph, 65% = 9.2 gph, 75% = 10.2 gph. At 1000 ft: 55% = 2100 RPM, 65% = 2400 RPM, 75% = 2400 RPM.

NOTE: Power setting table manifold pressures (inhg) at 1,000 ft: 55% = 22.7, 65% = 20.2, 75% = 25.6.

NOTE: Power setting table manifold pressures (inhg) at 2,000 ft: 55% = 22.4, 65% = 20.0, 75% = 25.4.

NOTE: Power setting table manifold pressures (inhg) at 3,000 ft: 55% = 22.2, 65% = 19.8, 75% = 25.1.

NOTE: Power setting table manifold pressures (inhg) at 4,000 ft: 55% = 21.9, 65% = 19.5, 75% = 24.8.

NOTE: Power setting table manifold pressures (inhg) at 5,000 ft: 55% = 21.7, 65% = 19.3.

NOTE: Power setting table manifold pressures (inhg) at 6,000 ft: 55% = 21.4, 65% = 19.1.

NOTE: Power setting table manifold pressures (inhg) at 7,000 ft: 55% = 21.2, 65% = 18.9.

NOTE: Power setting table manifold pressures (inhg) at 8,000 ft: 55% = 21.0, 65% = 18.7.

NOTE: Power setting table manifold pressure at 9,000 ft: 65% = 18.5.

NOTE: Power setting table manifold pressures (inhg) at 1,000 ft: 75% row also shows 22.7? Reading uncertain; original table is garbled.

NOT FOR FLIGHT — source fidelity: unreviewed | operational review: none | completeness: full

Source: Piper/PA-28-200R Arrow/John_Peck_N98JG_Piper_PA-28R_Checklist.txt

Rights: licensed

Checklist hash: 6d2be8a7936ebef6a9046f69f6ecad5006f3a3d1ae1f301333d7564f90ef6bd7

Verify against the aircraft's own approved documentation before use.